3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2003860

# Electronic meeting, 20 – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 30k SSB SCS for Band n34, n39 and n50 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add 30k SSB SCS for bands n34, n39 and n50. Initial NR deployment of SA and NSA in TDD bands, Band n41 and FR1 bands above 3 GHz, is based on 30k SCS for SSB and data. In order to leverage current BS and UE TDD implementations also for lower-frequency TDD bands, 30k SSB SCS should be added to these. For NSA operation a UE can be provided with 30k SSB SCS by ssbSubcarrierSpacing but this would not allow SA operation in the same cell in case only 15k SSB SCS is applicable for the band. 30k SSB SCS would not necessitate support of the UE capability simultaneousRxDataSSB-DiffNumerology in deployments with 30k data. It may be possible to avoid simultaneous SSB measurements and receptions of e.g. PDCCH/PDSCH/CSI-RS etc at a different numerology by network configuration, but would impose restrictions. The 15k SSB SCS should be retained to allow 15k data SCS without mixed numerology in deployments with larger cell sizes (longer CP). The downside of adding 30k SSB SCS is increased cell-search time at initial access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 30k SSB SCS and SSB block pattern Case B are added for n34, n39 and n50. Isolated impact analysis: legacy UEs supporting n34, n39 or n50 can not attach to cells in these bands for SA operation when 30k SSB SCS is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 15k SSB SCS would necessitate support of the UE capability simultaneousRxDataSSB-DiffNumerology or use of restricted network configurations to avoid mixed numerology operation in SA deployments with 30k data SCS based on current BS and UE TDD implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| n3 | 15 kHz | Case A | 4517 – <1> – 4693 |
| n5 | 15 kHz | Case A | 2177 – <1> – 2230 |
|  | 30 kHz | Case B | 2183 – <1> – 2224 |
| n7 | 15 kHz | Case A | 6554 – <1> – 6718 |
| n8 | 15 kHz | Case A | 2318 – <1> – 2395 |
| n12 | 15 kHz | Case A | 1828 – <1> – 1858 |
| n20 | 15 kHz | Case A | 1982 – <1> – 2047 |
| n25 | 15 kHz | Case A | 4829 – <1> – 4981 |
| n28 | 15 kHz | Case A | 1901 – <1> – 2002 |
| n34 | 15 kHz | Case A | 5030 – <1> – 5056 |
|  | 30 kHz | Case B | 5036 – <1> – 5050 |
| n38 | 15 kHz | Case A | 6431 – <1> – 6544 |
| n39 | 15 kHz | Case A | 4706 – <1> – 4795 |
|  | 30 kHz | Case B | 4712 – <1> – 4789 |
| n40 | 15 kHz | Case A | 5756 – <1> – 5995 |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n50 | 15 kHz | Case A | 3584 – <1> – 3787 |
|  | 30 kHz | Case B | 3590 – <1> – 3781 |
| n51 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |
| n70 | 15 kHz | Case A | 4993 – <1> – 5044 |
| n71 | 15 kHz | Case A | 1547 – <1> – 1624 |
| n74 | 15 kHz | Case A | 3692 – <1> – 3790 |
| n75 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n76 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n77 | 30 kHz | Case C | 7711 – <1> – 8329 |
| n78 | 30 kHz | Case C | 7711 – <1> – 8051 |
| n79 | 30 kHz | Case C | 8480 – <16> – 8880 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8] |  |  |  |

### 5.4.4	TX–RX frequency separation

< end of changes >