3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003908

Electronic Meeting, 16 April –30 April, 2020

Source: 	OPPO

Title: 	UE maximum input level for FR2 DL 256QAM

Agenda Item:	6.12.3

Document for:	Approval

Introduction

Work item on 256QAM DL for FR was approved in RAN#83[1]. In the last meeting on UE side core requirements for FR2 DL 256QAM was discussed. The UE maximum input level was captured in drafted WF as “Discuss on how to define the UE maximum input level for 256QAM in the next meeting without considering testing feasibility”

In this contribution, we provide our views on UE core requirements for FR2 DL 256QAM.

Discussion

We will explain the motivation to introduce Maximum input level for 256QAM from following perspectives

- Performance

- Testability

- 64QAM

Observation 1: the feasibility of FR2 DL 256QAM has been evaluated, including link level simulation, system level simulation and implementation study output. It can be concluded that FR2 DL 256 QAM is feasible and beneficial as concluded in section 5.2.1.10, 5.2.2.6 and 5.3.

Observation 2: for FR1 the maximum input level was defined for high order modulation. In Table 7.4-1 Max input level for 256QAM is relaxed by 2dB compared to 64QAM.

Table 7.4-1 of 38.101-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252 |  |  |  | -242 | -232 | -222 | -212 | -202 |  |  |  |  |
|  |  | -273 |  |  |  | -263 | -253 | -243 | -233 | -223 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Because higher order modulation scheme has different SNR compared to lower order modulation, it is critical to define Maximum input level to guarantee receivers linearity performance. Not defining this core requirement puts receivers at the risk of degraded performance when operating in higher modulation.

Testability issue should be decoupled from introducing Max input level for 256QAM. A UE core requirement provides design engineers/ testing engineers a clear target to achieve. So far in FR2, requirements that are not testable have been introduced. 256 QAM should follow the same approach.

Requirement for 64QAM should be decouple from requirement of 256 QAM. It does look confusing to only define 256QAM without 64QAM. However, requirement on 256QAM has no impact on 64QAM. The work to define 64QAM can be done independently.

In summary, because of the importance and necessity of maximum input level requirements, we propose to introduce UE maximum input level for FR2 DL 256QAM

Proposal: Introduce UE maximum input level core requirements for FR2 DL 256 QAM.

Conclusion

In this contribution, we discussed the necessity to define maximum input level for DL 256 QAM from three perspectives. Namely performance, testability and decouple from 64QAM.

Proposal: Introduce UE maximum input level core requirements for FR2 DL 256 QAM.

- References

- RP-190760, New WID: Add support of NR DL 256QAM for FR2