3GPP TSG-RAN WG4 Meeting #94bis-e														R4-2003977

Electronic Meeting, 20 – 30 April, 2020

Title: 		 		Further discussion on RRM measurement relaxation for power saving

Source: 	 		OPPO

Agenda item:	  	6.7.2.1

Document for:	Discussion

- Introduction

In last meeting, a WF [1] on RRM measurement relaxation for Power Saving was approved. In this document, we further discuss the RRM measurement relaxation requirement for power saving.

- Discussion

- Issue 1：Relaxation with longer intervals

- the scaling factor of measurement interval

  - Option 1: Fixed value

    - FFS on one fixed value for both scenario#1 and scenario #2

    - FFS on separate values for scenario#1 and scenario #2

  - Option 2: Network configurable value

  - FFS on the transition period between scenario#1 or scenario#2 and scenario#3

    - Option1:

      - When switching from scenario#2 to scenario#3, UE shall fulfill the requirements corresponding to scenario#2 for N DRX cycles and thereafter switch to requirements corresponding to scenario#3

      - When switching from scenario#3 to scenario#2, UE shall fulfill the requirements corresponding to scenario #2 immediately.

    - Other option is not precluded.

On RRM measurement relaxation with longer intervals for Scenario #1 and #2, we prefer the same fixed scaling factor should apply for Scenario #1 and #2. From our side, N=2 can be good.

Proposal 1: For scenario#1 and scenario #2, define the same fixed scaling factor by N for RRM measurements with longer intervals, e.g., N=2.

On the transition period between scenario#1 or scenario#2 and scenario#3, we support option 1 that

- When switching from scenario#2 to scenario#3, UE shall fulfill the requirements corresponding to scenario#2 for N DRX cycles and thereafter switch to requirements corresponding to scenario#3

- When switching from scenario#3 to scenario#2, UE shall fulfill the requirements corresponding to scenario #2 immediately.

Proposal 2: Support option 1 for transition between scenario#1 or scenario#2 and scenario#3.

- Issue 2：RRM measurement relaxation for inter-frequency layer with higher priority

- Option 1:

  - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  no relaxation of the current measurement delay requirement is expected for inter-frequency measurement with higher priority.

  - When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the relaxed requirement for the frequency layer of higher priority shall use the same relaxed measurement requirement as those for the frequency layer of equal/lower priority.

- Option 2:

  - Measurements of higher priority carriers shall not be relaxed in high mobility scenarios (scenario #2)

- Option 3:

  - Measurements of higher priority carriers shall not be relaxed

In our view, the threshold for RRM measurement relaxation of inter-frequency measurement and criteria of different scenarios should be up to RAN2’s decision. From RAN2’s agreements in last meeting, the relaxation can be decoupled with criteria, as follows,

- For the case where Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, does RAN4 envision to relax higher priority carriers measurements further than Thigher_priority_search if RAN2-defined relaxation criterion(s) is/are met?

- For the case where Srxlev < SnonIntraSearchP or Squal < SnonIntraSearchQ, does it make sense / is there a performance benefit to only relax equal/lower priority carriers but not higher priority carriers measurements if RAN2-defined relaxation criterion(s) is/are met?

From our side, we agree with 1st bullet of option 1, but consider different relaxed measurement requirements for different priority layers when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

if Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds, where Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information. As Thigher_priority_search is relaxed already, no relaxation of the current measurement delay requirement is for higher priority.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, different relaxation for higher priority layer and equal/lower priority layer can be discussed in RAN4. In addition, we can compromise to Option 2 or 3 that measurements of higher priority carriers do not be relaxed.

Proposal 3: Different relaxation for higher priority layer and equal/lower priority layer can be defined for RRM measurement relaxation.

- Issue 3：relaxation by reducing the number of frequency layer

Proposal 4: Do not introduce the RRM measurement relaxation by reducing the number of frequency layer

- Issue 4：EMR impact in power saving mode

Assuming power saving as a higher priority, UE is not allowed to relax or enter any relaxed measurement modes if UE is configured with EMR and T331 timer is running.

Proposal 5: EMR frequency layer shall not be relaxed if T331 is running.

- Conclusions

In this contribution, we have the following observations and proposals:

Proposal 1: For scenario#1 and scenario #2, define the same fixed scaling factor by N for RRM measurements with longer intervals, e.g., N=2.

Proposal 2: Support option 1 for transition between scenario#1 or scenario#2 and scenario#3.

Proposal 3: Different relaxation for higher priority layer and equal/lower priority layer can be defined for RRM measurement relaxation.

Proposal 4: Do not introduce the RRM measurement relaxation by reducing the number of frequency layer

Proposal 5: EMR frequency layer shall not be relaxed if T331 is running.

- Reference

- R4-2002338, WF on RRM measurement relaxation for Power Saving

- R4-1915946	, WF on RRM measurement relaxation for Power Saving