3GPP TSG-RAN WG4 Meeting #94bis-e-Bis	R4-2004027

Electronics Meeting, 20 – 30 April 2020

Agenda Item:	6.17.1.3

Source:	Ericsson

Title:	RLM for NR HST

Document for:	Discussion

# 1	Introduction

RAN4#94-e agreed with the way forward on RRM for NR high speed train scenario [1], which includes the agreement on RLM as follows. This contribution discusses the remaining issue on RLM for NR HST.

RLM - Q1: whether the outcome on the scaling factor for L3 measurement can be reused for RLM? - Option 1: YES - Option 2: NO - Q2: If the answer to Q1 is NO, whether 1.5x relaxation factor for RLM shall be kept? - Option 1: keep the 1.5x scaling factor - Option2: 1.5x relaxation factor is kept when TSSB >= TBD, 1.5x relaxation factor is removed when TSSB < TBD

# 2	Discussion

## 2.2	SSB based RLM

The existing Rel-15 L1 evaluation period of SSB based Qin/Qout for FR1 is specified as follows:

Table 8.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Since RAN4 has agreed to reuse the Rel-15 RLM requirements for HST scenario, we should keep the same number of SSB/CSI-RS samples, but the remaining issue is whether to keep the relaxation factor of 1.5 with DRX cycle <= 320ms.

In our understanding, the scaling factor of 1.5 comes from the cell measurement requirement to save the UE power consumption especially when SMTC is misaligned with the UE DRX reception cycle. According to TS38.133, the SSB-based L1 evaluation periods for in-synch and out-of-synch become 2,400ms (1.5*5*320ms) and 4,800ms (1.5*10*320ms), respectively, in the case of DRX cycle of 320ms even if P=1. Since Rel-16 NR HST assumes the UE velocity of 500km/h (139m/s), UE moves 139(m/s)*2.4(s)=333m for in-synch and 666ms for out-of-synch within one L1 evaluation period. Considering the inter-RRH distance, e.g. 700m for scenario assumed in UE demodulation performance for HST-SFN, it is expected the large SNR difference during the measurement period. It may cause the wrong Qin/Qout indications because the measurement accuracy evaluation in RAN4 is based on the fixed SNR condition.

We need some enhancement of requirements to facilitate high speed operation, but at the same time it does not seem reasonable for the network to expect that the UE can provide the same performance with 160ms SMTC period as with 5ms SMTC period. Although we think it would be technically feasible to implement a UE without 1.5x scaling factor for any SMTC periodicity and adapt RLM for high speed up to 500km/h, we think network also need to provide sufficient reference symbols for the UE to be implemented in a reasonably power efficient way. We therefore propose:

Proposal 1: For SSB based RLM, 1.5x scaling factor is retained for some SMTC periods.

20ms SMTC periodicity is the period assumed by UEs for initial synchronization, so we think that this will be a commonly used setting in NR. On the other hand, it is reasonable to expect a different tradeoff between power saving in DRX mode versus performance for high speed operation, given that the use of 1.5 scaling factor probably results in insufficient UE performance, at least in HST scenario assuming 500km/h. Hence, we propose to remove 1.5x relaxation factor when TSSB < 40ms and keep the scaling factor with TSSB >= 40ms.

Proposal 2: For SSB based RLM, 1.5x relaxation factor is kept when TSSB >= 40ms, 1.5x relaxation factor is removed when TSSB < 40ms.

## 2.2	CSI-RS based RLM

RAN4#94-e WF discussed the condition of scaling factor for SSB based RLM. Since RLM is also applicable for CSI-RS, RAN4 need to discuss the scaling factor for CSI-RS based RLM in the table below.

Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P)×TCSI-RS) | Max(100, Ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P) × TDRX | Ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Although the minimum evaluation period is same as SSB based RLM, i.e., 200ms and 100ms for Qut-of-synch and In-synch, respectively, CSI-RS based RLM assumes different measurement samples, i.e., (Mout, Min)=(20,10) with CSI-RS density of 3, which is twice the number of samples compared with SSB based RLM. As we discussed in 2.1, the longer evaluation period degrades the measurement performance in high speed train scenario although it could save the UE power consumption. Considering the same logic as SSB based RLM, we propose to remove 1.5x relaxation factor when TCSI-RS < 20ms and keep the scaling factor with TCSI-RS >= 20ms.

Proposal 3: For CSI-RS based RLM, 1.5x relaxation factor is kept when TCSI-RS >= 20ms, 1.5x relaxation factor is removed when TCSI-RS < 20ms.

# 3	Conclusion

Proposal 1: For SSB based RLM, 1.5x scaling factor is retained for some SMTC periods.

Proposal 2: For SSB based RLM, 1.5x relaxation factor is kept when TSSB >= 40ms, 1.5x relaxation factor is removed when TSSB < 40ms.

Proposal 3: For CSI-RS based RLM, 1.5x relaxation factor is kept when TCSI-RS >= 20ms, 1.5x relaxation factor is removed when TCSI-RS < 20ms.

# References

- R4-2002253, “WF on RRM for NR HST”, CMCC.