3GPP TSG-RAN WG4 Meeting #94bis-e		R4-2004075

# Electronic Meeting, 20 – 30 April, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On NRSRP processing in multicarrier operation

Agenda item:	5.11.3.3

Document for:	Discussion and Agreement

Introduction

At RAN4 #94-e, the discussion on RRM measurement aspects for multicarrier operation, as listed in the WI on Additional enhancements for NB-IoT [1] was continued. In the WF [2], the following agreements and open issues are reflected.

Multi carrier operation non-anchor RRM - Whether combining NRSRP samples between anchor carrier and non-anchor carriers are allowed, following two options are under consideration: - Option 1: NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor without any conditions or prerequisites. - Option 2: NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor with the condition is fulfilled, where the condition is FFS.

This contribution investigates the above depicted open issue.

Discussion

The discussion in this section refers to the open issue in the WF on conditions for combining measurements done on anchor carrier and non-anchor carrier.

Combining anchor and non-anchor NRSRP samples

The previously approved WF at RAN4 #93 [3] allows to combine measurements done on anchor carrier and non-anchor carrier by applying the power offset signalled to the UE. The power offset reflects the difference in transmit power on anchor and non-anchor carrier and hence this difference in transmit power is compensated. The combining of measurements on anchor and non-anchor carrier was discussed at RAN4 #93 based on the scenario [4], that a UE may identify the need to perform re-synchronization on the anchor carrier or fallback due to other purposes while still performing serving cell evaluation across DRX cycles on the non-anchor carrier.

Aside the difference in transmit power on both carriers, a difference in the NRSRP samples for both carriers may originate from different path loss conditions for anchor carrier and non-anchor carrier. The path loss may be different

- due to frequency selective fading of the radio channel, in case the channel bandwidth is smaller than the coherence bandwidth of the radio channel and

- due to high intra- and intercell interference for one of the carriers, which will degrade NRSRP estimation (e.g. in the scenario of large channel separation in frequency).

Furthermore, the UE may be in noise limited condition at the cell boundary or even in extreme coverage. Then NRSRP samples taken for both carriers could differ significantly.

If we assume that NRSRP evaluation serves for triggering neighbour cell measurements for cell reselection and for start / abortion of the relaxed monitoring mode, then the combination of NRSRP samples from both carriers should improve measurement accuracy and thus be allowed only conditionally.

Two cases are distinguished.

1) Normal coverage

Assuming normal coverage of the UE, the combination of NRSRP samples from anchor and non-anchor carrier should thus not be allowed, if a significant path loss difference is detected by the UE. To this purpose the UE should compare measurements on anchor and non-anchor carrier, adding to the latter the signalled power offset of the anchor carrier (in logarithmic scale) for a sufficient long measurement period comparable to that for serving cell evaluation. Either concurrent measurements in the same subframes are done or subframe interlaced measurements between anchor and non-anchor carrier.

If the inequation

| NRSRP (anchor) – (NRSRP (non-anchor) + Power_offset_anchor) | < M          (1)

is not satisfied, then the UE should refrain from performing a combination of NRSRP samples from anchor and non-anchor carriers. It is noted that this condition for combining NRSRP samples from anchor and non-anchor carriers has also been discussed in [5] and [6].

The above margin M for combining NRSRP samples in multicarrier operation could have a fixed predefined value such as 3 or 6 dB, taking into account achievable measurement accuracy in the measurement period, alternatively it is signalled by the network to allow network control related to measurement accuracy. For example, the network may temporarily disallow combination of NRSRP samples from anchor and non-anchor carrier by signalling 0 dB. Thus, the signalling option is preferred.

Otherwise if above inequation (1) is satisfied, then combination of NRSRP samples is possible and is based on signalled power offset for anchor carrier.

It was argued during RAN4 #94-e [7], that a large variation of NRSRP can also occur on anchor carrier only measurements at subsequent DRX cycles. While this is true, the anchor carrier may experience different interference conditions than the non-anchor carrier (e.g. experiencing worse conditions). Thus, combining NRSRP from both carriers in this case will degrade NRSRP estimation for a considerable period of time. This issue was previously discussed in context of NRSRQ translation from non-anchor to anchor carrier. In our view the comparison between anchor and non-anchor carrier measurement, in order to remove fast fading component, will require more than 2 measurement samples, as used for serving cell evaluation in TS 36.133, subclause 4.6.2.1, referred to in [7] which is done at least every DRX cycle. However, the frequency of this comparison between NRSRP measurement samples from both carriers could be significantly lower than that for serving cell evaluation, in case of shorter DRX cycles.

2) Enhanced coverage

In enhanced coverage (large CE), the receiver is considerably impacted by noise, and hence high variation of NRSRP measurements taken on either carrier is expected. In this case a combination of NRSRP samples from both carriers, applying the signalled power offset, may be beneficial to improve the measurement accuracy. Thus, the target coverage range as part of the enhanced coverage range in terms of SINR < -6 dB should be identified. The UE then determines that it is in enhanced coverage and is allowed to combine NRSRP samples from both carriers without condition.

Proposals

From above discussion, it is proposed to select option 2 in the WF for UE in normal coverage and option 1 in the WF for UE in enhanced coverage and to modify them accordingly.

The following proposals are made:

Option 1 applies for UE in enhanced coverage: NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor without any conditions or prerequisites, provided that the UE is in enhanced coverage (SINR < -6 dB).

Option 2 applies for UE in normal coverage: NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor, provided that

- the UE is in normal coverage (SINR ≥ -6 dB) and

- the comparison of NRSRP samples between anchor and non-anchor carrier, taken during the same measurement period and accounting for the signal power offset, yields a difference within a margin M (FFS if a fixed predefined value or a value signaled by the network should be applied).

Conclusion

This contribution has discussed the open issue in the WF on conditions for combining measurements done on anchor carrier and non-anchor carrier.in regard to NB-IoT multicarrier operation as included in [1].

Following proposals for agreement are made:

- Option 1 of the WF applies for UE in enhanced coverage: NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor without any conditions or prerequisites, provided that the UE is in enhanced coverage (SINR < -6 dB).

Option 2 of the WF applies for UE in normal coverage: NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor, provided that

- the UE is in normal coverage (SINR ≥ -6 dB) and

- the comparison of NRSRP samples between anchor and non-anchor carrier, taken during the same measurement period and accounting for the signal power offset, yields a difference within a margin M (FFS if a fixed predefined value or a value signaled by the network should be applied).

References

- RP-200293, "WID revision: Additional enhancements for NB-IoT", source: Futurewei

- R4-2002337, "Way forward on RRM requirements of R16 enhancement for NB-IoT", source: Huawei, HiSilicon

- R4-1915890, "Way forward on RRM requirements of R16 enhancement for NB-IoT", source: Huawei, HiSilicon

- R4-1915195, "Remaining issues on RRM measurements in non-anchor carrier for NB-IoT", source: Qualcomm Incorporated

- R4-1914731, "Remaining discussions on non-anchor carrier RRM measurements", source: Ericsson

- R4-2001752, "Remaining discussions on non-anchor carrier RRM measurements", source: Ericsson

- R4-2002296, "Email discussion summary for RAN4#94e_#71_NB_IOTenh3_RRM", source: Moderator (Huawei)