3GPP TSG-RAN WG4 Meeting #94bis-e                                R4-2004133

Electronic Meeting, 20th April – 30th April, 2020

Source: 	Huawei, HiSilicon

Title: 	Updated TP for TR 37.716-31-11 to add UL configuration DC_7C_n78A for DC_1A-7C-28A_n78A

Agenda Item:	8.5.2

Document for:	Approval

Introduction

This contribution provides an update for DC_1-7-28_n78 for TR 37.716-31-11.

- Text Proposal

<TP for TR 37.716-31-11>

- 5.1.24	DC_1A-7A-28A_n78A_BCS0
DC_1A-7C-28A_n78A_BCS0

- 5.1.24.1	Operating bands for DC

Table 5.1.24.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7-28_n781 | CA_1-7-28 | n78 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.1.24.2	Configuration for DC

Table 5.1.24.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-7A-28A_n78A DC_1A-7C-28A_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A | CA_1A-7A-28A CA_1A-7C-28A | n78A |

#### 5.1.24.3	∆TIB and ∆RIB values

For DC_1-7-28_n78 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.24.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.24.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

### 5.1.24.4	REFSENS requirements

No further MSD are needed to be specified for DC_1A-7A-28A_n78A or DC_1A-7C-28A_n78A.

<End of TP >