3GPP TSG-RAN WG4 Meeting#94-e-Bis	R4- 2004212

Online, 20th Apr – 30th Apr, 2020

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for NR CA PDSCH Demodulation Performance Tests

Agenda item:	6.18.1.1

Document for:	Discussion

# Introduction

In this paper, we present simulation results for NR CA as per simulation assumptions in [1].

# FDD 15kHz

Simulation results are as below.

Table 1: Simulation results for FDD 15kHz

| BW | NRx | SNR in dB at 70% of peak throughput for alignment with 4 HARQ processes | SNR in dB at 70% of peak throughput with impairments with 4 HARQ processes | SNR in dB at 70% of peak throughput for alignment with 8 HARQ processes | SNR in dB at 70% of peak throughput with impairments with 8 HARQ processes |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 11.15 | 12.65 | 11.23 | 12.73 |
| 10 | 2 | 11.42 | 12.92 | 11.59 | 13.09 |
| 15 | 2 | 11.36 | 12.86 | 11.54 | 13.04 |
| 20 | 2 | 11.72 | 13.22 | 11.9 | 13.4 |
| 25 | 2 | 12.01 | 13.51 | 12.21 | 13.71 |
| 30 | 2 | 11.94 | 13.44 | 11.87 | 13.37 |
| 40 | 2 | 12.02 | 13.52 | 12.2 | 13.7 |
| 50 | 2 | 12.54 | 14.04 | 12.89 | 14.39 |
| 5 | 4 | 6.15 | 7.65 | 6.26 | 7.76 |
| 10 | 4 | 6.39 | 7.89 | 6.49 | 7.99 |
| 15 | 4 | 6.34 | 7.84 | 6.39 | 7.89 |
| 20 | 4 | 6.31 | 7.81 | 6.37 | 7.87 |
| 25 | 4 | 6.49 | 7.99 | 6.64 | 8.14 |
| 30 | 4 | 6.41 | 7.91 | 6.43 | 7.93 |
| 40 | 4 | 6.64 | 8.14 | 6.61 | 8.11 |
| 50 | 4 | 6.94 | 8.44 | 6.94 | 8.44 |

# TDD 15kHz

Simulation results are as below.

Table 2: Simulation results for TDD 15kHz

| BW | NRx | SNR in dB at 70% of peak throughput for alignment with 4 HARQ processes | SNR in dB at 70% of peak throughput with impairments with 4 HARQ processes | SNR in dB at 70% of peak throughput for alignment with 8 HARQ processes | SNR in dB at 70% of peak throughput with impairments with 8 HARQ processes |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 11.21 | 12.71 | 11.38 | 12.88 |
| 10 | 2 | 11.55 | 13.05 | 11.83 | 13.33 |
| 15 | 2 | 11.49 | 12.99 | 11.73 | 13.23 |
| 20 | 2 | 11.81 | 13.31 | 11.99 | 13.49 |
| 25 | 2 | 12.13 | 13.63 | 12.31 | 13.81 |
| 30 | 2 | 11.83 | 13.33 | 12.07 | 13.57 |
| 40 | 2 | 12.26 | 13.76 | 12.44 | 13.94 |
| 50 | 2 |  |  | 12.71 | 14.21 |
| 5 | 4 | 6.24 | 7.74 | 6.38 | 7.88 |
| 10 | 4 | 6.48 | 7.98 | 6.62 | 8.12 |
| 15 | 4 | 6.41 | 7.91 | 6.49 | 7.99 |
| 20 | 4 | 6.34 | 7.84 | 6.41 | 7.91 |
| 25 | 4 | 6.58 | 8.08 | 6.65 | 8.15 |
| 30 | 4 | 6.36 | 7.86 | 6.45 | 7.95 |
| 40 | 4 | 6.63 | 8.13 | 6.71 | 8.21 |
| 50 | 4 |  |  | 6.79 | 8.29 |

# TDD 30kHz

Simulation results are as below.

Table 3: simulation results for TDD 30kHz

| BW | NRx | SNR in dB at 70% of peak throughput for alignment | SNR in dB at 70% of peak throughput with impairments |
| --- | --- | --- | --- |
| 5 | 2 | 11.53 | 13.03 |
| 10 | 2 | 11.33 | 12.83 |
| 15 | 2 | 11.42 | 12.92 |
| 20 | 2 | 11.58 | 13.08 |
| 25 | 2 | 11.7 | 13.2 |
| 30 | 2 | 11.57 | 13.07 |
| 40 | 2 | 11.83 | 13.33 |
| 50 | 2 | 12.13 | 13.63 |
| 60 | 2 | 12 | 13.5 |
| 80 | 2 | 12.87 | 14.37 |
| 90 | 2 | 12.7 | 14.2 |
| 100 | 2 | 13.21 | 14.71 |
| 5 | 4 | 6.63 | 8.13 |
| 10 | 4 | 6.35 | 7.85 |
| 15 | 4 | 6.39 | 7.89 |
| 20 | 4 | 6.55 | 8.05 |
| 25 | 4 | 6.53 | 8.03 |
| 30 | 4 | 6.43 | 7.93 |
| 40 | 4 | 6.59 | 8.09 |
| 50 | 4 | 6.81 | 8.31 |
| 60 | 4 | 6.72 | 8.22 |
| 80 | 4 | 7.18 | 8.68 |
| 90 | 4 | 7.03 | 8.53 |
| 100 | 4 | 7.46 | 8.96 |

# TDD 120kHz

Simulation results are as below.

Table 4: simulation results for TDD 120kHz

| BW | MCS | Rank | SNR in dB at 70% of peak throughput for alignment | SNR in dB at 70% of peak throughput with impairments |
| --- | --- | --- | --- | --- |
| 50 | 13 | 1 | 5.71 | 7.21 |
| 100 | 13 | 1 | 5.88 | 7.38 |
| 200 | 13 | 1 | 5.76 | 7.26 |
| 400 | 13 | 1 | 6.13 | 7.63 |
| 50 | 10 | 2 | 8.35 | 9.85 |
| 100 | 10 | 2 | 8.15 | 9.65 |
| 200 | 10 | 2 | 8.27 | 9.77 |
| 400 | 10 | 2 | 8.64 | 10.14 |

# Conclusions

This paper presents the simulation results for NR CA as per assumptions in [1].

# References

[1] R4-2002392, “Simulation assumptions for NR normal CA UE performance requirements”, 3GPP TSG RAN WG4 Meeting #94-e, Online, Feb 24 - Mar 6, 2020.