3GPP TSG-RAN WG4 Meeting #94bis-e			  											R4-2004235

Online, 20th Apr. 2020 – 24th Apr. 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NR-U – Wideband operation and Intra-Carrier Guardbands

Agenda item: 	6.1.2.1 Transmitter characteristics [NR_unlic-Core]

Document for:	Discussion and agreement

# Introduction

Definition of guardbands applicable for NR-U wideband operation has been discussed since the start of the work item in multiple contributions. Four WFs have been compiled [1, 2, 3, 4]. The agreed main points the two WFs from RAN4#92 [1] and RAN4#92bis [2] are captured below.

- Define two different categories of guardbands for NR-U:

  - Carrier guardbands (inter-carrier guardbands)

  - In-carrier guardbands (intra-carrier guards)

  - Type-1 Intra-carrier guardband.

  - Type-2 Intra-carrier guardband.

- Define minimum Intra-carrier guard PRBs for a carrier configured by a serving cell as an integer number of full PRBs.

  - The Intra-carrier guard PRBs are defined on a common RB grid which reference point is configured by the gNB as specified in section 5.3.4 of 38.101-1.

  - Point A and first usable PRB are given by the gNB according to Rel-15 mechanisms.

  - No intra-carrier guards are defined for 20 MHz carriers.

- RAN4 is tasked to specify necessary sizes of the inter-carrier guardbands and number of PRBs needed for intra-carrier guards to meet spectral emission mask defined for NR-U.

- The intra-carrier guard band could be scheduled if it is located within the adjacent contiguous LBT successful sub-bands where all scheduled LBT sub-bands have passed.

  - From the coexistence perspective, there is no need for Type-1 intra-carrier GBs.

  - Scheduling in Type-2 intra-carrier GBs is not feasible.

Further at RAN4#93 the third WF [3] was agreed which addresses the second last built point in above. The agreements from [3] are captured in the following.

- For Inter-carrier guardbands reuse the minimum guard-bands definition dependent on configured bandwidth as done for Rel-15 and given in Table 5.3.3-1 (TS 38.101-1).

- The default pattern is that for which the radio requirements apply given the specified channel raster points, starting at the first usable PRB, for usable PRBs and Intra-carrier guard PRBs are given for different SCS and CBW in the following table together with maximum number of RBs:

| SCS | 20MHz Channels | 40MHz Channels |  | 60MHz Channels |  | 80MHz Channels |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz | 106 | 105-6-105 | Max. 216 | N/A |  | N/A |  |
| 30KHz | 51 | 50-6-50 | Max. 106 | 50-6-50-6-50 | Max. 162 | 50-6-50-5-50-6-50 | Max. 217 |
| Alt. 1 60KHz | 24 | [23-5-23] | Max. 51 | [23-5-23-5-23] | Max. 79 | [23-5-23-5-23-5-23] | Max. 107 |
| Alt. 2 60KHz | [25] | [24-3-24] | Max. 51 | [24-3-25-3-24] | Max. 79 | [24-4-24-3-24-4-24] | Max. 107 |

- NR Rel-15 minimum guard-bands definition [Table 5.3.3-1 (TS 38.101-1)] is used as baseline for determining the number of Intra-carrier guard PRBs presented in the table above.

- The number of usable RBs per sub-channel and the number of intra-carrier guard PRBs can for different SCS vary due to configuration according to the following table:

| 15kHz |  | 30kHz |  | 60kHz |  |
| --- | --- | --- | --- | --- | --- |
| Usable RBs | Guard RBs | Usable RBs | Guard RBs | Usable RBs | Guard RBs |
| 104, 105 | 6, 7 | 49, 50, 51 | 5, 6, 7 | [22, 23, 24] | [3, 4, 5] |

- Note that for 20MHz channels and 30kHz SCS only [50] and 51 can be used for number of usable RBs

- Definition of intra-carrier guard PRBs for channel bandwidths larger than 80MHz are FFS

- Alternatives for 60kHz SCS.

  - Alt.1 is based on the existing number of RBs that a UE has to support for 60kHz SCS.

  - Alt.2 is based on the a increased number of RBs which is applicable only for NR-U.

- RB shift for PRB grid alignment FFS.

At RAN#94-e the fourth WF was agreed [4], the agreements herein are given below.

- Freq shift due to RB alignment between component carriers and wideband operation carrier is not needed to be specified

- [Up to 200KHz freq shift due to the allowed ±200 kHz shift of the Wi-Fi channel raster specified in the EN harmonised standard is not precluded in NR-U]

In this contribution, we continue the discussion and addresses some of the FFS and bracket agreements related to Intra-carrier guardbands. Further, the capturing of the existing agreements to specification (38.101-1) are discussed.

# Discussion

Intra-carrier guardbands or more precise intra-carrier guard PRBs are used when wanting to utilize wideband operation but, due to the unlicensed spectrums requirement of LBT, need to be able to cope with parts of the wideband carrier not being available.

Wideband operations utilize a Bandwidth Part (BWP) to define the wideband carrier. For NR Rel-15 the carrier BWPs can be configured only within the usable (full) physical resource blocks (PRB) of a carrier as illustrated in Figure 1.

Figure 1 Configuration of BWP in NR Rel-15

As seen in Figure 1, the Common Resource Blocks (CRBs) are aligned to Point A which as defined in section 4.4.4.2 of 38.211 serves as a common reference point for Common Resource Block (RB) grid on which BWPs are configured. The first usable PRB is configured by the gNB with PRB granularity for the carrier. Therefore, PRB grid of every BWP configured on common RB grid of given SCS is also aligned to Point A.

Inter-carrier

The inter-carrier guardbands, as illustrated in blue in the figure in the introduction, is as agreed in [3] following the NR Rel-15 framework. This meaning that the gNB configures Point A and the first usable PRB of the carrier as illustrated in Figure 1. The first usable PRB needs to be configured such that it has sufficient offset to the carrier edge to allow for the by RAN4 defined minimum inter-carrier GBs given in Hz, given by Table 5.3.3-1 in 38.101-1. An illustration of this is given in Figure 2.

Figure 2 Illustration of inter-carrier parameters, SCS = 30 kHz.

Intra-carrier

To allow the UE to derive the position of the intra-carrier guard PRBs a default guardband design was agreed in [3]. Utilizing this default pattern and the configured Point A the intra-carrier guard PRBs can be placed as illustrated in Figure 3.

Figure 3 Illustration of intra-carrier parameters, SCS = 30 kHz.

In the example shown in Figure 2 and 3, the minimum guardband for an 80 MHz carrier of 925 kHz as given in Table 5.3.3-1 of 38.1010-1 are used for the inter-carrier guardband. For intra-carrier guard PRBs the guardband for a 20 MHz carrier of 805 kHz was used as a baseline when deriving the default and the configurable number of intra-carrier guard PRBs as given in [3]. This as rounding the 805 kHz to full PRBs shows that a minimum of 5 PRBs is needed for the intra-carrier guardband for the 30 kHz SCS.

The default guardbands for both inter-carrier guardband and intra-carrier guard PRBs as given in [3] follows the minimum of the current Rel-15. Under some conditions or in some regions additional guardbands are needed. As already defined for NR Rel-15, if there is a need for additional inter-carrier guardbands this will be signalled as a part of the NS. In a similar manner is it possible to also signal the need for additional intra-carrier guard PRBs as this flexibility is already enabled by design (RAN1#98b Agreement), for example by increasing the intra-carrier guardbands by additional PRB on each side of intra-carrier guardband as per.

Open issues

The remaining agreements needed tor the intra-carrier guard PRBs relates to:

- 60 kHz SCS default guardbands

- Options for configuring usable RB for the 20MHz channels on a carrier larger than 20MHz

1) For the first issue, it is our belief that the discussion relates more to the definition of mandatory UE support of either 24 or 25 RBs and therefrom a baseline operation mode. In our view reusing NR design as much as possible is a compelling argument for adopting the 24 RBs for 60 kHz SCS baseline/default. On the other hand, the possibility of increasing SU by adopting the 25 RBs, as agreed with the restriction of relaxed emission requirements, is something which should be considered. During the RAN4#94-e meeting arguments for choosing both 24 and 25 RBs as a baseline was presented. From a BS point of view, it would be beneficial to choose the 25 RBs but it is understood that from the UE perspective this might not be as straightforward. As 60 kHz is an optional UE feature [5], we have no strong preference for either option at the moment.

Observation 1: 	60 kHz SCS is an optional feature for the UE to support.

2) For the second issue, it is noted in the WF that “for 20MHz channels and 30kHz SCS only [50] and 51 can be used for number of usable RBs”. This means that the option of using 49 RBs are not available for this specific channel bandwidth. However, in the case of needed additional guardband, 50 RBs can be configured, keeping 51 as default to correspond to NR Rel-15 design. We are fine to impose this restriction.

Proposal 1: 	Only 50 or 51 RBs should be configurable for 20 MHz channels and 30 kHz SCS on a carrier larger than 20MHz.

A further point of discussion, not related directly to intra-carrier guard PRBs are:

- Allowing shift of  ±200 kHz of the channel center as allowed for WiFi and by the EN harmonized standard for 5 GHz, EN 301 893.

3) As captured in [4] we do not see a reason to preclude the opportunity of improving channel alignment between operational services by allowing the ±200 kHz shift. It can be noted that there is no need to further specify how this shift can be applied as this is already possible to place the RB grid with an offset to the carrier by the configuration of Point A as described in relation to Figure 1.

Proposal 2: 	Allowing shift (up to ±200 kHz) of channel centre (RB grid) should not be precluded for NR-U.

# Capturing GB design in TS

At RAN4#94-e a draftCR was provided in [6] proposing how to capture the guardband design for NR-U in TS 38.101-1. The draft was not agreed as some parts needed revision and draftCRs was not treated fully at the meeting due to time restrictions. As a result, a draftCRs R4-2004236 for 38.101-1, have been provided for this meeting.

In [6] is was proposed to denote the intra-carrier guard PRBs as intra-cell guard bands. The same naming has been adopted in R4-2004236.

In TS 38.101-1 the addition of the guardband design for NR-U is done by adding a suffix F (Purposely skipping E as that is discussed for V2X) section to 5.3. Even though the agreements, as of now, are only related to band n46 this option means the suffix can be reused and contain all emission requirements specific for NR-U. Also, its seen beneficial as some of the agreements are expected reuse e.g. for a new band in the 6 GHz spectrum. Further, the suffix approach can be reused in section 6.5 for capturing the SEM and other NR-U specific agreements.

# Conclusion

In this contribution, we discussed the definition of guardbands for NR-U and related open issues for wideband operation.

Observation 1: 	60 kHz SCS is an optional feature for the UE to support.

Proposal 1: 	Only 50 or 51 RBs should be configurable for 20 MHz channels and 30 kHz SCS.

Proposal 2: 	Allowing shift (up to ±200 kHz) of channel centre (RB grid) should not be precluded for NR-U.

# References

- R4-1910536	WF on Guardbands for NR-U, Nokia

- R4-1913060 	WF on Guardbands for NR-U, Nokia

- R4-1916160	WF on Guardbands for NR-U, Nokia

- R4-2002746	WF on wideband operation, ZTE

- R4-1910388	WF on spectrum utilization for NR-U, Nokia

- R4-2001320	Nominal intra-cell guard bands for wideband operation, Ericsson