3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004318

# Online, 20th – 30th April, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on L1 SL-RSRP accuracy requirements for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, the L1 SL-RSRP accuracy requirements need to be introduced for NR V2X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To define L1 SL-RSRP accuracy requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The L1 SL-RSRP accuracy requirements will be missing for NR V2X. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.4.3	L1 SL-RSRP Accuracy requirements for FR1

#### 10.4.3.1	Intra-frequency PSCCH-RSRP Absolute Accuracy

The requirements for absolute accuracy of PSSCH-RSRP in this clause apply to a UE performing PSSCH-RSRP measurements on the same frequency as used by operating V2X sidelink communication in FR1.

The accuracy requirements in Table 10.4.3.1-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in 38.101-1 Clause x.x for reference sensitivity are fulfilled.

-	Conditions for PSCCH-RSRP measurements are fulfilled according to Annex B.4.x for a corresponding Band for each relevant PSCCH-DMRS.

Table 10.4.3.1-1: Intra-frequency PSCCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| [4.5] | [9] | [0] | NR_TDD_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_TDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
| [8] | [11] | [0] | NR_TDD_FR1_A, NR_TDD_FR1_G, | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSCCH-DMRS. |  |  |  |  |  |  |  |  |

#### 10.4.2.2	Intra-frequency PSSCH-RSRP absolute Accuracy

The requirements for absolute accuracy of PSSCH-RSRP in this clause apply to a UE performing PSSCH-RSRP measurements on the same frequency as used by operating V2X sidelink communication in FR1.

The accuracy requirements in Table 10.4.3.1-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one or two ports.

-	Conditions defined in 38.101-1 Clause x.x for reference sensitivity are fulfilled.

-	Conditions for PSSCH-RSRP measurements are fulfilled according to Annex B.4.x for a corresponding Band for each relevant PSSCH-DMRS.

Table 10.4.3.2-1: Intra-frequency PSSCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSL |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz | SCSSL = 60 kHz |  |  |
| [4.5] | [9] | [0] | NR_TDD_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_TDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
| [8] | [11] | [0] | NR_TDD_FR1_A, NR_TDD_FR1_G, | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the minimum of the Ês/Iot of PSCCH-DMRS and the Ês/Iot of PSSCH-DMRS |  |  |  |  |  |  |  |  |

<End of Change 1>