3GPP TSG-RAN4 Meeting #94bis-e	R4-2004371

# Electronic Meeting, 20 – 30 April, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | xxxx | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.133 on interruption requirements for CGI reading |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2020-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The interruption requirements need to be specified for CGI reading on LTE cells, when configured by NR PCell when UE is in NE-DC. The interruption requirements need to be specified for CGI reading on NR cells, when configured by in NE-DC, EN-DC and LTE SA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define interruption requirements for CGI reading on LTE cells, when configured by NR PCell when UE is in NE-DC. Define interruption requirements for CGI reading on NR cells, when configured NE-DC, EN-DC and LTE SA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements for CGI reading are missing in 36.133 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.36, 7.32, new section 7.37 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.36.2.x1	Interruption at E-UTRA measurement with autonomous gaps

When UE is configured by NR PCell to identify CGI of an E-UTRA FDD or E-UTRA TDD cell, UE may cause interruptions on PSCell, and SCell(s) in SCG if configured.

During the time period of  as specified in clause 9.4.6 of TS 38.133 [50], over which the UE identifies the new CGI of the E-UTRA cell, the UE shall transmit at least 60 ACK/NACKs on FDD PSCell and each of activated FDD SCell(s), and at least NACK/NACK ACK/NACKs on TDD PSCell and each of activated TDD SCell(s) where NACK/NACK is specified in Table 7.36.2.x1-1, provided that

-	there is continuous DL data allocation,

-	no DRX and no eDRX_CONN cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each subframe,

-	no MBSFN subframes are configured in the PCell or each of activated SCell(s).

Table 7.36.2.x1-1: Number of ACK/NACKs transmitted by the UE during the time for identification of CGI of an E-UTRA cell

| UL/DL configuration | Minimum number of transmitted ACK/NACKs |
| --- | --- |
| 0 (Note 1) | 18 |
| 1 | 35 |
| 2 | 43 |
| 3 | 36 |
| 4 | 39 |
| 5 | 42 |
| 6 | 30 |
| Note 1: When a UE is configured with EIMTA-MainConfigServCell via RRC signalling [2] only this requirement shall apply. |  |

7.36.2.x2	Interruption at NR measurement with autonomous gaps

When UE is configured by NR PCell to identify CGI of an NR cell, UE may cause interruptions on PSCell, and SCell(s) in SCG if configured.

Edtitor’s Note: The exact requirements are TBA

<End of Change 1>

<Start of Change 2>

7.32.2.y	Interruption at NR measurement with autonomous gaps

When UE is configured by PCell or NR PSCell to identify CGI of an NR cell, UE may cause interruptions on PCell, and SCell(s) in MCG if configured.

Edtitor’s Note: The exact requirements are TBA

<End of Change 2>

<Start of Change 3>

7.37	Interruptions at NR measurement with autonomous gaps

7.37.1	Introduction

This section contains the requirements for the interruptions on PCell and activated SCells if configured, when performing inter-RAT NR measurement with autonomous gaps cells.

7.37.2	Requirements

Edtitor’s Note: The exact requirements are TBA

<End of Change 3>