3GPP TSG RAN WG4 Meeting #94bis-e	R4-2004388

Electronic Meeting, 20 – 30 April, 2020

Source:		Rohde & Schwarz

Title:	TP to TR 38.827 to introduce PSP validation MU

Agenda Item:	9.1.3.2

Document for:	Approval

# Introduction

Following the technical discussion and proposal presented in [4], this contribution is introducing the TP for the MU contribution due to the PSP variation over the test volume.

It also corrects some typos on the clause numbering.

# References

- R4-2002482,	“TR 38.827 v1.2.0 NR MIMO OTA”, CAICT, RAN4 #94-e, February 2020

- R4-2004384, “IFF based system for NR FR2 MIMO OTA”, Rohde & Schwarz, RAN4 #94-e-Bis, April 2020

< Unchanged Text Deleted >

< Beginning of Changes >

# B.2	Measurement uncertainty budget for FR2

## B.2.1	Measurement Uncertainty assessment

Table B.2.1-1: Measurement uncertainty budget for FR2 3D-MPAC

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Details in |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Mismatch for measurement process |  |  | U-Shaped | B.2.2.1 |
| 2 | Measure distance uncertainty |  |  | Normal | B.2.2.2 |
| 3 | Quality of quiet zone |  |  | Rectangular | B.2.2.3 |
| 4 | Base Station simulator |  |  | Rectangular | B.2.2.4 |
| 5 | Channel Emulator -absolute value -stability -linearity |  |  | Normal | B.2.2.5 |
| 6 | Amplifier uncertainties |  |  | Rectangular | B.2.2.6 |
| 7 | Random uncertainty |  |  | Normal | B.2.2.7 |
| 8 | Throughput measurement: output level step resolution |  |  | Rectangular | B.2.2.8 |
| 9 | DUT sensitivity drift |  |  | Rectangular | B.2.2.9 |
| 10 | Signal flatness |  |  | Normal | B.2.2.10 |
| 11 | Emulated channel model PAS variation over the test volume |  |  | Gaussian | B.2.2.17 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 12 | Mismatch for calibration process - loopback cable path - system input path - reference antenna |  |  | U-Shaped | B.2.2.11 |
| 13 | Reference antenna positioning misalignment |  |  | Normal | B.2.2.12 |
| 14 | Quality of quiet zone |  |  | Rectangular | B.2.2.3 |
| 15 | Total uncertainty of the Network Analyzer |  |  | Rectangular | B.2.2.13 |
| 16 | Uncertainty of an absolute gain of the calibration antenna |  |  | Normal | B.2.2.14 |
| 17 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  |  | Normal | B.2.2.15 |
| 18 | Offset of the Phase Center of the Reference Antenna |  |  | Normal | B.2.2.16 |

## B.2.2	Measurement error contribution descriptions

#### B.2.2.1	Mismatch for measurement process

This term comes from the mismatch between the system input cables connecting to the base station simulator output port.

#### B.2.2.2	Measure distance uncertainty

The cause of this uncertainty contributor is due to the reduction of distance between the measurement antenna and the DUT. Given that 0.75m is defined as the minimum range length for FR2 3D-MPAC system, this term could be set as 0 dB.

#### B.2.2.3	Quality of quiet zone

The quality of the quiet zone procedure characterizes the quiet zone performance of the anechoic chamber, specifically the effect of reflections within the anechoic chamber including any positioners and support structures.

#### B.2.2.4	Base Station simulator

gNB emulator is used to drive a signal to the channel emulator and then to the device under test. Generally there occurs uncertainty contribution from absolute level accuracy, non-linearity and frequency characteristic of the gNB emulator.

For practical reasons, in a case that a VNA is used as a calibration equipment, gNB emulator is connected to the system after the calibration measurement is performed by the VNA. Hence, the uncertainty on the absolute level of gNB emulator (transmitter device) cannot be assumed as systematic. This uncertainty should be calculated from the manufacturer’s data in logs with a rectangular distribution, unless otherwise informed. Furthermore, the uncertainty of the non-linearity is included in the absolute level uncertainty.

#### B.2.2.5	Channel Emulator

The channel emulator is also working as a signal source in the FR2 MIMO OTA system, therefore there occurs uncertainty contribution from absolute level accuracy, non-linearity, frequency characteristic and stability of the channel emulator. These uncertainty contributions shall be taken from the manufacturer’s data sheet. This uncertainty value shall be the final value after mmWave radio head.

#### B.2.2.6	Amplifier uncertainties

Any components in the setup can potentially introduce measurement uncertainty. It is then needed to determine the uncertainty contributors associated with the use of such components. For the case of external amplifiers, the following uncertainties should be considered but the applicability is contingent to the measurement implementation and calibration procedure.

-	Stability

-	An uncertainty contribution comes from the output level stability of the amplifier. Even if the amplifier is part of the system for both measurement and calibration, the uncertainty due to the stability shall be considered. This uncertainty can be either measured or determined by the manufacturers’ data sheet for the operating conditions in which the system will be required to operate.

-	Linearity

-	An uncertainty contribution comes from the linearity of the amplifier since in most cases calibration and measurements are performed at two different input/output power levels. This uncertainty can be either measured or determined by the manufacturers’ data sheet.

-	Noise Figure

-	When the signal goes into an amplifier, noise is added so that the SNR at the output is reduced with regard to the SNR of the signal at the input. This added noise introduces error on the signal which affects the Error Rate of the receiver thus the EVM (Error Vector Magnitude). An uncertainty can be calculated through the following formula:

-	Where SNR is the signal to noise ratio in dB at the signal level used during the sensitivity measurement.

-	Mismatch

-	If the external amplifier is used for both stages, measurement and calibration the uncertainty contribution associated with it can be considered systematic and constant -> 0dB. If it is not the case, the mismatch uncertainty at its input and output shall be either measured or determined by the method described in [7].

-	Gain

-	If the external amplifier is used for both stages, measurement and calibration the uncertainty contribution associated with it can be considered systematic and constant -> 0dB. If it is not the case, this uncertainty shall be considered.

#### B.2.2.7	Random uncertainty

This contribution is used to account for all the unknown, unquantifiable, etc. uncertainties associated with the measurements. Random uncertainty MU contributions are normally distributed. The random uncertainty term, by definition, cannot be measured, or even isolated completely. A value of 0.2dB aligned with FR1 is suggested.

#### B.2.2.8	Throughput measurement: output level step resolution

The cause of this uncertainty contributor is due to the step size in the power level used in the throughput measurement stage. Depending on the system provider implementation, the power level adjustment is based on changing the output power of BS simulator or channel model emulator. Fixed 0.5dB step is defined for NR MIMO OTA testing, an uncertainty contribution of 0.25dB with a rectangular distribution should be reported.

#### B.2.2.9	DUT sensitivity drift

Due to statistical uncertainty of throughput measurement, drift in the TRMS can not be monitored. An uncertainty value of 0.2dB can be used, or the TRMS drift should be measured, with a setup corresponding to the actual MIMO OTA measurement.

#### B.2.2.10	Signal flatness

For wireless technologies with wide channel bandwidths, the test system might not have a flat frequency response across the entire channel. While the range calibration corrects for any variation of frequency response as a function of the center frequency of the channel, the broadband radiated power measured or delivered to the test zone will be a function of the entire channel bandwidth as opposed to just the center frequency. Thus, any deviation of the rest of the channel from the signal level at the center frequency will result in an error in the measured result. The determination of the MU element is FFS.

#### B.2.2.11	Mismatch for calibration process

During calibration stage, there will be impendence mismatch between the various RF cables and components used within the system. Standing waves are created by the reflections between any two components and uncertainty in the signal level will be generated.  In general, three mismatch for calibration process should be considered:

- Loopback cable path: This item comes from the mismatch between the reference cable and the loopback cable during the loopback cable measurement step.

- System input path: This item comes from the mismatch between the loopback cable and the system input cable (generally the output cable after BS simulator). The reflectivity of the source output port is measured at the end of the loopback cable connecting to the system input cable.

- Reference antenna: This item comes from the mismatch between the VNA input port and the reference antenna. The reflectivity of the VNA input port is measured at the end of the reference cable connecting to the reference antenna.

#### B.2.2.12	Reference antenna positioning misalignment

This contribution originates from reference antenna alignment and pointing error. In this measurement if the maximum gain directions of the reference antenna and the receiving antenna are aligned to each other, this contribution can be considered negligible and therefore set to zero.

#### B.2.2.13	Total Uncertainty of the Network Analyzer

This contribution originates from all uncertainties involved transmission magnitude measurement (including drift and frequency flatness) with a network analyser. The uncertainty value will be indicated in the manufacturer's data sheet. It needs to be ensured that appropriate manufacturer's uncertainty contribution is specified for the absolute levels measured.

#### B.2.2.14	Uncertainty of an absolute gain of the calibration antenna

The calibration antenna only appears in calibration phase (Stage 1). Therefore, the gain uncertainty has to be taken into account. This uncertainty will come from a calibration report with traceability to a National Metrology Institute with measurement uncertainty budgets generated following the guidelines outlined in internationally accepted standards.

#### B.2.2.15	Positioning and pointing misalignment between the reference antenna and the receiving antenna

This contribution originates from reference antenna alignment and pointing error. In this measurement if the maximum gain direction of the reference antenna and the transmitting antenna are aligned to each other, this contribution can be considered negligible and therefore set to zero.

#### B.2.2.16	Offset of the Phase Center of the Reference Antenna

Gain is defined at the phase centre of the antenna. If the phase centre of the calibration antenna is not aligned at the centre of the set up during the calibration, then there will be uncertainty related to the measurement distance.

#### B.2.2.17	Emulated channel model PAS variation over the test volume

<Editor’s note: the final content for this clause depends on the PSP validation procedure to be defined in clause 7.4.1.6>

[The emulated PAS experience variations over the test volume with regards to the reference PAS depending on the range length, probe weights and system type (IFF or DFF). The uncertainty related to how the emulated PAS varies over the test volume with respect to the reference PAS for an actual test system is assessed by the PSP validation procedure described in clause 7.4.1.6.

The final result of this validation can be expressed in terms of PSP Standard Deviation in percentage over the test volume, and then the equivalent value in dB is obtained as follows:

PSP Standard Deviation dB=10log1+PSP Standard Deviation %

]

< End of Changes >