3GPP TSG RAN WG4 Meeting #94bis-e	R4-2004389

Electronic Meeting, 20 – 30 April, 2020

Source:		Rohde & Schwarz

Title:	Discussion on Dynamic Geometry for NR FR2 MIMO OTA

Agenda Item:	9.1.3.2

Document for:	Approval

# Introduction

In the last meeting, [4] was presented to discuss the framework to reuse the 3D MPAC system defined for NR FR2 MIMO OTA to implement dynamic geometry-based testing, according to one of the objectives in the SID [5]. The following information was captured in the Way Forward [1]:

- FR2 dynamic testing

- A dynamic geometry-based MIMO OTA testing approach has not been thoroughly studied as the second priority in the SI.

- Clarification of the FR2 test method is not for dynamic testing can be added in the TR38.827 test method part.

In this contribution we analyse the options to adapt current definition of 3D MPAC systems as per TR 38.827 [2] to implement such dynamic geometry-based testing.

# Discussion

As shown in [4], current test procedures for RRM and Demod do not consider any sort of UE beam management while the UE is moving with respect to the environment, simulated either by 1 or maximum 2 AoA depending on the test case.

In case of FR2 MIMO OTA, currently defined in TR 38.827 [2], the test procedure requires a NR MIMO sensitivity search at each test point and thus call drops and re-connections are to be considered as part of the normal execution, what prevent this kind of testing to represent the aforementioned dynamic geometry-based testing.

Additionally, the latest version of TR 38.827 [2] only defines MIMO Throughput as the figure of merit but, with respect to dynamic testing scenarios, the MIMO Throughput definition needs to be re-discussed since one of the major points of dynamic testing would be RRM testing. Additional RRM metrics in regards to e.g. RLM would need to be added.

As mentioned in [4] and discussed during last meeting [3], complex scenarios where AoD changes are implemented or multiple TRxP (BS) are considered should be avoided due to the high level of complexity that they will imply from OTA test setup point of view. In addition, it is also desired to hold the assumptions made to derive current channel models in TR 38.827 [2], so the number of probes and layout can be also reused.

It should be noted that any continuous movement during measurements should be avoided since it will affect the relative directions and Doppler effects, and thus inherently change the characteristics of the channel model. However, it will still be necessary to define how quickly the test system needs to change from one test direction to another. This will have a large impact on test system hardware, mainly the positioner, and feasible speeds need to be investigated. Also the impact of this will likely also affect which scenarios can be tested, so it should be taken into account as well.

Taking all this into account, further discussion to define dynamic geometry-based testing should focus on the following topics:

- Test Trajectory: Assuming a relative orientation between the UE and the TRxP, like current test method for LTE and NR FR1 MIMO OTA (i.e. DMSU, DMP and DML), it is possible to define a set of “most common” directions or trajectories that the UE will experience based on the channel model. The number of points per test trajectory shall also be defined, as well as the speed in which the test directions need to be changed

- Selection of test directions: For current MIMO OTA testing, 36 test directions are defined in TR 38.827 [2]. For dynamic testing, it should be discussed whether these directions can be reused as part of the test trajectory or if it is necessary to define different directions. How to define these different directions is TBD.

- DUT testing condition: Current approach for FR2 testing considers only one DUT testing condition (P0), since the final result is an average across points evenly distributed over the whole sphere, equivalent to some sort of “MIMO OTA spherical coverage” map. Since a test trajectory is to be defined, the relative orientation between the DUT and the channel is critical.

- Measurement time per point: As mentioned above, any continuous movement during measurement should be avoided. In addition, depending on the target metric, a minimum time is required for measurements at each point in the selected trajectory. It seems clear that a step-by-step approach is the better compromise here. The measurement time per point (e.g. number of frames) shall be defined.

- Figure of Merit: Discuss the definition of MIMO throughput in dynamic testing scenarios and the addition of new metrics with respect to RRM testing. Such metrics have to be in accordance to the UE Demod and RRM requirements that will be defined later on.

- Fixed DL level: In order to avoid, or at least minimize, the call drops and re-connections, it is recommended to consider a fixed DL level at all points in the test trajectory.

Proposal 1: For the definition of dynamic geometry-based testing focus on the items #1 to #6 above.

# References

- R4-2002471,	“WF on finalizing FR2 MIMO OTA”, CAICT, Keysight, RAN4 #94-e, February 2020

- R4-2002482,	“TR 38.827 v1.2.0 NR MIMO OTA”, CAICT, RAN4 #94-e, February 2020

- R4-2002526, “Email discussion summary for RAN4#94e_#97_FS_NR_MIMO_OTA_test”, Moderator (CAICT), RAN4 #94-e, February 2020

- R4-2000080, “Dynamic geometry-based MIMO OTA Testing”, Qualcomm Incorporated, RAN4 #94-e, February 2020

- RP-182691, “Revised SID: Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs,” CAICT, OPPO, Samsung, RAN #82, December 2018