3GPP TSG-RAN WG4 Meeting #94bis-e 	R4-2004538

Online, 20 Apr - 01 May 2020

Source: 	Huawei

Title: 	TP to TR 37.941 - Spurious emissions MU value derivation sub-clause 12 update (FR2)

Agenda Item:	6.19.5

Document for:	Approval

# Introduction

This TP updates the MU value derivation sections for the FR2 out of band spurious emissions requirements.

It uses the pre-approved tables agreed in R4-2004529.

# TP to TR 39.941 v1.0.0

--- Start of changes ---

#### 12.2.2.4	MU value derivation, FR2

As opposed to FR1, for FR2 the IAC, CATR and reverberation chamber were analysed separately for the MU value derivation. Table 12.2.2.4-1 captures derivation of the expanded measurement uncertainty values for OTA TX spurious emissions measurements in IAC (Normal test conditions, FR2).

Up to 40GHz the OBUE MU values are assumed.

Table 12.2.2.4-1: IAC MU value derivation for TX spurious emissions, FR2

| UID | Uncertainty source |  | Distribution of the probability | Divisor based on distribution shape | ci |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 40GHz<f ≤60GHz |  |  |  | 40GHz<f ≤60GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the AAS BS and the reference antenna | 0.02 | Rectangular | 1.73 | 1 | 0.01 |
| A1-2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.00 | Rectangular | 1.73 | 1 | 0.00 |
| A1-3 | Quality of quiet zone | 0.10 | Gaussian | 1.00 | 1 | 0.10 |
| A1-4a | Polarization mismatch between the AAS BS and the receiving antenna | 0.02 | Rectangular | 1.73 | 1 | 0.01 |
| A1-5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | Rectangular | 1.73 | 1 | 0.00 |
| A1-6 | Phase curvature | 0.05 | Gaussian | 1.00 | 1 | 0.05 |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - low power (UEM) | 0.60 | Gaussian | 1.00 | 1 | 0.60 |
| A1-16 | test system frequency flatness | 0.25 | Gaussian | 1.00 | 1 | 0.25 |
| A1-24 | Uncertainty of the LNA | 0.00 | Gaussian | 1.00 | 1 | 0.00 |
| A1.25 | Uncertainty of the Mixer | 2.25 | Gaussian | 1.00 | 1 | 2.25 |
| A1-7 | Impedance mismatch in the receiving chain | 0.42 | U-shaped | 1.41 | 1 | 0.30 |
| A1-8 | Random uncertainty | 0.10 | Rectangular | 1.73 | 1 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer | 0.57 | U-shaped | 1.41 | 1 | 0.40 |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.43 | Rectangular | 1.73 | 1 | 0.25 |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer. | 0.57 | U-shaped | 1.41 | 1 | 0.40 |
| A1-3 | Quality of quiet zone | 0.10 | Gaussian | 1.00 | 1 | 0.10 |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna | 0.02 | Rectangular | 1.73 | 1 | 0.01 |
| A1-5 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | Rectangular | 1.73 | 1 | 0.00 |
| A1-6 | Phase curvature | 0.07 | Gaussian | 1.00 | 1 | 0.07 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.30 | Gaussian | 1.00 | 1 | 0.30 |
| A1-12 | Influence of the reference antenna feed cable | 0.18 | Rectangular | 1.73 | 1 | 0.10 |
| A1-13 | Reference antenna feed cable loss measurement uncertainty | 0.10 | Gaussian | 1.00 | 1 | 0.10 |
| A1-14 | Influence of the receiving antenna feed cable | 0.18 | Rectangular | 1.73 | 1 | 0.10 |
| C1-4 | SGH Calibration uncertainty | 0.52 | Rectangular | 1.73 | 1 | 0.30 |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | Rectangular | 1.73 | 1 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  | 2.49 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  | 4.88 |
| TRP Summation error |  |  |  |  |  | 0.75 |
| Total MU |  |  |  |  |  | 4.94 |

.

--- Next change ---

#### 12.2.3.3	MU value derivation, FR2

Table 12.2.3.3-1 captures derivation of the expanded measurement uncertainty values for OTA TX spurious emissions measurements in CATR (Normal test conditions, FR2).

Table 12.2.3.3-1: CATR value derivation for TX spurious emissions, FR2

| UID | Uncertainty source |  | Distribution of the probability | Divisor based on distribution shape | ci |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 40GHz<f ≤60GHz |  |  |  | 40GHz<f ≤60GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| A2-18 | Misalignment DUT & pointing error (TRP) | 0.30 | Exp. normal | 2.00 | 1 | 0.15 |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - low power (UEM) | 0.60 | Gaussian | 1.00 | 1 | 0.60 |
| A2-19 | Uncertainty of the LNA | 0.00 | Gaus | 1.00 | 1 | 0.00 |
| A2-20 | Uncertainty of the Mixer | 2.25 | Gaus | 1.00 | 1 | 2.25 |
| A2-2a | Standing wave between DUT and test range antenna | 0.21 | Gaussian | 1.00 | 1 | 0.21 |
| A2-3 | RF leakage, test range antenna cable connector terminated. | 0.00 | Gaussian | 1.00 | 1 | 0.00 |
| A2-4a | QZ ripple with DUT | 0.09 | Gaussian | 1.00 | 1 | 0.09 |
| A2-12 | Frequency flatness | 0.25 | Gaussian | 1.00 | 1 | 0.25 |
| A2-21 | Miscellaneous uncertainty | 0.00 | Rectangular | 1.73 | 1 | 0.00 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| C1-3 | Network Analyzer | 0.30 | Gaussian | 1.00 | 1 | 0.30 |
| A2-5a | Mismatch of receiver chain | 0.57 | U-shaped | 1.41 | 1 | 0.40 |
| A2-5b | Mismatch of receiver chain for low power | 0.00 | U-shaped | 1.41 | 1 | 0.00 |
| A2-6 | Insertion loss variation in receiver chain | 0.18 | Rectangular | 1.73 | 1 | 0.10 |
| A2-3 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Gaussian | 1.00 | 1 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.29 | U-shaped | 1.41 | 1 | 0.21 |
| C1-4 | SGH Calibration uncertainty | 0.52 | Rectangular | 1.73 | 1 | 0.30 |
| A2-8 | Misalignment positioning system | 0.00 | Exp. normal | 2.00 | 1 | 0.00 |
| A2-1b | Misalignment of calibration antenna and test range antenna | 0.00 | Exp. normal | 2.00 | 1 | 0.00 |
| A2-9 | Rotary joints | 0.00 | U-shaped | 1.41 | 1 | 0.00 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | U-shaped | 1.41 | 1 | 0.06 |
| A2-4b | QZ ripple calibration antenna | 0.01 | Gaussian | 1.00 | 1 | 0.01 |
| A2-11 | Switching uncertainty | 0.43 | Rectangular | 1.73 | 1 | 0.25 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  | 2.45 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  | 4.81 |
| TRP Summation error |  |  |  |  |  | 1.20 |
| Total MU |  |  |  |  |  | 4.96 |

--- Next change ---

#### 12.2.4.3	MU value derivation

Table 12.2.3.4-1 captures derivation of the expanded measurement uncertainty values for OTA TX spurious emissions measurements in Reverberation Chamber (Normal test conditions).

Table 12.2.4.3-1: Reverberation Chamber  value derivation for TX spurious emissions, 380 MHz – 26 GHz

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

Table 12.2.4.3-2: Reverberation Chamber value derivation for TX spurious emissions, 18 GHz – 60 GHz

| UID | Uncertainty source |  | Distribution of the probability | Divisor based on distribution shape | ci |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 40GHz<f ≤60GHz |  |  |  | 40GHz<f ≤60GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - low power (UEM) | 0.60 | Gaussian | 1.00 | 1 | 0.60 |
| A6-1 | Impedance mismatch in the receiving chain | 0.45 | U-shaped | 1.41 | 1 | 0.32 |
| A6-2 | Random uncertainty | 0.10 | Rectangular | 1.73 | 1 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| A6-3 | Reference antenna radiation efficiency | 0.50 | Normal | 1.00 | 1 | 0.50 |
| A6-4 | Mean value estimation of reference antenna radiation efficiency | 0.27 | Normal | 1.00 | 1 | 0.27 |
| C1-3 | Network Analyzer | 0.30 | Gaussian | 1.00 | 1 | 0.30 |
| A6-5 | Influence of the reference antenna feed cable | 0.20 | Normal | 1.00 | 1 | 0.20 |
| A6-6 | Mean value estimation of transfer function | 0.27 | Normal | 1.00 | 1 | 0.27 |
| A6-7 | Uniformity of transfer function | 1.50 | Normal | 1.00 | 1 | 1.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  | 1.80 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  | 3.53 |

--- Next change ---

### 12.2.5	Maximum accepted test system uncertainty

{FR1 updated in accompanying TP}

For FR2 only the frequency range 40GHz<f ≤60GHz has been considered in this subclause, the other frequency ranges are common with existing FR1 and FR2 in-band TRP emissions measurements

- 30MHz<f≤6 GHz the MU values are the same as for FR1,

- 6GHz<f ≤40GHz The MU values are the same as the in-band TRP emission measurements in subclause 11.2

Table 12.2.5-2: Spurious emissions MU values – FR2

|  | Expanded uncertainty [dB] |  |  |
| --- | --- | --- | --- |
|  | 30MHz<f≤6 GHz | 6GHz<f ≤40GHz | 40GHz<f ≤60GHz |
| General Directional Chamber | x | x | 4.94 |
| CATR | x | x | 4.96 |
| Reverberation Chamber | x | x | 3.53 |
| Common maximum accepted test system uncertainty | 2.5 | 2.7 | 5 |

--- End of changes ---