3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004623

Online, 20th April – 30th April 2020

Source:	Ericsson

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_7_n8-n78

Agenda item:	8.7.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 to add DC_7_n8-n78 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_7_n8-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n8-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n8A-n78A | DC_7A_n8A DC_7A_n78A | 7A | CA_n8A-n78A |

### 6.x.3	Co-existence studies

Co-existence analysis shows that there are IMD2 and IMD4 impact from DC_7_n8 UL to NR Band n78 DL.

Co-existence analysis shows that there is IMD2 and IMD5 impact from DC_7_n78 UL to NR Band n8 DL.

### 6.x.4	∆TIB and ∆RIB values

For DC_7_n8-n78 the TIB,c and RIB,c values are same as for DC_7-8_n78 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n8-n78 | 7 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n8-n78 | 7 | 0 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 6.x.4	MSD

MSD values for IMD2 and IMD5 impact from DC_7_n78 UL to NR Band n8 DL are reused from DC_7A-8A_n78A

MSD values for IMD2 impact from DC_7_n8 UL to NR Band n78 DL are reused from CA_1A-7A-28A.

MSD values for IMD4 impact from DC_7_n8 UL to NR Band n78 DL are reused from CA_19A-21A-42A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n8A-n78A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 7 | 2535 | 5 | 25 | 2655 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 28.0 | IMD2 |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | 13.0 | IMD4 |

##### ---End of changes---

# Reference

[1]	RP-200080, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics