3GPP TSG-RAN WG4 Meeting #94-bis-e 	R4-2004669

# Electronic Meeting, April 20 – April 30, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurement report mapping for PRS RSTD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Measurement report mapping is missing for PRS RSTD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement report mapping is introduced for PRS RSTD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement report mapping is missing for PRS RSTD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: 10.1.X1, 10.1.X2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.1.X1	DL RSTD measurement accuracy requirements

### 10.1.X2	DL RSTD measurement report mapping

To report an RSTD measurement, the UE shall report a reference quantity value RSTD_i based on Table 10.1.X2-1 and a relative quantity value RSTD_delta_i based on Table 10.1.X2-2. The relative quantity value RSTD_delta_i is chosen from Table 10.1.X2-2 so that the difference between the measured RSTD quantity and the lower bound of its corresponding range from Table 10.1.X2-1 is between  and + resolutionStep, where

resolutionStep=2k,

is the measured relative quantity from Table 10.1.X2-1 for which mod(RSTD,2k)==0 and <step_i, where step_i corresponds to RSTD_i in Table 10.1.X2-1,

kmin≤k≤kmax,

kmin and kmax are as defined in Table 10.1.X2-1.

If the value of k is configured by LMF but it is not in [kmin,kmax], the UE determines the applicable k-value closest to the configured k, otherwise the UE uses the configured k.

Editor’s note: FFS when the value of k is not configured.

The mapping between the measured quantity and the reported reference quantity values is defined in Table 10.1.X2-1.

The reporting range of the reference RSTD is defined from -1539164Tc to 1539164Tc with the step of 64Tc when the absolute value of the measured RSTD is less or equal to 409664Tc and the step of 564Tc when the absolute value of the measured RSTD is greater than 409664Tc, where Tc is defined in TS 38.211 [6].

Table 10.1.X2-1: Reference quantity report mapping

| Reported Reference Quantity Value RSTD_i | Measured Quantity Value RSTD | Reporting step, step_i | Unit | k |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | kmin | kmax |
| RSTD_0000 | -1539164 > RSTD | - | Tc | 5 (FR1), 3 (FR2) | 5 |
| RSTD_0001 | -1539164 RSTD < -1538664 | 564Tc | Tc | 5 (FR1), 3 (FR2) | 5 |
| … |  |  | … |  |  |
| RSTD_2258 | -410664 RSTD < -410164 | 564Tc | Tc | 5 (FR1), 3 (FR2) | 5 |
| RSTD_2259 | -410164 RSTD < -409664 | 564Tc | Tc | 5 (FR1), 3 (FR2) | 5 |
| RSTD_2260 | -409664 RSTD < -409564 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| RSTD_2261 | -409564 RSTD < -409464 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
|  |  |  | … |  |  |
| RSTD_6353 | -364 RSTD -264 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| RSTD_6354 | -264 RSTD -164 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| RSTD_6355 | -164 RSTD 0 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| RSTD_6356 | 0 < RSTD 164 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| RSTD_6357 | 164 < RSTD 264 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| RSTD_6358 | 264 < RSTD 364 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| … | … |  | … |  |  |
| RSTD_10450 | 409464 < RSTD 409564 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| RSTD_10451 | 409564 < RSTD 409664 | 164Tc | Tc | 3 (FR1), 0 (FR2) | 5 |
| RSTD_10452 | 409664 < RSTD 410164 | 564Tc | Tc | 5 (FR1), 3 (FR2) | 5 |
| RSTD_10453 | 410164 < RSTD 410664 | 564Tc | Tc | 5 (FR1), 3 (FR2) | 5 |
| … | … |  | … |  |  |
| RSTD_12709 | 1538164 < RSTD 1538664 | 564Tc | Tc | 5 (FR1), 3 (FR2) | 5 |
| RSTD_12710 | 1538664 < RSTD 1539164 | 564Tc | Tc | 5 (FR1), 3 (FR2) | 5 |
| RSTD_12711 | 1539164 < RSTD | - | Tc | 5 (FR1), 3 (FR2) | 5 |

The mapping between the measured relative quantity and the repoted relative quantity values is defined in Table 10.1.X2-2.

Table 10.1.X2-2: Relative quantity report mapping

| Reported Relative Quantity Value RSTD_delta_i | Measured Relative Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_delta_0 | 0 | Tc |
| RSTD_delta_1 | 1 | Tc |
| RSTD_delta_2 | 2 | Tc |
| … | … | … |
| RSTD_delta_63 | 63 | Tc |
| RSTD_delta_64 | 64 | Tc |
| RSTD_delta_65 | 80 | Tc |
| RSTD_delta_66 | 96 | Tc |
| RSTD_delta_67 | 112 | Tc |
| RSTD_delta_68 | 128 | Tc |
| RSTD_delta_69 | 144 | Tc |
| RSTD_delta_70 | 160 | Tc |
| RSTD_delta_71 | 176 | Tc |
| RSTD_delta_72 | 192 | Tc |
| RSTD_delta_73 | 208 | Tc |
| RSTD_delta_74 | 224 | Tc |
| RSTD_delta_75 | 240 | Tc |
| RSTD_delta_76 | 256 | Tc |
| RSTD_delta_77 | 272 | Tc |
| RSTD_delta_78 | 288 | Tc |
| RSTD_delta_79 | 304 | Tc |

Editor’s note: report mapping for differential PRS RSTD reporting is FFS.