3GPP RAN WG4 Meeting #94bis-e		R4-2004693

Online, 20 April – 30 April 2020

Agenda item:	8.20.1

Source:	Apple Inc.

Title:	A-MPR Proposal for n1

WI/SI:	NR_n65_BW-Core

Release:	Rel-16

Document for:	Approval

# 1	Introduction

A way forward was agreed in RAN4#94-e [1] to contribute simulations for n65 with 50MHz channel bandwidth. This paper presents A-MPR values.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions for the simulations were used:

- Power Class 3

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

- Filter rejection at B3 RX: 37dB

- Filter rejection at B34 RX: 0 dB

The simulations were done for 50MHz channel bandwidth with center frequency of 1945MHz and 1980MHz. The spurious co-existence was according to table

Table1: Co-existence for n65

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n65 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

###### 2.2 Simulation results

Table 2: Regions for n65

| Channel BW (MHz) | Carrier Frequency Fc (MHz) | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 50 | 1945 | ≤ 9 | > 0 | A3 |
|  |  | > 9, < 37.8 | ≥ 9 | A4 |
|  |  | > 9, < 37.8 | < 9 | A5 |
|  |  | ≥ 37.8 | > 0 | A3 |
| 50 | 1945 < Fc ≤ 1980 | ≤ 18 | > 0 | A1 |
|  |  | > 18, < 27 | > 0 | A2 |
|  |  | ≥ 27 | > 0 | A1 |

Table 3: A-MPR for n65

| Waveform | Modulation | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer / Inner | Outer / Inner | Outer / Inner | Outer / Inner | Outer / Inner |
| DFT-s-OFDM | PI/2 BPSK | 14.5 | 10.0 | 14.0 | 5.0 | 5.0 |
|  | QPSK | 14.5 | 10.0 | 14.0 | 7.0 | 5.0 |
|  | 16QAM | 14.5 | 10.0 | 14.5 | 7.0 | 5.0 |
|  | 64QAM | 14.5 | 10.0 | 14.5 | 7.0 | 5.0 |
|  | 256QAM | 14.5 | 10.0 | 14.5 | 7.0 | 5.0 |
| CP-OFDM | QPSK | 16.5 | 11.5 | 14.5 | 8.5 | 6.0 |
|  | 16QAM | 16.5 | 11.0 | 14.5 | 8.5 | 6.0 |
|  | 64QAM | 16.5 | 11.5 | 14.5 | 8.5 | 6.0 |
|  | 256QAM | 16.5 | 11.5 | 14.5 | 8.5 | 6.0 |

Proposal: Use regions and values found in Table 2 and Table 3 to define n65 A-MPR.

# 3  Simulations

# 4	Conclusions

Proposal: Use regions and values found in Table 2 and Table 3 to define n65 A-MPR.

# 4	References

- R4-2002856, “WF on UE RF requirements for adding channel BW to band n65”, Ericsson