3GPP TSG-RAN WG4 Meeting #94bis	R4-2004806

E-meeting, 20 April – 1 May, 2020

Agenda item:	6.5.3.4

Source: 	Qualcomm Incorporated

Title: 	IAB-MT RLM requirements

Document for:	Discussion

# Introduction

During the last meeting, RAN4 made a couple of agreements regarding RLM. Several issues are still open for discussion. We discuss those issues and provide our opinions in this contribution.

# Evaluation Period for RLM

RAN4 discussed the evaluation period of RLM measurement during the last meeting [1]. Several options were proposed and a couple of them are shown below. The first option proposed to relax RLM evaluation period. The second option proposed to keep the period same as Rel-15. The third option proposed to keep this issue FFS till RAN4 RF session finalizes details of different IAB-MT classes.

Open Issues from [1] Issues: Relaxed RLM requirement. Proposals: Apply relaxed RLM requirement of NR UE to the requirement of IAB-MT during no-DRX case. - Increase the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period. Issues: Reuse of Rel-15 requirements. Proposals: Reused the SSB and CSI-RS based RLM evaluation period of Rel-15 UEs to define the requirements for IAB-MT nodes.

IAB-MTs will be much more stationary than UEs. Hence, it makes sense to relax the evaluation period of IAB-MTs than that of Rel-15 UEs.

Observation 1: Reusing Rel-15 RLM evaluation period works for IAB-MTs. But, since Rel-16 IAB-MTs are stationary, relaxing RLM evaluation period saves power and reduces complexity of IAB-MTs without harming its link monitoring performance.

Proposal 1: Support relaxed RLM evaluation period requirement for IAB-MTs compared to that for NR UEs.

- Increase the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period.

# Sharing factor of RLM evaluation period

During the last meeting, there was a proposal to remove the sharing factor P from RLM evaluation periods [2]

Open Issues from [1] Issues: Sharing factor P. Proposals: Remove the sharing factor P in Evaluation Period calculation for IAB RLM requirement.

One of the motivation of this proposal is the following: since requirements for intra-frequency and inter-frequency measurements have not been configured for IAB-MTs, configuration of measurement gap is not necessary. This means that the sharing factor P will not be necessary, too.

However, although RAN4 has not allowed measurement requirements, the signalling for intra and inter-frequency measurement is still allowed by RAN1/RAN2. As a result, network can configure these measurement gaps and the IAB-MTs that support this feature can follow those configurations. Hence, sharing factor P does not need to be removed from the spec. If network doesn’t configure measurement gap due to the lack of RAN4 requirements, value of P will be 1, in any case.

Observation 2: Although RAN4 has not allowed measurement requirements, network can still configure these measurement gaps and the IAB-MTs that support this feature can follow those configurations.

Proposal 2: Sharing factor P does not need to be removed from the RLM evaluation period of IAB-MTs.

# Density of CSI-RS for RLM requirements

During the last meeting, one company proposed to use density =1 for RLM-CSI-RS [2]

Open Issues from [1] Issues: Density of CSI-RS to define CSI-RS based RLM requirements. Proposals: “For IAB RLM requirement, the case where Evaluation Period for CSI-RS of RLM with Density=1 should be taken into consideration.”

The proponent argued that the delay spread of IAB-MTs would be much shorter than that of UEs. Hence, density = 1 would work for RLM-CSI-RS.

However, this assumption may not hold for local area IAB-MTs because they might be located in NLOS scenarios.

The assumption may somewhat hold for wide area IAB-MTs but: I) A reduced density of CSI-RS will also reduce total CSI-RS power (assuming that the tones FDMed with CSI-RS are used for other purposes) and II) defining two different types of CSI-RS resources for checking the performance of two different IAB-MT classes may further complicate the RRM tests.

Observation 3: Local area IAB-MTs are likely to be in NLOS scenarios and may experience similar delay spread as UEs.

Observation 4: A reduced density of CSI-RS will also reduce total CSI-RS power (assuming that the tones FDMed with CSI-RS are used for other purposes).

Proposal 3: Use RLM-CSI-RS with density = 3 for local area IAB-MTs.

Discuss further if it is essential to introduce RLM-CSI-RS with density = 1 for wide area IAB-MTs.

# Other Requirements for RLM

Other RLM requirements (e.g. scheduling availability, measurement restrictions, etc.), that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

Proposal 4: The following RLM related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

- Measurement restrictions for SSB based RLM and CSI-RS based RLM

- Minimum requirement at transitions

- Minimum requirement for L1 indication.

- Scheduling availability during RLM

# Conclusion

Observation 1: Reusing Rel-15 RLM evaluation period works for IAB-MTs. But, since Rel-16 IAB-MTs are stationary, relaxing RLM evaluation period saves power and reduces complexity of IAB-MTs without harming its link monitoring performance.

Observation 2: Although RAN4 has not allowed measurement requirements, network can still configure these measurement gaps and the IAB-MTs that support this feature can follow those configurations.

Observation 3: Local area IAB-MTs are likely to be in NLOS scenarios and may experience similar delay spread as UEs.

Observation 4: A reduced density of CSI-RS will also reduce total CSI-RS power (assuming that the tones FDMed with CSI-RS are used for other purposes).

Proposal 1: Support relaxed RLM evaluation period requirement for IAB-MTs compared to that for NR UEs.

- Increase the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period.

Proposal 2: Sharing factor P does not need to be removed from the RLM evaluation period of IAB-MTs.

Proposal 3:

- Use RLM-CSI-RS with density = 3 for local area IAB-MTs.

- Discuss further if it is essential to introduce RLM-CSI-RS with density = 1 for wide area IAB-MTs.

Proposal 4: The following RLM related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

- Measurement restrictions for SSB based RLM and CSI-RS based RLM

- Minimum requirement at transitions

- Minimum requirement for L1 indication.

- Scheduling availability during RLM

# Reference

- R4-2002307, “Email discussion summary for RAN4#94e_#53_NR_IAB_RRM”, Qualcomm Incorporated.

- R4-2000889, Samsung.