3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004841

Electronic Meeting, 20 – 30 Apr., 2020

Source:	ZTE

Title:	RRC connection control for IAB MTs

Agenda item:	6.5.3.2

Document for:	Discussion

# 1	Introduction

In RAN4 94e, companies already reached consensus to a large extent regarding RRC connection control for IAB-MTs. However, the proposed WF was noted, not approved.

In the suggested WF, the following content is captured:

Issue 4: Necessity of defining RRC re-establishment delay requirement when the periodicity of SMTC window is larger than 160 ms and the serving cell is in low SNR. - Agreement: IAB MT is not required to successfully identify a cell when the SMTC window is larger than 160ms and the serving cell is in low SNR Issue 5: Necessity of defining RRC re-establishment requirement when the SSB transmission periodicity is larger than 160 ms. - Agreement: There is no requirement for RRC re-establishment for IAB-MTs if the SSB transmission periodicity is larger than 160 ms. Issue 6: Necessity of defining RRC release with re-direction requirement when the periodicity of SSB is greater than 160 ms Agreement: There is no requirement for RRC release with re-direction when the periodicity of SSB is greater than 160 ms.

Thus, for this meeting, we submit a paper again to confirm our point of view. The proposals in this paper are consistent with the suggested WF in last meeting.

- RRC Connection Control

The suggested WF makes sense to us based on the reasons that the IAB-MT can assume that the SSB periodicity for initial access is not larger than 160 ms. Thus, no requirement shall be made if the SSB periodicity is larger than 160 ms.

Confirm the WF from last meeting that IAB MT is not required to successfully identify a cell when the SMTC window is larger than 160ms and the serving cell is in low SNR.

Confirm the WF from last meeting that there is no requirement for RRC re-establishment for IAB-MTs if the SSB transmission periodicity is larger than 160 ms.

Confirm the WF from last meeting that there is no requirement for RRC release with re-direction when the periodicity of SSB is greater than 160 ms.

An issue which is not captured in the suggested WF is whether to extend cell identification time. It seems intuitive to us that since IAB-MTs assume an SSB periodicity of 160 ms while the current UE requirements are made assuming 20 ms SSB periodicity, the requirements should be extended by a factor of 8.

Cell identification time shall be extended by a factor of 8 compared to UE requirements.

3	Conclusion

- Proposal 1: Confirm the WF from last meeting that IAB MT is not required to successfully identify a cell when the SMTC window is larger than 160ms and the serving cell is in low SNR.

- Proposal 2: Confirm the WF from last meeting that there is no requirement for RRC re-establishment for IAB-MTs if the SSB transmission periodicity is larger than 160 ms.

- Proposal 3: Confirm the WF from last meeting that there is no requirement for RRC release with re-direction when the periodicity of SSB is greater than 160 ms.

- Proposal 4: Cell identification time shall be extended by a factor of 8 compared to UE requirements.

References