3GPP TSG-RAN WG4 Meeting #94bis-e		            R4-2004914

Electrical Meeting, 20th April – 30th April, 2020

Agenda item:	6.17.2.1.2

Source:	Samsung

Title:	Simulation results for UE demodulation requirements under HST SFN

Document for:	Discussion

# Introduction

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we provide simulation results for HST – SFN channels based on the assumptions agreed in RAN4#94e found in [1].

# Common test setup

Table1: Simulation assumption for HST-SFN channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 13 based on 64QAM table |  |
| Propagation condition | HST-SFN |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=700m, Dmin=150m |  |
| Rank | Rank = 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | - Option 1: 870Hz - Option 2: 851Hz | - Option 1: 1500Hz - Option 2: 1667Hz |
| Testing metric | SNR @70% of maximum throughput |  |

# Simulation results

## TDD Simulation results

Figure 3-2:TDD Simulation results for NR PDSCH under HST SFN

## Summary of @ 70% of max throughput

Table 2: Ideal Simulation results for NR PDSCH under HST-SFN channel (to be updated)

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 870 | 2Tx 2Rx ULA Low | 13 |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  | 2Tx 4Rx ULA Low | 13 |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  | 851 | 2Tx 2Rx ULA Low | 13 |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  | 2Tx 4Rx ULA Low | 13 |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 | 40MHz/30kHz | 1500 | 2Tx 2Rx ULA Low | 13 | 11.8 |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |
| 16 |  |  | 2Tx 4Rx ULA Low | 13 | 9.1 |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  | 1667 | 2Tx 2Rx ULA Low | 13 | 12.3 |
| 20 |  |  |  |  |  |
| 21 |  |  |  |  |  |
| 22 |  |  | 2Tx 4Rx ULA Low | 13 | 9.5 |
| 23 |  |  |  |  |  |
| 24 |  |  |  |  |  |

# Conclusion

In this contribution we provide simulation results for HST – SFN channels.

# Reference

- R4-1915926 , Draft WF on UE demodulation for NR HST, RAN4#93, CMCC

- R4-2002418, WF on UE demodulation for NR HST, RAN4#94e, CMCC