3GPP TSG-RAN WG4 Meeting #94bis-e		            R4-2004916

Electrical Meeting, 20th April – 30th April, 2020

Agenda item:	6.17.2.1.4

Source:	Samsung

Title:	Simulation results for UE demodulation requirements under fading channel with high Doppler value

Document for:	Discussion

# Introduction

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we provide simulation results for TDLC channel based on the assumptions agreed in RAN4#94e found in [2] for alignment

# Common test setup

Table1: Simulation assumption for multi-path fading channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | - 600Hz | - 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |

# Simulation results

## FDD Simulation results

Figure 3-1:     Simulation results for NR PDSCH FDD under multi-path fading TDLC channel

## TDD Simulation results

Figure 3-2:     Simulation results for NR PDSCH TDD under multi-path fading TDLC channel

Observation 1:  The SNRs with 70% TP for MCS 4, 13, 17 are feasible under Doppler value with 600Hz and 1200Hz for Rank 1.

With considering the test complexity, we suggest to choose one MCS for requirement with multi-path fading channel under high Doppler value, i.e., MCS 13.

Proposal 1:  Introduce MCS 13 for requirement with multi-path fading channel under high Doppler value.

## Summary of @ 70% of max throughput

Table 2: Ideal Simulation results for NR PDSCH under multi-path fading channel

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | Rank | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 1 | 4 | -1.5 |
| 2 |  |  |  |  | 13 | 7.1 |
| 3 |  |  |  |  | 17 | 11.7 |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  | 2Tx 4Rx ULA Low | 1 | 4 | -4.1 |
| 8 |  |  |  |  | 13 | 3.2 |
| 9 |  |  |  |  | 17 | 6.9 |
| 10 |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 13 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 1 | 4 | -2.1 |
| 14 |  |  |  |  | 13 | 7.2 |
| 15 |  |  |  |  | 17 | 10.9 |
| 16 |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |
| 19 |  |  | 2Tx 4Rx ULA Low | 1 | 4 | -4.6 |
| 20 |  |  |  |  | 13 | 4.1 |
| 21 |  |  |  |  | 17 | 8.0 |
| 22 |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |

# Conclusion

In this contribution we provide simulation results for Multi-Path Fading channels – TDLC channel for alignment.

Observation 1:  The SNRs with 70% TP for MCS 4, 13, 17 are feasible under Doppler value with 600Hz and 1200Hz for Rank 1.

Proposal 1:  Introduce MCS 13 for requirement with multi-path fading channel under high Doppler value.

# Reference

- R4-1915926 , Draft WF on UE demodulation for NR HST, RAN4#93, CMCC

- R4-2002418, WF on UE demodulation for NR HST, RAN4#94e, CMCC