3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004940

Electronic Meeting, 20 – 30 Apr., 2020

Source:	ZTE

Title:	L1-RSRP measurement in NR-U

Agenda item:	6.1.5.11

Document for:	Discussion

# 1	Introduction

In RAN4 94-e, the topic of L1-RSRP measurement delay requirements have been discussed. Companies’ views were summarized in WF [1], the content copied below:

- This paper discusses the open issues listed above in the WF and provide our view.

2	Pending issues

First, we agree with Option 1 in the WF (blue part) that CSI-RS L1-RSRP measurements follow the same approach of SSB L1-RSRP measurements. The L1-RSRP measurements configured with CSI-RS or SSB face similar issues regarding LBT failures, thus it makes most sense to re-use the mechanism for SSB based L1-RSRP measurement. We’re fine with the proposal that the same number of samples are also used for CSI-RS based L1-RSRP measurement.

CSI-RS L1-RSRP measurements follow the same approach of SSB L1-RSRP measurements, which means same number of samples for both SSB based L1-RSRP measurement and CSI-RS and same extension value L1 and L1-max as in SSB based L1-RSRP.

Regarding whether to extend the semi-persistent L1-RSRP reporting delay under NR-U, the discussions were more diversified during the last meeting. There are two sub-topics which companies haven’t reached consensus, to which we provide our views here.

New Issue 1-4-2: Detailed UE behavior when receiving the MAC CE deactivation command for semi-persistent CSI reporting, in case of UL LBT failure for sending the ACK

We think under this case, it only makes sense for the UE to remain in its old state (activated). This is because from the network perspective, no ACK message could mean the UE fails to receive / decode the MAC-CE deactivation command. In this case, it’s only logical for the network to assume that the UE would remain in activated state, to avoid mismatch between the network and the UE. During RAN4 94-e, it was pointed out by some company [2] that the network may have allocated the UL resource to another UE after sending the MAC-CE deactivation command. In our view there is such a probability, however it’s up to the network to schedule UEs in uplink so the described scenario can be avoided. This seems like a corner case to us.

If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE continues to be in its previous state, i.e., it should measure and report L1-RSRP until it successfully transmits HARQ-ACK.

Issue 1-3: Semi-persistent L1-RSRP reporting delay

Similarly, in our view we should reuse R15 delay requirements. We don’t see a lot of difference between periodic / aperiodic reporting and semi-persistent reporting. Moreover, it could be hard to decide on how to extend such reporting delay since this could possibly mean that the network will allocate longer UL resource in time domain in case the UE encounters with UL LBT failure. This would seem a waste of resources and will decrease the efficiency.

Semi-persistent L1-RSRP reporting delay in NR-U reuses the Rel-15 reporting delay.

3	Conclusion

- Proposal 1: CSI-RS L1-RSRP measurements follow the same approach of SSB L1-RSRP measurements, which means same number of samples for both SSB based L1-RSRP measurement and CSI-RS and same extension value L1 and L1-max as in SSB based L1-RSRP.

- Proposal 2: If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE continues to be in its previous state, i.e., it should measure and report L1-RSRP until it successfully transmits HARQ-ACK.

Proposal 3: Semi-persistent L1-RSRP reporting delay in NR-U reuses the Rel-15 reporting delay.

References

- R4-2002278, WF on NR-U RRM requirements (Part 3), Nokia, Nokia Shanghai Bell

- R4-2001361, Ericsson