3GPP TSG-RAN WG4 Meeting # 94-e-Bis 												R4-2005260

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			5.10.3

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [94e Bis][128] LTE_eMTC5_RRM

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

The open issues of release 16 MTC RRM work item are addressed in this email discussions. In particular, following subtopics are addressed: PUR, RSS, DL quality reporting, MPDCCH improvement. First priority is given to the completing the open issues belonging to the core part of the WI, and second priority is given to the discussions on performance requirements.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Collect companies view on the open issues for sub-features. If there are no open issues for the sub-features, collect comments for the CRs submitted

- 2nd round: Reach technical agreements on the open issues and assign CRs to companies.  If possible, high-level agreement on performance requirements can be reached.

# Topic #1: Mobility enhancement

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003565 | Qualcomm Incorporated | Proposal 1: In idle mode, RSS-based RSRP measurement requirements apply for serving cell only if the RSS of serving cell - share the same NB as that of paging occasion MPDCCH for N successive DRX cycles, and - RSS occasion precedes paging occasion MPDCCH for N successive DRX cycles (i.e., no overlap), (FFS: the min/max allowed distance between the last subframe of RSS and first subframe of MPDCCH), and - RSS power offset with respect to CRS is equal to or greater than 0 dB, and - RSS-based measurement period is not longer than CRS-based measurement period Where N=3 for normal coverage and N=5 for enhanced coverage. Proposal 2: In idle mode, RSS-based RSRP measurement requirements apply for intra-frequency neighbor cell only if the RSS of corresponding neighbor cell - share the same NB as that of paging occasion MPDCCH for N successive DRX cycles, and - at least two RSS subframes exists immediately before the first subframe of paging occasion MPDCCH , or immediately after the last subframe of paging occasion, for N successive DRX cycles (FFS: min/max distance of start/end of RSS occasion w.r.t end/start of MPDCCH), and - RSS power offset with respect to CRS is equal to or greater than 0 dB, and - RSS-based measurement period is not longer than CRS-based measurement period Where N=3 for normal coverage and N=5 for enhanced coverage. Proposal 3: In connected mode, RSS-based RSRP measurement requirements apply for either serving or intra-frequency neighbor cell only if - RSS frequency location of the cell being measured occurs in the NB(s) that UE monitors for serving cell MPDCCH/PDSCH for N successive RSS occasions, and - At least two successive subframes of RSS occasion of the cell being measured are available to UE during in the NB(s) that UE monitors, and during its DRX OnDuration (if configured), for N successive RSS occasions, and - RSS power offset with respect to CRS is equal to or greater than 0 dB Where N=3 for normal coverage and N=5 for enhanced coverage. Proposal 4: UE is NOT required to perform RSS-based RSRP measurement in RRC_CONNECTED mode for neighbour cells that are not in the neighbour cell list with signalled RSS parameters. |
| R4-2004380 | Huawei, HiSilicon | Proposal 1: The RSS measurement requirements apply provided that - RSS from neighbour cell is with same time offset as serving cell RSS - RSS from neighbour cell is within same 2-RB layer as serving cell RSS - For connected mode, RSS does not collide with measurement gap in time Proposal 2: RAN4 to further discuss the applicability of delay requirements for RSS measurement and CRS measurement. |
| R4-2004185 | Ericsson | Proposal 1: We support the proposal in [4], i.e. for a UE that supports RSS-based RSRP measurement, UE shall be required to use RSS for RSRP measurement of a serving or neighbour cell in RRC_CONNECTED mode and meet the corresponding accuracy requirements only if: - RSS frequency location of the cell being measured occurs in the NB(s) that UE monitors for MPDDCH for the N number of samples, and - RSS time location of the cell being measured does not coincide with UE’s measurement gap (if configured), and - RSS power offset of the cell being measured is not smaller than 0 dB Proposal 2: L1 measurement period for RSS based RSRP measurement is defined as 480 ms and 800 ms in normal and enhanced coverage respectively for BL and non-BL UEs. |
| R4-2004186 | Ericsson | Draft CR to support RSS for CE level selection. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: RSS Measurement conditions

Sub-topic description:

The measurement conditions for IDLE- and CONNECTED mode for serving- and neighbor cell measurements are treated in this subtopic.

Open issues and candidate options before e-meeting:

Issue 1-1: IDLE mode

Option 1 (serving cell measurement):

RSS-based RSRP measurement requirements apply for serving cell only if the RSS of serving cell

- share the same NB as that of paging occasion MPDCCH for N successive DRX cycles, and

- RSS occasion precedes paging occasion MPDCCH for N successive DRX cycles (i.e., no overlap),  (FFS: the min/max allowed distance between the last subframe of RSS and first subframe of MPDCCH), and

- RSS power offset with respect to CRS is equal to or greater than 0 dB, and

- RSS-based measurement period is not longer than CRS-based measurement period

Where N=3 for normal coverage and N=5 for enhanced coverage.

Option 2 (neighbour cell measurements):

In idle mode, RSS-based RSRP measurement requirements apply for intra-frequency neighbor cell only if the RSS of corresponding neighbor cell

- share the same NB as that of paging occasion MPDCCH for N successive DRX cycles, and

- at least two RSS subframes exists immediately before the first subframe of paging occasion MPDCCH , or immediately after the last subframe of paging occasion, for N successive DRX cycles (FFS: min/max distance of start/end of RSS occasion w.r.t end/start of MPDCCH), and

- RSS power offset with respect to CRS is equal to or greater than 0 dB, and

- RSS-based measurement period is not longer than CRS-based measurement period

Where N=3 for normal coverage and N=5 for enhanced coverage.

Option 3 (neighbour cell measurements):

The RSS measurement requirements apply provided that

- RSS from neighbour cell is with same time offset as serving cell RSS

- RSS from neighbour cell is within same 2-RB layer as serving cell RSS

- Recommended WF

  - More discussions needed.

Issue 1-2: CONNECTED mode

Option 1:

In connected mode, RSS-based RSRP measurement requirements apply for either serving or intra-frequency neighbor cell only if

- RSS frequency location of the cell being measured occurs in the NB(s) that UE monitors for serving cell MPDCCH/PDSCH for N successive RSS occasions, and

- At least two successive subframes of RSS occasion of the cell being measured are available to UE during in the NB(s) that UE monitors, and during its DRX OnDuration (if configured), for N successive RSS occasions, and

- RSS power offset with respect to CRS is equal to or greater than 0 dB

Where N=3 for normal coverage and N=5 for enhanced coverage.

Option 2:

For connected mode, RSS does not collide with measurement gap in time

Option 3:

UE is NOT required to perform RSS-based RSRP measurement in RRC_CONNECTED mode for neighbour cells that are not in the neighbour cell list with signalled RSS parameters.

- Recommended WF

  - More discussions needed.

### Sub-topic 1-2: RSS measurement delays

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: IDLE delay

- Proposals 1

  - L1 measurement period for RSS based RSRP measurement is defined as 480 ms and 800 ms in normal and enhanced coverage respectively for BL and non-BL UEs.

- Proposal 2

  - RAN4 to further discuss the applicability of delay requirements for RSS measurement and CRS measurement.

- Recommended WF

  - The number of samples to use for measurements was agreed as 3 and 5 for normal and enhanced coverage. How to translate that in to the measurement delays in IDLE and CONNECTED mode needs discussions.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Qualcomm | Issue 1-1: we support option 1 for serving cell and option 2 for neighbor cells. In our view, option 3 is not clear enough. What does same time offset mean? If it means same start and end for each RSS occasion, it seems an unnecessarily restrictive condition. As long as there is overlap for 2 subframes, measurement should be possible. Also, what is the relationship of RSS with respect to PO in option 3? And our view is still that as long as RSS (2-PRB) falls in the same NB (6-PRB) for PO, UE should be able to measure. Issue 1-2: we cannot agree on Proposal 1. Proposal 1 means UE has to violate its DRX operation and wake up just for the sake of RSS measurement. To us, improving measurement accuracy via RSS is secondary to saving in power consumption. |
| Ericsson | Sub topic 1-1: For the serving cell measurement and neighbor cell measurements, our preference is to define the measurements under the condition that RSS is available in N DRX cycles, where N=3 for normal coverage and N=5 for enhanced coverage. Since the UE is in IDLE mode, UE wakes up prior to the paging reception and perform the measurement. We can also agree to set RSS power offset with respect to CRS is equal to or greater than 0 dB and that RSS-based measurement period is not longer than CRS-based measurement period. Sub topic 1-2: We agree to option 1. Option 2 needs more clarification. Does it mean that RSS measurements are defined without using measurement gaps? If so, we have similar understanding. Sub topic 1-3: The measurement delay of course depends on the configured RSS periodicity, and the delay can be derived assuming 3 samples in normal coverage and 5 samples in enhanced coverage. One approach is to scale the current delays with the RSS periodicity, this is agreeable to us. |
| Huawei, HiSilicon | Issue 1-1: IDLE mode For serving cell measurement, - we are fine with condition 1 - we need to further check on condition 2 as it is a bit restrictive from network perspective - we are fine with condition 3 - we have one question on condition 4: What does measurement period mean in idle mode – is it Nserv for serving cell evaluation? For neighbor cell measurement, - we have a bit different understanding here, and in our view the requirements should only apply for neighbor cell RSS in the same 2-RB as the serving cell RSS. Neighbor cell RSS in a different 2-RB, even in the same NB, is considered as a different frequency layer because it requires separate resources for sampling, processing and filtering, and we don’t think UE should be required to handle multiple frequency layers for RSS. - we agree with Qualcomm comment that same time offset as serving cell in option 3 may be unnecessarily restrictive, but condition 2 in option 2 may be also restrictive from network perspective so we need to further check on this - we are fine with condition 3 - we have same question on condition 4: What does measurement period mean in idle mode – is it Tdetect, Tmeasure or Tevaluate for intra-frequency reselection? Issue 1-2: CONNECTED mode We think at least option 2 is needed. To Ericsson comment – we meant the RSS requirements are defined only if RSS measurement does not require measurement gap (otherwise it will impact existing RRM requirements) and is not colliding with measurement gap in time domain (if they are colliding UE cannot measure RSS because it retunes to center 6-PRB). For option 1: - condition 1 is similar as option 2, but we are not sure about NB for PDSCH, e.g. what happens if UE is not scheduled? - condition 2, similar as idle mode, we need some time to check - we are fine with condition 3. We are also fine with option 3, but is it also needed for idle mode? We also notice that there is no condition proposed related to measurement period for connected mode, does it mean RSS requirements apply even its measurement period is longer than CRS based? If so, which measurement period should apply e.g. for event triggering? Issue 1-3: IDLE delay (seems the numbering is wrong in section 1.2) Proposal 1 is defining the measurement period as fixed number, but should it be based on the RSS periodicity? For proposal 2, we just want to confirm if it is correct understanding that UE is supposed to measure both CRS and RSS no matter which measurement period requirement applies? How about the delay in connected mode? |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004186 | Huawei, HiSilicon: The CR is a bit confusing, as an implication is that RSS based measurement is not used for other tasks such as serving cell evaluation, intra-freq neighbour cell measurement for reselection. |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Conditions for RSS based RSRP measurement in IDLE mode serving cell measurements: - if the RSS of serving cell share the same NB as that of paging occasion MPDCCH for N successive DRX cycles, and - RSS power offset with respect to CRS is equal to or greater than 0 dB, and - RSS-based measurement period is not longer than CRS-based measurement period, definition of measurement period in IDLE mode will be discussed further in 2nd round. Conditions for RSS based RSRP measurement in IDLE mode neighbor cell measurements: - RSS power offset with respect to CRS is equal to or greater than 0 dB, and For CONNECTED mode measurements: - RSS power offset with respect to CRS is equal to or greater than 0 dB Measurement delays in IDLE: - RSS-based measurement period is not longer than CRS-based measurement period Measurement delays in CONNECTED: - RSS-based measurement period is not longer than CRS-based measurement period - Existing measurement period is to be scaled with the RSS periodicity assuming 3 samples for normal coverage and 5 samples for enhanced coverage. Candidate options: Recommendations for 2nd round: To be discussed in 2nd round for RSS based RSRP measurement in IDLE mode neighbor cell measurements: Companies are encouraged to provide the views on whether the remaining conditions shall apply: For IDLE mode serving cell measurements: - Companies are encouraged to provide views on whether RSS occasion precedes paging occasion MPDCCH for N successive DRX cycles (i.e., no overlap), (FFS: the min/max allowed distance between the last subframe of RSS and first subframe of MPDCCH), and For IDLE mode neighbor cell measurements: - More discussions needed and companies are encouraged to provided comments earlier to facilitate the discussions. To be discussed in the 2nd round for CONNECTED mode measurements: FFS on : RSS frequency location of the cell being measured occurs in the NB(s) that UE monitors for serving cell MPDCCH/PDSCH for successive RSS occasions, and FFS on: At least two successive subframes of RSS occasion of the cell being measured are available to UE during in the NB(s) that UE monitors, and during its DRX OnDuration (if configured), for successive RSS occasions, and FFS on: RSS does not collide with measurement gap in time To be discussed in the 2nd round for measurement delays in IDLE: Existing measurement period (Nserv) is to be scaled with the RSS periodicity assuming 3 samples for normal coverage and 5 samples for enhanced coverage. To be discussed in the 2nd round for measurement delays in CONNECTED: - Existing measurement period is to be scaled with the RSS periodicity assuming 3 samples for normal coverage and 5 samples for enhanced coverage. To be discussed in 2nd round: FFS: is the UE supposed to measure both CRS and RSS no matter which measurement period requirement applies? |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Preconfigured uplink resources

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2004187 | Ericsson, Nokia, Nokia Shanghai Bell | Proposal: Tsearch = 5 SF in normal coverage and enhanced coverage when in DRX. |
| R4-2004188 | Ericsson, Nokia, Nokia Shanghai Bell | This CR contains the PUR RRM requirements as agreed in earlier way forwards [R4-1907733, R4-1910107, R4-1912735, R4-1915889]. |
| R4-2004378 | Huawei, HiSilicon | Proposal 1: RAN4 do not need to further specify the UE synchronization requirements before PUR. Proposal 2: RAN4 does not define additional measurement requirements for RSRP1. |
| R4-2004379 | Huawei, HiSilicon | CR to introduce UE measurement requirements for RSRP change based TA validation. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: Open issues of PUR

Sub-topic description:

In current agreements Tsearch is FFS when the UE is not configured with eDRX [R4-1907733]. Tsearch is the time required for performing tracking loop and exact duration.

Open issues and candidate options before e-meeting:

Issue 2-1:

- Proposals

  - Tsearch = 5 SF in normal coverage and enhanced coverage when in DRX.

- Recommended WF

  - Try to agree to set Tsearch to 5 SFs for normal coverage and enhanced coverage when in DRX.

### Sub-topic 2-2: How to capture the agreements in specification

Sub-topic description:

Two CRs were presented at last meeting to capture the PUR agreements. They differed quite a lot, and companies were encouraged to go through the agreements that were made during the whole WI phase.

Open issues and candidate options before e-meeting:

Issue 2-2:

- Proposals

  - Option 1: CR which captured all agreements as presented in R4-2004188

  - Option 2: CR which captured only subset of the agreements as presented in R4-2004379

- Recommended WF

  - Further discussions are needed on how to capture the previous agreements, see R4-2004187 and R4-2004378.

  - Companies are to review the CRs and provide their input.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 2-1: we cannot agree to the proposal and the WF. More details in issue 2-2. Issue 2-2: In our view, the options are expressed a little subjectively. If we understand correctly, one of the main sources of disagreement is whether to capture the synchronization clause or not. It is true that RAN4 made some agreements in May 2019 meeting about this but we still believe that a simple statement that mandates UE to meet transmit timing accuracy for transmission in PUR occasion would suffice. We would also like to bring up that in May 2019 when RAN4 were discussing the synchronization issue, there were yet no agreements on what the minimum periodicity of PUR occasions could be in RAN1/2. That’s why RAN4 discussed the issue in detail. Now, we know what they are. In eMTC, the minimum periodicity of PUR occasion is 8x10.24 = 81.92s (please refer to requestedPeriodicity-r16 in PURConfigurationRequest-r16 in TS 36.331). The maximum synchronization time that is captured in R4-2004188 is less than 1s. So synchronization time is significantly smaller than PUR periodicity. In this case, we believe specifying when UE should be allowed to transmit and when UE should drop a PUR occasion is not necessary. UE should always be able to transmit if it has data to transmit because it has ample time to synchronize. Our preference is still with the approach in option 2 but perhaps a few other things not related to synchronization in option 2 is missing that should be added. |
| Ericsson | We are trying to understand the technical motivation against the synchronization part covered in section 4.7.3.2 in CR R4-2004188. For us, the most important thing is to avoid any PUR transmission if the UE is not synchronized towards the cell and this agreement is important to capture in the Cr. But if companies are not very happy with the exact delays as agreed, that can be discussed. For example, if company feels that Tsearch is too short for normal DRX and eDRX, that can be discussed. We also agree that there are other agreements not related to the synchronization that are missing in option 2, but they have been captured in option 1. If we have missed any, please let us know. Having said this, we hope companies can stick to the previous agreements and we can make better progress in this area in this meeting. |
| Huawei, HiSilicon | Issue 2-1: We do not agree with the recommended WF. We do not the point to define Tsearch, as we discussed in our discussion paper. Issue 2-2: We have similar view as Qualcomm on the need for synchronization requirements. We also want to clarify from Qualcomm and Ericsson more specifically what is missed in option 2. |
| Ericsson | Issue 2-1 and Issue 2-2: In order to make progress, we are open to discuss the exact delays if companies have concern on the agreed values. Since companies spent a lot of valuable time and effort in analyzing and discussing the PUR feature which eventually resulted in the above agreements, it is important that they are respected and properly captured the CR. On what part is missing in R4-2004379, please read R4-2004187 from Ericsson, Nokia, Nokia Shanghai Bell. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004188 | Company A |
|  | Company B |
| R4-2004379 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Not possible to make any agreement at this stage, more discussions are needed. Candidate options: Recommendations for 2nd round: Continue the discussions from the 1st round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: MPDCCH Improvement

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003564 | Qualcomm Incorporated | Observation 1. Precoder granularity of 1 PRB in frequency domain in distributed mode is not meaningful. Per RAN1 agreement in RAN1#99, there is a fixed predetermined relationship defined in both frequency and time domain for distributed mode which is followed in our simulation results. Observation 2. (RL, AL) pair of (2,4) is not valid for CE Mode B. This permutation is not simulated. Therefore, in-sync results for Set 1 in CE Mode B are absent. Observation 3. Simulation results show the most significant gain from R16 MPDCCH performance improvement is seen in SNR operating points less than -10 dB and in AWGN channel in RLM scenarios. Proposal 1: RAN4 does not consider defining new RLM test cases based on R16 MPDCCH performance improvement in SNR points above -10 dB or in fading channel conditions. Proposal 2: RAN4 to discuss the possibility of adding new RLM test cases based on R16 MPDCCH performance improvement in SNR points below -10 dB and AWGN channel. |
| R4-2004016 | Ericsson | Observation: Performance gain of MPDSCH with DMRS+CRS is more than 1dB compared with DMRS-only MPDCCH, except for In-synch with static channel condition. Proposal: When the network configures the enhanced RLM (rlm-ReportConfig) and improved MPDCCH (mpdcch-crs-connected-config), UE applies the improved MPDCCH transmission parameters for evaluating the out-of-synch when UE reports the Event E1 to the network. This is applicable for both CE Mode A UE and CE Mode B UE. |
| R4-2004017 | Ericsson | Draft CR to capture the enhanced MPDCCH requirement |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: SNR test points

Sub-topic description:

Simulation results have been presented in R4-2003564 and R4-2004016. Based on the observations made, it seems the view from Qualcomm and Ericsson to apply RLM based on improved MPDCCH when channel quality gets bad, i.e., out-of-synch, is quite aligned.

Open issues and candidate options before e-meeting:

Issue 3-1: SNR test points

- Proposals 1

  - RAN4 does not consider defining new RLM test cases based on R16 MPDCCH performance improvement in SNR points above -10 dB or in fading channel conditions.

- Proposal 2

  - RAN4 to discuss the possibility of adding new RLM test cases based on R16 MPDCCH performance improvement in SNR points below -10 dB and AWGN channel.

- Proposal 3

  - When the network configures the enhanced RLM (rlm-ReportConfig) and improved MPDCCH (mpdcch-crs-connected-config), UE applies the improved MPDCCH transmission parameters for evaluating the out-of-synch when UE reports the Event E1 to the network. This is applicable for both CE Mode A UE and CE Mode B UE.

- Recommended WF

  - In R4-2004016, it is proposed to apply enhanced RLM new when UE reports Event E1. In R4-2003564, it is proposed to apply enhanced RLM when SNR < -10 dB. Since Event E1 is reported typically when SNR < -10 dB.  In summary, it seems both proposals are quite aligned. Try to agree on proposal 3.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 3-1: we cannot agree on proposal 3. Our simulation results show that DMRS+CRS performance improvement is only seen in static channel in low SNR. Low SNR is only one of the conditions. We also have a question for proponent of option 3. Why is MPDCCH performance improvement only considered for enhanced RLM (event E1) and not for normal RLM OOS? Enhanced RLM is an optional UE capability so RAN4 should not apply an optional UE capability (MPDCCH performance improvement) on top of another optional UE capability (enhanced RLM). |
| Ericsson | We support the recommended WF. |
| Huawei, HiSilicon | Just question for clarification on proposal 3 – does it mean the MPDCCH performance improvement is only considered for early OOS indication but not for OOS indication? |
| Ericsson | To answer to Qualcomm, in our view, the gain is there also for the fading channel below -10dB in R4-2003564, but it is lower. For example, for EPA5 and ETU30 cases in set 2 we have observed improvement of 1.25dB and 0.75dB improvement, respectively, although it depends on the definition of improvement. We can also agree to allow enhanced MPDCCH for both enhanced RLM E1 event as well as normal RLM OOS. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004017 | Huawei, HiSilcon: the CR is based on Proposal 3, and we suggest to first clarify the understanding. |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Try to see if companies can agree on following: Based on the discussions, following is the potential agreement. Enhanced MPDCCH is used: - When enhanced RLM E1 event is triggered - When in normal RLM OOS is triggered Candidate options: Recommendations for 2nd round: If the tentative agreement cannot be agreed, more discussions can be expected in the second round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: DL Quality reporting

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2004014 | Ericsson | Proposal 1: RAN4 wait for RAN2 decision for the number of reporting values for 2-bit channel quality report table. Proposal 2: RAN4 use the channel quality mapping table shown in Table 2 if RAN2 decide to assign 2 bits for the channel quality report. (see below) Proposal 3: RAN4 use the channel quality mapping table as shown in Table 3 if RAN2 decide to assign 3 bits for the channel quality report. (see below) |
| R4-2004015 | Ericsson | Draft CR to introduce DL channel quality reporting. |
| R4-2004377 | Huawei, HiSilicon | Draft CR to introduce DL channel quality reporting. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Reporting tables

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1: Reporting table for 2-bit reporting (pending RAN2 decision)

- Proposals 1:

RAN4 use the channel quality mapping table shown in Table 2 if RAN2 decide to assign 2 bits for the channel quality report.

|  | Rmax | 1 or 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value 1 | Rmax / 8 | 1 | 1 | 1 | 2 | 4 | 8 | 16 | 32 |
| Value 2 | Rmax / 4 | 2 | 2 | 2 | 4 | 8 | 16 | 32 | 64 |
| Value 3 | Rmax | 4 | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| Value 4 | 4 x Rmax | 8 | 16 | 32 | 64 | 128 | 256 | 256 | 256 |

- Recommended WF

  - Try to agree on proposal 1.

Issue 3-2: Reporting table for 3-bit reporting (pending RAN2 decision)

- Proposals 1:

RAN4 use the channel quality mapping table as shown in Table 3 if RAN2 decide to assign 3 bits for the channel quality report.

| Configured Rmax | Rmax <= 16 | Rmax >= 32 |
| --- | --- | --- |
| Value 1 | No measurement reporting | No measurement reporting |
| Value 2 | 1 | 4 |
| Value 3 | 2 | 8 |
| Value 4 | 4 | 16 |
| Value 5 | 8 | 32 |
| Value 6 | 16 | 64 |
| Value 7 | 32 | 128 |
| Value 8 | 64 | 256 |

- Recommended WF

  - Try to agree on proposal 1.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: …. Others: |
| Qualcomm | Issue 3-1: we believe no agreement should be made until this issue is concluded in RAN2. We do not support the WF. Issue 3-2: we believe no agreement should be made until this issue is concluded in RAN2. We do not support the WF. |
| Ericsson | Issue 3-1/3-2: Wait for RAN2 decision. |
| Huawei, HiSilicon | Issue 3-1: Reporting table for 2-bit reporting (pending RAN2 decision) Issue 3-2: Reporting table for 3-bit reporting (pending RAN2 decision) Similar comment as Qualcomm and Ericsson. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004015 (Ericsson) | Qualcomm: Typo highlighted below The MPDCCH repetition level for CQI-NPDCCH-Short-NB is chosen with regard to the signalled Also, the following Table should be removed and added only after RAN2 has finalized the issue in order to avoid the risk of making it void. Table 9.1.22.15-2: Downlink channel quality measurement report mapping of CQI-NPDCCH-Short-NB when the DL channel quality reporting is supported [7] |
|  | Company B |
| R4-2004377 (Huawei) | Company A Ericsson: We need to refer to TS36.321. Need channel quality report table and accuracy requirements. In 4.7.X, what is T2? Where is T1? |
|  | Huawei, HiSilicon: This CR has been submitted for several meetings, so it does not cover the accuracy and mapping table which were agreed in last meeting. We could add reference to 36.321. The definition of T2 is included in the CR already: T2 is the period from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting. We have agreed in R4-1912735 that the measurement period in idle mode only includes T2. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: For reporting table for 2-bit and 3-bit, RAN4 shall wait for RAN2 decision. Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX R4-2004015 can be revised. | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #5: Performance Requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2004185 | Ericsson | Proposal 3: RAN4 shall discuss the RF margin to use for defining the accuracy requirements for RSS based RSRP measurement. Options include: - RF margin of 3 dB - Averaged value of the RF margin proposals from the different companies - Rel-8 RF margin of 4 dB |
| R4-2004380 | Huawei, HiSilicon | Proposal 3: RAN4 should take the averaged value of the baseband errors in deriving the accuracy requirements for RSS based measurement. Proposal 4: RAN4 to use RF margin of 4dB for BL UE and 2.5dB for non-BL UE in deriving the accuracy requirements for RSS based measurement. |
| R4-2004381 | Huawei, HiSilicon | Draft CR to introduce accuracy requirements. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Accuracy requirements

Sub-topic description:

It was agreed to use same RF margin that was used for deriving the CRS based RSRP measurement accuracy requirements [R4-2002268] for cat-M. It is noted that RF margin of Rel-8 UEs which was used as reference for cat-M has been tightened since then. Companies have assumed following RF margins in their simulations:

| Company | RF margin |
| --- | --- |
| Ericsson | 3 dB [R4-1914726] |
| Intel | 2.5 dB [R4-1913454] |
| Nokia | 2.0 dB [R4-1915167] |
| Huawei | 4 dB for BL UE and 2.5dB for non-BL UE [R4-2001653] |
| Qualcomm | 3 dB [RAN4#93 meeting minutes] |

Open issues and candidate options before e-meeting:

Issue 5-1: RF margin to use for RSS based measurements

- Proposals 1:

RAN4 shall discuss the RF margin to use for defining the accuracy requirements for RSS based RSRP measurement. Options include:

  - RF margin of 3 dB

  - Averaged value of the RF margin proposals from the different companies

  - Rel-8 RF margin of 4 dB

- Proposals 2: RAN4 should take the averaged value of the baseband errors in deriving the accuracy requirements for RSS based measurement.

- Proposal 3: RAN4 to use RF margin of 4dB for BL UE and 2.5dB for non-BL UE in deriving the accuracy requirements for RSS based measurement.

- Recommended WF

  - Companies are encouraged to provide their view on what RF margin to use.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: …. Others: |
| Qualcomm | We cannot support an RF margin of less than 3 dB for BL UEs. We also cannot support averaging of RF margin used from different companies because some may have assumed a quite optimistic margin. Our view is that one margin should be decided for BL UE and another for non-BL UE and the averaged values of baseband errors should be added with these margins for final accuracy requirements. For the actual RF margin value, we support 3 dB for BL UE (but can also agree on 4 dB). For non-BL UE, we can support 2.5 dB. |
| Ericsson | Issue 5-1: We are also OK to use 3 dB RF margin for the BL UE. We also OK to take the averaged value of the baseband errors and later add the agreed RF margin for final accuracy requirements. For the non-BL UEs, we are OK to use 2.5 dB and follow the same approach. |
| Huawei, HiSilicon | We do not agree to use averaged value of the RF margin as also commented by Qualcomm. We understand RAN4 has well-established assumption for RF margin, and we are not sure why this RSS accuracy should take a different approach. On the exact value, we think Ericsson paper R4-1711455 has explained what was assumed for CRS based accuracy. Basically, we were using the Rel-8 assumption 4dB for BL UE (because it’s low-cost so the enhanced calibration was not assumed), and Rel-12 assumption of 2.5dB for non-BL UE. |
| Ericsson | Actually, the RF margin for low complexity UEs is not well established. The work on tightening of RF margin was done while the MTC work was going on, and we did not spend much time discussing both features. Since most companies have used 3 dB in their own simulation assumptions, I think we should be open to discuss whether the same number can be assumed for deriving accuracy requirements. This will improve the overall performance for both UE and NW, so why not? In fact the 4 dB was reused from rel-8, and now we are in rel-16, and there has been a lot of progress since rel-8. Also, if we agree to use 3 dB RF margin, if companies want, to can use same margin also for CRS based measurement. Otherwise, we can only focus on the RSS. We can open for both. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004381 | Qualcomm: it would be beneficial and clearer if the title or the beginning of the clause mentions that these are accuracy requirements for RSS-based RSRP measurement. We didn’t find any mention of RSS in this CR. |
|  | Ericsson: We see impact on the core requirement due to RSS, e.g. the measurement conditions and measurement delays in IDLE/CONNECTED mode are missing. Moreover, the discussions on the RF margin needs to be agreed before we can work on the CR. So our prefer is to focus on these open issues and then later try to agree on the CR. |
|  | Huawei, HiSilicon: To address the comments: We can clarify the applicability in the beginning of the section. Measurement conditions and measurement delay are core part and will be captured in section 4 and 9. We can update the numbers after the conclusion on RF margin is reached. |
|  | Ericsson In our view, we should first try to agree on the technical issues we need in order to derive the exact accuracy requirements. Once these issues are resolved, it’s easier to work on the CR. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Continue the discussions from 1st round. Candidate options: Recommendations for 2nd round: Continue the discusssions from the 1st round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on RRM agreements for release 16 MTC Comment: intention of this WF is to capture the agreements from all topics above (topic 1-5). | Ericsson |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |