3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	DRAFT R4-2005510

# Electronic Meeting, 20 – 30 Apr., 2020

Source: 	Huawei

Title:	TP to TR 37.941: co-location measurements description

Agenda Item:	6.19.7

Document for:	Approval

# Introduction

In this contribution we provide TP to TR 37.941 for the description of the co-location requirements, for the purpose of the requirements classification.

# References

[1]		RP-193225		Over the air (OTA) base station (BS) testing TR, WID

# TP to the TR 37.941

------------------------------ Modified section ------------------------------

## 6.4	Co-location measurements

### 6.4.1		General

Co-location measurements are required for the OTA co-location requirements which are based on assuming the BS is co-located with another BS of the same base station class. They ensure that both co-located systems can operate with minimal degradation to each other.

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

CLTA is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. Basic principle of the CLTA are outlines in clause 6.4.2. For more details on the co-location test antenna characteristics, refer to TS 37.145-2 [x] clause 4.15.2.2, or TS 38.141-2 [x] clause 4.12.2.2. The CLTA is suitable for testing BS implemented with a planar antenna array.

The alignment between the BS under test and the co-location test antenna is depicted in figure 6.4-1. For more details on the co-location test antenna alignment and test arrangement, refer to TS 37.145-2 [x] clause 4.15.2.3, or TS 38.141-2 [x] clause 4.12.2.3. The same physical alignment applies to in-band and out-of-band co-location requirements.

Co-location requirements are not applicable to FR2.

Figure 6.4.1-1: Alignment of BS and CLTA

### 6.4.2		Co-location test antenna

The co-location test antenna is a practical passive antenna based on the definition of the co-location reference antenna, and it is used for conformance testing of the co-location requirements. Thus the CLTA should be within a certain tolerance of the co-location reference antenna definition, so that commonly available passive BS antennas can be used to test the co-location requirements without having to use a large number of test antennas and without having to design specialized co-location test antennas.

For example, where the co-location reference antenna definition specifies that the co-location reference antenna, should be a single column passive antenna which has the same vertical radiating dimension (h) as the composite antenna of BS, this would be very restrictive when selecting a test antenna. Hence the CLTA should be within a certain tolerance for the vertical radiating dimension (h) such that a suitable practical BS antenna can be found for each BS.

The CLTA is hence defined as follows:

- The CLTA is a single column passive antenna which has a vertical radiating dimension (h) which is equal to the vertical radiating dimension of the BS composite antenna ±30%.

- For in-band requirements, the CLTA supports the same frequency range and polarizations as the BS composite antenna.

- For out-of-band co-location requirements, the half power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ±3°.

- The polarization should match the in-band CLTA polarization

- The CLTA has a horizontal half-power beamwidth of 65° ±10°.

- The front faces of the BS and the CLTA need to be aligned in a common plane perpendicular to the mechanical bore-sight direction.

- The centre of the vertical radiating regions of the CLTA and the BS composite antenna need to be aligned.

### 6.4.3		Standard test antenna

Co-location requirements may also be tested with a standardised test antenna such as a dipole. This has a number of advantages:

-	Standard test antennas are easily available and easier to specify.

-	Repeatability between different conformance testing runs will be greater.

-	Using standard test antennas, fewer antennas will be required for conformance testing.

-	A standard test antenna can have higher gain in the direction of the BS than a CLTA, so that signal levels can be higher relative to the noise floor in emission measurements and lower test levels can be used in interference measurements.

In order to test with a standard test antenna, the translation between measured power levels in the standard test antenna and the co-location reference antenna must be shown. The method for translating the power levels in the co-location reference antenna to the standard test antenna and vice versa is not covered by the present release of this technical report.

----------------------------- End of modified section ------------------------------