3GPP TSG-RAN WG4 Meeting #94bis-e			  											R4-2005072

Electronic Meeting, 20th Apr. 2020 – 24th Apr. 2020

Source: 	Nokia, Optus

Title: 		TP for 37.716-21-21 to introduce DC_28_n40-n78

Agenda Item: 			8.7.2 [DC_R16_xBLTE_2BNR_yDL2UL-Core]

Document for:	Agreement

# 1	Introduction

This contribution is a TP for TR 37.716-21-21 to introduce DC_28_n40-n78.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 6.x	DC_28_n40-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Co-existence studies

Co-existence studies are captured in TR 38.716-02-00 and TR 37.716-11-11. There are 3rd & 4th IMDs problems from dual uplink DC_28A_n40A which could impact own Rx band n78. Also 3rd IMD problem from dual uplink DC_28A_n78A could impact own Rx band n40. But CA_n40-n78 only allow one uplink transmission, then there are no self-interference problems

### 6.x.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 6.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n40-n78 | 28 | 0.5 |
|  | n40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 6.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n40-n78 | 28 | 0.2 |
|  | n40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 6.x.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD requirements since CA_n40-n78 only allow one uplink transmission.

*********************** End of the TP ***************************************************

# 3	References