TSG-RAN Working Group 4 (Radio) meeting #94bis-E	R4-2005493

Electronic Meeting, April 20th – 30th 2020

Source:	Ericsson

Title:	WF on IAB-MT In-band selectivity

Agenda item:	6.5.2.2.3

Document for:	Approval

# Introduction

At RAN4#94bis-E meeting a way-forward for IAB-MT In-Band Blocking was created based on the discussion [1].

# Way-Forward

Way-forward for IAB In-Band Blocking requirement;

- FR2 IAB-MT ACS wanted signal power:

  - For WA IAB-MT use REFSENS + 6dB as wanted signal power.

  - For LA IAB-MT consider;

    - REFSENS + 14dB as the wanted power

    - REFSENS +6 dB as wanted power

- FR2 IAB-MT In-Band Blocking wanted signal and interfering signal:

  - Reuse BS In-Band Blocking wanted signal and interference level for WA IAB-MT.

  - Reuse UE In-Band blocking wanted signal and interference level for LA IAB-MT

- FR2 IAB-MT In-Band Blocking interfering signal type:

CP-OFDM in-band interferer shall be used for IAB-MT.

# References

[1]	R4-2005439, “Email discussion summary for [94e Bis][209] NR_IAB_RF_Part_3”, Samsung