3GPP TSG-RAN WG4 Meeting # 95-e	R4-2006111

Electronic Meeting, 25 May – 5 June 2020

Agenda Item:	11.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	AAS BS antenna characteristics for frequency ranges below 6 GHz

Document for:	Approval

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in different frequency ranges for IMT [2]. One set of parameters requested by ITU-R WP5D are the AAS BS beamforming antenna characteristics in these frequency ranges. This topic was discussed in RAN4#94-e-Bis, and two WF related to the BS antenna parameters was agreed in [3], where some parameters were FFS.

This contribution provides proposals to decide on the FFS AAS BS antenna parameters in [3] for frequency ranges below 6 GHz. Note that it was agreed in [3] that AAS BS beamforming antenna characteristics are only considered above 1710MHz.

# 2.	Discussion

In the following, we provide proposals on the FFS AAS BS antenna parameters in the agreed WF in [3].

- Element parameters

  - Horizontal/vertical element spacing: 0.5/0.7 lambda

  - Horizontal/vertical 3 dB beamwidth: 80/65 degrees

  - Element gain (GE,max): 8dB (plus 3dB dual polarization gain)

- Antenna configuration:

  - 8x8 (each with dual polarization)

- Additional parameters

  - Horizontal coverage range: +/- 60 degrees

  - Vertical coverage range: 90-108 degrees

  - BS output power, TRP: 43 dBm/100 MHz

  - Mechanical down-tilt angle: 9 degrees

The corresponding horizontal and vertical beamforming antenna pattern at the maximum steering angles of 60 degrees horizontal and 108 degrees vertical are shown in Figures 1 and 2 below. It can be seen from the figures that there is no severe side-lobe even at the maximum steering angles.

Figure 1: Horizontal beamforming antenna pattern (at 60 degrees with 0.5 lambda)

Figure 2: Vertical beamforming antenna pattern (at 108 degrees with 0.7 lambda)

On the other hand, if the horizontal/vertical element spacing is changed to 0.6/0.9 lambda, the corresponding horizontal and vertical beamforming antenna pattern at the maximum steering angles of 60 degrees horizontal and 108 degrees vertical will be those shown in Figures 3 and 4 below. It can be seen from the figures that there are severe side-lobes at the opposite side of the maximum steering angles. The side-lobe in the horizontal plane would significantly degrade the UE throughput in that horizontal direction, while the side-lobe in the vertical plane would significantly degrade the service performance (like satellite) in that vertical direction. Therefore, it is proposed to adopt the horizontal/vertical element spacing of 0.5/0.7 lambda as AAS BS antenna parameters for frequency ranges below 6GHz (and above 1710MHz).

Figure 3: Horizontal beamforming antenna pattern (at 60 degrees with 0.6 lambda)

Figure 4: Vertical beamforming antenna pattern (at 108 degrees with 0.9 lambda)

# 3.	Conclusion

This contribution has provided proposals to decide on the FFS BS antenna parameters in [3] for frequency ranges below 6GHz (and above 1710MHz).

Proposals:

- Element parameters

  - Horizontal/vertical element spacing: 0.5/0.7 lambda

  - Horizontal/vertical 3 dB beamwidth: 80/65 degrees

  - Element gain (GE,max): 8dB (plus 3dB dual polarization gain)

- Antenna configuration:

  - 8x8 (each with dual polarization)

- Additional parameters

  - Horizontal coverage range: +/- 60 degrees

  - Vertical coverage range: 90-108 degrees

  - BS output power, TRP: 43 dBm/100 MHz

  - Mechanical down-tilt angle: 9 degrees

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	R4-2005170, “WF on LS Reply to ITU-R WP5D for the frequency ranges below 6 GHz”, Ericsson.