3GPP TSG-RAN4 Meeting #95-e	R4-2006185

# Online, 25th May - 5th Jun 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0619 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSSF correction for R15 TS38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CSSF outside MG in current spec does not consider the following cases: 1. In EN-DC with FR1 +FR2 CA (FR1 PSCell) only one SCell is configured only one NR Scell is configured 2. in FR1 +FR2 CA (FR1 PCell) only one SCell is configured only one NR Scell is configured 3. in FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) no SCell is configured 4. in NE-DC with FR1 +FR2 CA (FR1 PCell) only one SCell is configured only one NR Scell is configured |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the note in the CSSF table for the above cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CSSF outside MG in current spec is missing the above cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, 9.1.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

##### 9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.1-1.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCells |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2Note 3 | 2×(Number of configured SCell(s)-1) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | Number of configured SCell(s) | 1 | N/A | Number of configured SCell(s) |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and neighbour cell measurement is required on this Scell. |  |  |  |  |  |

##### 9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCell(s) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 Note 3 | 2×(Number of configured SCell(s)-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and neighbour cell measurement is required on this Scell. |  |  |  |  |  |

##### 9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with NR-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.3-1.

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1 | 1 | 2×(Number of configured SCell(s)) | 2 Note 2 | 2×(Number of configured SCell(s)) |
| Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. Note 2: CSSFoutside_gap,i =1 if no SCell is configured. |  |  |  |  |

##### 9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with NE-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.4-1.

Table 9.1.5.1.4-1: CSSFoutside_gap,i scaling factor for NE-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| NE-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| NE-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCell(s) |
| NE-DC with FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 Note 3 | 2×(Number of configured SCell(s)-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and neighbour cell measurement is required on this Scell. |  |  |  |  |  |

End of Change 3