3GPP TSG-RAN4 Meeting #95-e		      R4-2006225

Electronic meeting, 25th May – 5th June, 2020

Title:	Further discussion on CSI-RS based UE measurement capabilities

Source:	CATT

Agenda Item:	6.16.1.3

Document for:	Discussion

- Introduction

In RAN4#94-ebis meeting, CSI-RS based UE measurement capability has been discussed, and the potential options to define the UE measurement capability was summarized in approved WF [1]. In this contribution, we further discuss CSI-RS based UE measurement capabilities, and give our proposal.

- Discussion

CSI-RS resources can be configured by network for mobility purpose, and UE shall be able to perform CSI-RSRP/CSI-RSRQ/CSI-SINR measurements on configured CSI-RS resource(s). According to TS38.214 [2], if CSI-RS resources are configured for mobility purpose, the maximum number of CSI-RS resource for each UE is set to be 64 per frequency layer when associated SSB is not configured. And UE can be configured up to 96 CSI-RS resources if associated SSB is configured. And according to TS38.331, for each CSI-RS resource, the number of PRB can be configured as {24, 48, 96, 192, 264} in frequency domain, the density of CSI-RS can be configured as {0.5, 1, 3} RE(s) per PRB. In order to guarantee the mobility performance, the CSI-RS based UE measurement shall be discussed and specified.

In TS38.133 [3], SSB based UE measurement capabilities are specified on number of carrier to be monitored, the number of cell and beam to be monitored per frequency layer. We think the similar approached can be used to defined CSI-RS based UE measurement capability. Thus, the CSI-RS based UE measurement capabilities shall specified in terms of:

- Number of carrier to be monitored if CSI-RS resources are configured

- Number of cell to be monitored per frequency layer

- Number of CSI-RS resources to be monitored per frequency

Proposal 1: CSI-RS based UE measurement capabilities shall specified in terms of:

- Number of carrier to be monitored

- Number of cell to be monitored per frequency layer

- Number of CSI-RS resources to be monitored per frequency layer

For number of CSI-RS based carriers to be monitored if CSI-RS resources are configured, as we support to only consider associated SSB case in Rel-16, thus, when the CSI-RS resources are associated with SSB, UE is required to perform SSB detection to get the timing of the target cell, then UE can measure the configured CSI-RS resources. So the measurement capability on the frequency layer which is associated SSB can be also considered as the measurement capability on SSB frequency layer. However, UE may need monitor more carriers due to CSI-RS measurement, hence, we propose UE is required to monitor one more frequency layer due to CSI-RS measurement.

Regarding the total number of SSB and CSI-RS frequency layers to be measured, by considering the trade-off between mobility performance and UE power consumption, it is propose to add 1 more frequency layer compared with the existing requirements.

Proposal 2: UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

Proposal 3: UE shall be able to measure at least 14 carriers of all RATs in total.

Regarding the number of cell to be monitored per frequency layer, it would be straightforward to reuse the same requirements defined for SSB based measurement, we think in many cases, the effort of SSB based intra-/inter-frequency measurement and CSI-RS based intra-/inter-frequency measurement are similar. Thus, we propose that the requirement of number of cell to be monitored defined for SSB based UE measurement capability can be reused to define the number of cells to be monitored for CSI-RS based UE measurement capabilities.

Proposal 4: The requirement of number of cell to be monitored defined for SSB based UE measurement capability can be reused to define the number of cells to be monitored for CSI-RS based UE measurement capabilities.

For the number of CSI-RS resource, according to TS38.214, the maximum number of CSI-RS resource for each UE is set to be 64 per frequency layer when associated SSB is not configured. And UE can be configured up to 96 CSI-RS resources if associated SSB is configured. We think UE shall monitor at least 24 CSI-RS resources per frequency layer considering mobility performance and UE’s complexity.

Proposal 5: For the number of CSI-RS resource, UE shall monitor at least 24 CSI-RS resources per frequency layer.

- Conclusion

In this contribution, we further discussed the CSI-RS based UE measurement capabilities, and give our proposal as follows.

Proposal 1: CSI-RS based UE measurement capabilities shall specified in terms of:

- Number of carrier to be monitored

- Number of cell to be monitored per frequency layer

- Number of CSI-RS resources to be monitored per frequency layer

Proposal 2: UE shall be able to measure at least 8 NR frequency layers in total, including SSB frequency layers and CSI-RS frequency layers.

Proposal 3: UE shall be able to measure at least 14 carriers of all RATs in total.

Proposal 4: The requirement of number of cell to be monitored defined for SSB based UE measurement capability can be reused to define the number of cells to be monitored for CSI-RS based UE measurement capabilities.

Proposal 5: For the number of CSI-RS resource, UE shall monitor at least 24 CSI-RS resources per frequency layer.

- Reference

[1]	R4-2005357	WF on CSI-RS based L3 measurement capability and requirements, OPPO

[2]	TS38.214

[3]	TS38.133