3GPP RAN WG4 Meeting #95-e	R4-2006239

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0643 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UL RTOA measurement report mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UL RTOA measurement report mapping is missing in 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | UL RTOA measurement report mapping is introduced in 38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL RTOA measurement report mapping is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New chapter and sections: 11, 11.1, 11.1.1, 11.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 11 Measurement Performance Requirements for NR

### 11.1 UL RTOA measurement

11.1.1 Measurement accuracy requirements

11.1.2 Measurement report mapping

The reporting range of UL RTOA is defined from - 985024Tc to 985024Tc with resolution Tc*2^k, where k is a configured value from set {0,1,2,3,4,5}.

The mapping of measured quantity is defined in Table 11.1.2-1 to Table 11.1.2-6 respectively. The range in the signalling may be larger than the guaranteed accuracy range.

Table 11.1.2-1: UL RTOA report mapping, k=0

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RTOA_000000 | -985024 > RTOA | Tc |
| RTOA_000001 | -985024 RTOA < -985023 | Tc |
| RTOA_000002 | -985023 RTOA < -985022 | Tc |
|  |  | … |
| RTOA_985022 | -3 RTOA -2 | Tc |
| RTOA_985023 | -2 RTOA -1 | Tc |
| RTOA_985024 | -1 RTOA 0 | Tc |
| RTOA_985025 | 0 < RTOA 1 | Tc |
| RTOA_985026 | 1 < RTOA 2 | Tc |
| RTOA_985027 | 2 < RTOA 3 | Tc |
| … | … | … |
| RTOA_1970047 | 985022 < RTOA 985023 | Tc |
| RTOA_1970048 | 985023 < RTOA 985024 | Tc |
| RTOA_1970049 | 985024 < RTOA | Tc |

Table 11.1.2-2: UL RTOA report mapping, k=1

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RTOA_000000 | -985024 > RTOA | Tc |
| RTOA_000001 | -985024 RTOA < -985022 | Tc |
| RTOA_000002 | -985022 RTOA < -985020 | Tc |
|  |  | … |
| RTOA_492510 | -6 RTOA -4 | Tc |
| RTOA_492511 | -4 RTOA -2 | Tc |
| RTOA_492512 | -2 RTOA 0 | Tc |
| RTOA_492513 | 0 < RTOA 2 | Tc |
| RTOA_492514 | 2 < RTOA 4 | Tc |
| RTOA_492515 | 4 < RTOA 6 | Tc |
| … | … | … |
| RTOA_985023 | 985020 < RTOA 985022 | Tc |
| RTOA_985024 | 985022 < RTOA 985024 | Tc |
| RTOA_985025 | 985024 < RTOA | Tc |

Table 11.1.2-3: UL RTOA report mapping, k=2

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RTOA_000000 | -985024 > RTOA | Tc |
| RTOA_000001 | -985024 RTOA < -985020 | Tc |
| RTOA_000002 | -985020 RTOA < -985016 | Tc |
|  |  | … |
| RTOA_246254 | -12 RTOA -8 | Tc |
| RTOA_246255 | -8 RTOA -4 | Tc |
| RTOA_246256 | -4 RTOA 0 | Tc |
| RTOA_246257 | 0 < RTOA 4 | Tc |
| RTOA_246258 | 4 < RTOA 8 | Tc |
| RTOA_246259 | 8 < RTOA 12 | Tc |
| … | … | … |
| RTOA_492511 | 985016 < RTOA 985020 | Tc |
| RTOA_492512 | 985020 < RTOA 985024 | Tc |
| RTOA_492513 | 985024 < RTOA | Tc |

Table 11.1.2-4: UL RTOA report mapping, k=3

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RTOA_000000 | -985024 > RTOA | Tc |
| RTOA_000001 | -985024 RTOA < -985016 | Tc |
| RTOA_000002 | -985016 RTOA < -985008 | Tc |
|  |  | … |
| RTOA_123126 | -24 RTOA -16 | Tc |
| RTOA_123127 | -16 RTOA -8 | Tc |
| RTOA_123128 | -8 RTOA 0 | Tc |
| RTOA_123129 | 0 < RTOA 8 | Tc |
| RTOA_123130 | 8 < RTOA 16 | Tc |
| RTOA_123131 | 16 < RTOA 24 | Tc |
| … | … | … |
| RTOA_246255 | 985008 < RTOA 985016 | Tc |
| RTOA_246256 | 985016 < RTOA 985024 | Tc |
| RTOA_246257 | 985024 < RTOA | Tc |

Table 11.1.2-5: UL RTOA report mapping, k=4

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RTOA_000000 | -985024 > RTOA | Tc |
| RTOA_000001 | -985024 RTOA < -985008 | Tc |
| RTOA_000002 | -985008 RTOA < -984992 | Tc |
|  |  | … |
| RTOA_61562 | -48 RTOA -32 | Tc |
| RTOA_61563 | -32 RTOA -16 | Tc |
| RTOA_61564 | -16 RTOA 0 | Tc |
| RTOA_61565 | 0 < RTOA 16 | Tc |
| RTOA_61566 | 16 < RTOA 32 | Tc |
| RTOA_61567 | 32 < RTOA 48 | Tc |
| … | … | … |
| RTOA_123127 | 984992< RTOA 985008 | Tc |
| RTOA_123128 | 985008< RTOA 985024 | Tc |
| RTOA_123129 | 985024 < RTOA | Tc |

Table 11.1.2-6: UL RTOA report mapping, k=5

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RTOA_00000 | -985024 > RTOA | Tc |
| RTOA_00001 | -985024 RTOA < -984992 | Tc |
| RTOA_00002 | -984992 RTOA < -984960 | Tc |
|  |  | … |
| RTOA_30780 | -80 RTOA -64 | Tc |
| RTOA_30781 | -64 RTOA -32 | Tc |
| RTOA_30782 | -32 RTOA 0 | Tc |
| RTOA_30783 | 0 < RTOA 32 | Tc |
| RTOA_30784 | 32 < RTOA 64 | Tc |
| RTOA_30785 | 64 < RTOA 80 | Tc |
| … | … | … |
| RTOA_61563 | 984960< RTOA 984992 | Tc |
| RTOA_61564 | 984992< RTOA 985024 | Tc |
| RTOA_61565 | 985024 < RTOA | Tc |