3GPP TSG-RAN WG4 Meeting # 95-e                                                            R4-2006257

Electronic Meeting, 25 May – 5 June, 2020

Source: 	CATT

Title: 	Simulation results for NR PUSCH UL timing adjustment demodulation requirement

Agenda item:	6.17.2.2.3

Document for:	Discussion

- Introduction

In the recent RAN4#94-e-bis meeting, adding the performance requirement of UL timing adjustment for scenario Z(500kph) in WF [1] was approved. This paper will provide our evaluation results for NR PUSCH UL timing for scenario Z (500kph) for simulation alignment.

- Evaluation results

As WF[1] on NR PUSCH UL TA, the 500kp UL TA scenraio Z was agreed to be introduced and shown as follows:

- New scenarios - Specify the 500kph UL TA scenario “Z”, with the following parameters: Channel model: Stationary UE: AWGN, Moving UE: AWGN. UE speed: 500 km/h. A: 15 kHz: 10 us, 30 kHz: 5 us, delta_omega: 15 kHz: 0.18 s-1, 30 kHz: 0.36 s-1.

So, we propose,

Proposal 1: add 500kph UL TA scenraio Z to simulation results summary.

The agreed test parameters[1, 2, 3] are summarized and shown in Table A-1 in Annex.

The evaluation results for  NR PUSCH UL timing adjustment are shown in Table 2-1.

Table 2-1 Evaluation results for NR PUSCH UL timing adjustment for scenario Z

| Scenario | CBW | SCS | DMRS | l0 | MCS | PUSCH mapping type | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Ideal | Impairment |
| Scenario Z | 10MHz | 15KHz | 1+1+1 | 2 | 16 | Type A | 5.81 | 8.31 |
|  | 10MHz | 15KHz | 1+1+1 | 0 | 16 | Type B | 5.78 | 8.28 |
|  | 40MHz | 30KHz | 1+1+1 | 2 | 16 | Type A | 5.86 | 8.36 |
|  | 40MHz | 30KHz | 1+1+1 | 0 | 16 | Type B | 5.84 | 8.34 |

- Conclusion

This paper provides our evaluation results for NR PUSCH UL timing for scenario Z for simulation alignment, and also gives our proposal.

Proposal 1: add 500kph UL TA scenraio Z to simulation results summary.

- References

- R4-2005536, WF on Rel-16 NR HSTUL TA BS demodulation requirements, ZTE, RAN4#94-ebis

- R4-1915886，Way forward on NR HST PUSCH demodulation requirements，Huawei, HiSilicon，RAN4#93

- R4-2002405，WF on Rel-16 NR HST BS demodulation requirements，Nokia, Nokia Shanghai Bell, RAN4#94-e

- Annex

Table A-1 Test parameters for NR PUSCH UL timing adjustment

| Parameter | value |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and type B |
| MCS index | MCS 16 |
| PTRS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV | {0,2,3,1} |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 |
| Front loaded DMRS start symbol | Type A: l0 = 2 Type B: l0 = 0 |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz |
| RB assignment | 15kHz SCS/10MHz CBW: 25 continuously allocated RBs for each UE 30kHz SCS/40MHz CBW: 50 continuously allocated RBs for each UE |
| Starting PRB index | 15kHz SCS/10MHz CBW: Moving UE 0, Stationary UE 25 30kHz SCS/40MHz CBW: Moving UE 0, Stationary UE 50 |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario Y: AWGN 15kHz SCS: A=10us, △ω=0.13s-1 30kHz SCS: A=5us, △ω=0.26s-1 Scenario Z: AWGN 15kHz SCS: A=10us, △ω=0.18s-1 30kHz SCS: A=5us, △ω=0.36s-1 |
| SRS transmission(optional) | Disabled |
| Testing metric | SNR @70% of maximum throughput |