3GPP TSG-RAN WG4 Meeting #95-e                                                             R4-2006258

# Electronic Meeting, 25 May – 5 June, 2020

Source: 	CATT

Title: 	Simulation results for NR HST PUSCH demodulation requirement

Agenda item:	6.17.2.2.1

Document for:	Discussion

- Introduction

In the recent RAN4#94-ebis meeting, adding NR HST PUSCH requirements for 5MHz CBW/15kHz SCS, 10MHz CBW/30kHz SCS in WF[1] was approved. This paper will provide our evaluation results for NR HST PUSCH for 5MHz CBW/15kHz SCS, 10MHz CBW/30kHz SCS for simulation alignment.

- Evaluation results

As WF[3] on NR HST PUSCH, the NR HST PUSCH requirements for 5MHz CBW/15kHz SCS, 10MHz CBW/30kHz SCS were agreed to be introduced and shown as follows:

Agreement 2nd round: Keep previous agreement. Add requirements for 5MHz CBW/15kHz SCS, 10Mhz CBW/30kHz SCS for CP-OFDM with applicability rule as for Rel-15 (i.e., only largest CBW per SCS needs to be tested).

So, we propose,

Proposal 1: add 5MHz CBW/15kHz SCS, 10MHz CBW/30kHz SCS for PUSCH to simulation results summary.

The agreed simuation asumptions in WF[1] are shown in Table A-1 in Annex.

The evaluation results for 350km/h and 500km/h are shown in Table 2-1 and Table 2-2 respectively.

Table 2-1 Evaluation results for 350km/h for minimum CBW

| Channel model | Propagation condition | Carrier frequency (GHz) | SCS | CBW | Maximum Doppler shift | Tx/Rx | MCS | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal | Impairment |
| AWGN | Tunnel | 2.1 | 15KHz | 5MHz | 1340Hz | 1T1R | 2 | -3.45 | -0.95 |
|  |  |  |  |  |  |  | 16[FFS] | 8.73 | 11.23 |
|  |  |  |  |  |  | 1T2R | 2 | -6.19 | -3.69 |
|  |  |  |  |  |  |  | 16 | 5.79 | 8.29 |
|  |  | 3.6 | 30KHz | 10MHz | 2334Hz | 1T1R | 2 | -3.47 | -0.97 |
|  |  |  |  |  |  |  | 16[FFS] | 8.51 | 11.01 |
|  |  |  |  |  |  | 1T2R | 2 | -6.23 | -3.73 |
|  |  |  |  |  |  |  | 16 | 5.56 | 8.06 |
|  | Open space | 2.1 | 15KHz | 5MHz | 1340Hz | 1T2R | 2 | -6.2 | -3.7 |
|  |  |  |  |  |  |  | 16 | 5.77 | 8.27 |
|  |  |  |  |  |  | 1T8R | 2 | -11.27 | -8.77 |
|  |  |  |  |  |  |  | 16 | -0.13 | 2.37 |
|  |  | 3.6 | 30KHz | 10MHz | 2334Hz | 1T2R | 2 | -6.23 | -3.73 |
|  |  |  |  |  |  |  | 16 | 5.55 | 8.05 |
|  |  |  |  |  |  | 1T8R | 2 | -11.29 | -8.79 |
|  |  |  |  |  |  |  | 16 | -0.34 | 2.16 |

Table 2-2 Evaluation results for 500km/h for minimum CBW

| Channel model | Propagation condition | Carrier frequency (GHz) | SCS | CBW | Maximum Doppler shift | Tx/Rx | MCS | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal | Impairment |
| AWGN | Tunnel | 1.8 | 15KHz | 5MHz | 1740Hz | 1T1R | 2 | -3.45 | -0.95 |
|  |  |  |  |  |  |  | 16[FFS] | 8.76 | 11.26 |
|  |  |  |  |  |  | 1T2R | 2 | -6.2 | -3.7 |
|  |  |  |  |  |  |  | 16 | 5.82 | 8.32 |
|  |  | 3.6 | 30KHz | 10MHz | 3334Hz | 1T1R | 2 | -3.47 | -0.97 |
|  |  |  |  |  |  |  | 16[FFS] | 8.54 | 11.04 |
|  |  |  |  |  |  | 1T2R | 2 | -6.22 | -3.72 |
|  |  |  |  |  |  |  | 16 | 5.6 | 8.1 |
|  | Open space | 1.8 | 15KHz | 5MHz | 1740Hz | 1T2R | 2 | -6.2 | -3.7 |
|  |  |  |  |  |  |  | 16 | 5.78 | 8.28 |
|  |  |  |  |  |  | 1T8R | 2 | -11.26 | -8.76 |
|  |  |  |  |  |  |  | 16 | -0.12 | 2.38 |
|  |  | 3.6 | 30KHz | 10MHz | 3334Hz | 1T2R | 2 | -6.22 | -3.72 |
|  |  |  |  |  |  |  | 16 | 5.57 | 8.07 |
|  |  |  |  |  |  | 1T8R | 2 | -11.27 | -8.77 |
|  |  |  |  |  |  |  | 16 | -0.33 | 2.17 |

- Conclusion

This paper provides our evaluation results for NR HST PUSCH for simulation alignment and gives our proposal.

Proposal 1: add 5MHz CBW/15kHz SCS, 10MHz CBW/30kHz SCS for PUSCH to simulation results summary.

- References

- R4-2005534,WF on Rel-16 NR HST PUSCH BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4##94-eBis

- Annex

Table A-1 Simulation assumptions for NR HST PUSCH with CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | v=350km/h | V=500km/h |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | Tunnel: 2, FFS on 1 Open space: 2, 8 | Tunnel: 2, FFS on 1 Open space: 2, 8 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| MCS index | Tunnel: MCS 2 and MCS 16 Open space: MCS 2 and MCS 16 MCS 2 and MCS [16] for 1T1R for tunnel | Tunnel: MCS 2, MCS 16 Open space: MCS 2 and MCS 16 MCS 2 and MCS [16] for 1T1R for tunnel |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| RV | {0,2,3,1} | {0,2,3,1} |
| DMRS type | type 1 | Type1 |
| Number of DMRS symbols | 1+1+1 | 1+1+1 |
| Front loaded DMRS start symbol | l0 = 2 or 3 | l0 = 2 or 3 |
| SCS and BW | 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz |  |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth |
| Carrier frequency (GHz) | For 15kHz SCS: 2.1GHz For 30kHz SCS: 3.6GHz | For 15kHz SCS: 1.8GHz For 30kHz SCS: 3.6GHz |
| fd | For 15kHz SCS: 1340Hz For 30kHz SCS: 2334Hz | For 15kHz SCS: 1740Hz For 30kHz SCS: 3334Hz |
| Propagation condition | Tunnel: Ds=300m, Dmin=2m; Open space: Ds=700m, Dmin=150m |  |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |