3GPP TSG-RAN WG4 Meeting #95-e                                                  R4-2006316

Electrical Meeting, 25th May-5th June, 2020

Agenda item:	6.11.3.3

Source:	Samsung

Title:		        Simulation results for Multi-TRP/Panel transmission

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, the WF was approved regarding to the demodulation requirement based on multi-TRP/Panel transmission for NR eMIMO [1]. The agreements related to Multi-TRP (eMBB basis) requirements were reached in the WF as listed below:

- Assumption for UE receiver implementation - The test case design should be ensure receiver implementation agnostic with assumption of single FFT operation - Timing offset configuration - Study suitable upper bounds on propagation delay difference. The following values can be used from LTE Rel-11 CoMP (-0.5us and 2us) as a starting point - FR1 FDD (15KHz SCS) - Positive upper bound - Option 1: [2] us - Other options are not precluded - FFS on Negative upper bound - FR1 TDD (30KHz SCS) - Positive upper bound - Option 1: [1] us - Option 2: [2] us - Other options are not precluded - FFS on Negative upper bound - FFS on FR2 TDD (120 KHz) - Positive upper bound - Option 1: [0.25] us - Other options are not precluded - FFS on Negative upper bound - Frequency offset configuration - FR1 - FDD (15KHz SCS) : - Option 1: 200 Hz - Option 2: 300Hz - Other options are not precluded - TDD (30KHz SCS): - Option 1 :300 Hz - Option 2: 600 Hz - Other options are not precluded - FFS on FR2 - TDD (120KHz SCS): (0~600 Hz) - Other options are not precluded

In this contribution, the initial simulation results are provided to verify proper time offset and frequency offset values.

# 2	Simulation results

We provide initial simulation results of Multi-TRP tests with two TRPs under Non-overlapping and Full-overlapping scenario respectively.  Besides, we also take the Timing offset and Frequency offset into consideration to observe the related performance influence.

- FDD

Observation 1:  The performance gap between Non-Overlap and Full-Overlap is 3dB based on same information bit payload configuration.

Fig2-1: SNR--Throughput of Full-overlapping with Frequency offset=200Hz

Fig2-2: SNR--Throughput of Full-overlapping with Timing offset=2us

Fig2-3: SNR--Throughput of Full-overlapping with Timing offset=-0.5us

Table2-1: SNR point at 70% of peak throughput FDD

| nTx ports | 2Tx*2TRP Full-overlapping |  |  |
| --- | --- | --- | --- |
| Scenario | Frequency offset=200Hz | Timing offset=2us | Timing offset=-0.5us |
| SNR point for PDSCH without TRS | 10.0dB | 8.33dB | 7.65dB |
| SNR point for PDSCH with TRS | 7.48dB | 7.45dB | 7.65dB |

As indicated in the figures, the performance has some degradation with impact of timing offset and frequency offset. With TRS configured, the performance can be close with ideal case on scenario of plus Timing offset and Frequency offset.

Observation 2:  With 200Hz frequency offset, there is 2.5 dB performance gap to distinguish different UE behavior with MCS13 (16QAM).

Observation 3: With 2us time offset, the performance gap is <1 dB under MCS 13 (16QAM)

Observation 4: With -0.5 us frequency offset, no much performance difference for MCS13 (16QAM).

In order to ensure proper UE behavior, higher MCS i.e. 64QAM need to be considered for further evaluation.

- TDD

Fig2-4: SNR--Throughput of Full-overlapping with Timing offset=-0.25us

Fig2-5: SNR--Throughput of Full-overlapping with Timing offset=1us

Fig2-6: SNR--Throughput of Full-overlapping with Frequency offset=300Hz

Table2-1: SNR point at 70% of peak throughput TDD

| nTx ports | 2Tx*2TRP Full-overlapping |  |  |
| --- | --- | --- | --- |
| Scenario | Frequency offset=300Hz | TA offset=2us | TA offset=-0.5us |
| SNR point for PDSCH without TRS |  |  |  |
| SNR point for PDSCH with TRS |  |  |  |

# 3	Conclusion

In this contribution, the initial simulation results are provided with Multi-TRP of eMBB under non-overlapping and full-overlapping with Timing offset and Frequency offset

Observation 1:  The performance gap between Non-Overlap and Full-Overlap is 3dB based on same information bit payload configuration.

Observation2:  With 200Hz frequency offset, there is 2.5 dB performance gap to distinguish different UE behavior with MCS13 (16QAM).

Observation 3: With 2us time offset, the performance gap is <1 dB under MCS 13 (16QAM)

Observation 4: With -0.5 us frequency offset, no much performance difference for MCS13 (16QAM).

In order to ensure proper UE behavior, higher MCS i.e. 64QAM need to be considered for further evaluation

# References

R4-2005529 “draft WF on PDSCH demodulation requirement and General aspects for NR eMIMO_v2” Samsung. 3GPP TSG-RAN WG4 Meeting #94e-b

# Appendix

Simulation assumption for FDD:

| Parameter |  | Unit | TRP1 | TRP2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLC300-100 |  |
| Antenna configuration |  |  | LOW 2x2 | LOW 2x2 |
| Cell ID |  |  | 0 | 2 |
| DMRS configuration | CDM group index |  | 0 | 1 |
|  | Additional number |  | 1 |  |
| CSI-RS configuration | ZP CSI-RS |  | Not configured |  |
|  | NZP CSIRS for measurement |  | Not configured |  |
|  | CSI IM |  | Not configured |  |
| TRS configuration | Resource number |  | 4 |  |
|  | Time domain location |  | l0=6;l1=10 |  |
|  | Frequency domain location |  | k0=0; k1=4; k2=8 | k0=2; k1=6; k2=10 |
|  | Time Config interval and offset | slot | 20/10 |  |
| Timing Offset |  | us | 0 | 2/-0.5 |
| Frequency Offset |  | Hz | 0 | 200 |
| CSI Feedback |  |  | Not configured |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | MCS 13 FDD (see FRC) |  |

Simulation assumption for TDD (different value from FDD):

| Parameter | Unit | TRP1 | TRP2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 40 |  |
| Subcarrier spacing | kHz | 30 |  |
| Time Config interval and offset | slot | 40/20 |  |
| Maximum number of HARQ transmission |  | 4 |  |
| Measurement channel |  | MCS 13 TDD (see FRC) |  |

PDSCH Reference Channel for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 FDD |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing | kHz | 15 |
| Number of allocated resource blocks | PRBs | 52 |
| Number of consecutive PDSCH symbols |  | 12 |
| Allocated slots per 2 frames | Slots | 19 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Number of DMRS rEs |  | 24 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slots i = 1,…, 19 | Bits | 12040 |
| Transport block CRC per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slots i = 1,…, 19 | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slot i = 0 | CBs | N/A |
| For Slots i = 1,…, 19 | CBs | 2 |
| Binary Channel Bits Per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slots i = 10, 11 | Bits | 23712 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 24960 |
| Max. Throughput averaged over 2 frames | Mbps | 11.438 |

PDSCH Reference Channel for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.PDSCH.2-2.1 TDD |
| Channel bandwidth | MHz | 40 |
| Subcarrier spacing | kHz | 30 |
| Allocated resource blocks | PRBs | 106 |
| Number of consecutive PDSCH symbols |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6})for i from {1,…,39} |  | 12 |
| Allocated slots per 2 frames |  | 31 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Number of DMRS rEs |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 12 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 24 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 7296 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24576 |
| Transport block CRC per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 4 |
| Binary Channel Bits Per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |
| For Slots i = 20, 21 | Bits | 47064 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 15264 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 50880 |
| Max. Throughput averaged over 2 frames | Mbps | 34.637 |