3GPP TSG-WG RAN4 Meeting #95-e	R4-2006590

# Online, 25th May – 5th June, 2020

Source: 		Nokia, Nokia, Shanghai Bell, BT plc

Title: 			TP to TR 37.716-21-11: DC_3-20_n1-n7

Agenda Item: 		8.7.2 [DC_R16_xBLTE_2BNR_yDL2UL-Core]

Document for:		Approval

# Introduction

This TP introduces the following EN-DC configurations for x bands LTE inter-band CA (2DL/1UL) and 2 NR band (2DL/1UL).

- DL_3A-20A_n1A-n7A_UL_3A_n1A

- DL_3A-20A_n1A-n7A_UL_3A_n7A

- DL_3A-20A_n1A-n7A_UL_20A_n1A

- DL_3A-20A_n1A-n7A_UL_20A_n7A

- DL_3C-20A_n1A-n7A_UL_3A_n1A

- DL_3C-20A_n1A-n7A_UL_3C_n1A

- DL_3C-20A_n1A-n7A_UL_3A_n7A

- DL_3C-20A_n1A-n7A_UL_3C_n7A

- DL_3C-20A_n1A-n7A_UL_20A_n1A

- DL_3C-20A_n1A-n7A_UL_20A_n7A

## TP to TR 37.716-21-21

## <Start of Changes>

## 6.X	DC_3-20_n1-n7

### 6.X.1	Operating bands for DC

Table 6.X.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_30-20_n1-n7 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

### 6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_20A_n1A DC_20A_n7A | 3 | 15 | See CA_3A-20A BCS in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 20 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| DC_3C-20A_n1A-n7A | DC_3A_n1A DC_3C_n1A DC_3A_n7A DC_3C_n7A DC_20A_n1A DC_20A_n7A | 3 | 15 | See CA_3C-20A BCS in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 20 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |

### 6.X.3	Co-existence studies

The harmonics and intermodulation studies have been already made in the fallback EN-DC, DC_20A_n1A-n7A, DC_3A_n1A-n7A, DC_3A-20A_n7A and DC_3A-20A_n1A.

### 6.X.4	∆TIB and ∆RIB values

For DC_3-20_n1-n7, the TIB,c and RIB values can be based on E-UTRA CA_1-3-7-20 given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1-n7 | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.6 |
|  | n7 | 0.6 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n1-n7 | 3 | 0 |
|  | 20 | 0 |
|  | n1 | 0 |
|  | n7 | 0 |

### 6.X.5	MSD

## All MSD issues are covered in 3DL/2UL and 2DL/2UL fallback modes.

## <End of Changes>