3GPP TSG-RAN WG4 Meeting #95-e	R4-2006610

Online, 25 May – 5 June, 2020

Source:	Huawei, HiSilicon, Bell Mobility, Telus

Title:	TP to TR 38.716-03-02 for CA_n5-n66-n78

Agenda item:	8.11.2

Document for:	Approval

# Background

This contribution provides text proposal on the NR CA band combination CA_n5-n66-n78 as defined in Revised WID on NR inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL [1].

# Text Proposal

##### ---Start of changes---

### 5.1.x	CA_n5-n66-n78

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: CA band combination of band n5+n66+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported bandwidths per CA band combination of band n5+n66+n78

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n5A-n66A-n78A | CA_n5A-n66A, CA_n5A-n78A, CA_n66A-n78A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

#### 5.1.x.3	UE co-existence studies

The harmonic and intermodulation issues have been studied for the constituent fall-back modes and the reference sensitivity exceptions have been define in TS 38.101-1.

Based on co-existence studies of Band n5 + Band n78, Band n66 + Band n78 and Band 5 + Band n66 captured in TR 38.716-02-00, own Rx impact of the 3rd band is the followings

-	3rd and 5th order IMD generated by dual uplink of Band n5 + Band n66 may fall into part of own band n78.

-	3rd  order IMD generated by dual uplink of Band n5 + Band n78 may fall into part of own band n66.

-	no IMD issue due to dual uplink of Band n66 + Band n78 falling into own band n5.

#### 5.1.x.4	∆TIB and ∆RIB values

The same TIB,c and RIB,c values specified for 1 band UL for CA_n3-n8-n78 are used as below.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n66-n78 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n66-n78 | n5 | 0.5 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.x.5	REFSENS requirements

Table 5.1.x.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD due to IMD3 of n5+n78 falling into n66 is reused from DC_5A-66A_n78A. Since Band n5 and Band n66 have similar UL frequency ranges to those of Band n8 and n3, respectively, the MSD values due to IMD3 and IMD5 of n5+n66 falling into n78 are derived from the CA_n3-n8-n78 in TR 38.716-03-02.

Table 5.1.x.5-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n5A-n66A-n78A | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | 16.1 | IMD3 ¦fn66+2*fn5¦ |
| CA_n5A-n66A-n78A | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | 4.5 | IMD5 ¦3*fn66 -2*fn5¦ |
| CA_n5A-n66A-n78A | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | 13.2 | IMD3 ¦2*fn5-fn78¦ |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |

##### ---End of changes---

# Reference

[1]	R4-2004181, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 3 bands DL with 2 bands UL”, ZTE Corporation