3GPP RAN-WG4 Meeting #95-e		R4-2006633

Electronic meeting, 25th May – 5th June 2020

Agenda item:	6.14.1.5

Source:	Apple Inc.

Title:		On common beam management assumptions and PSD difference in FR2 CA

WI/SI:	NR_RF_FR2_req_enh

Release:	Rel-16

Document for:	Approval

# 1	Introduction

Last meeting the PSD difference for inter-band FR2 discussion continued but no agreements were made.  Below you will find the agreements captured on the WF from RAN4 #93 meeting [1]. 
This contribution provides our view on the PSD imbalance for intra-band CA and inter-band CA as well as our view on common beam management.

WF on beam management - UE is assumed to be feasible to have independent beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 39 GHz combinations. - “28GHz” stands for a band group includes n257, n258, n261 - “39GHz” stands for a band group includes n259, n260 - Beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 28 GHz or 39 GHz + 39 GHz combinations. - Alt 1: UE is assumed to have common beam management - Alt 2: UE is assumed to have independent beam management WF on EIS spherical coverage - Spherical coverage requirements for inter-band CA are tested from single AoA for Rel-16 if the following testability solution can be provided. - Testability SI will study the TE capability of transmitting 28 GHz + 39 GHz, 28 GHz + 28 GHz, or 39 GHz + 39GHz from same direction simultaneously. - PSD condition among bands. - PSD difference up to TBD dB between 28GHz and 39GHz shall be considered in the conformance test configuration and [equal] PSD among 28+28 and 39+39 band groups - Confirm PSD condition for each scenario in RAN4#94. - Defining EIS spherical coverage requirement for inter-band CA. - The UE shall meet the EIS spherical coverage requirement simultaneously among bands, the common EIS spherical coverage range between the two bands shall be 50% for power class 3 UE. - Rel-15 EIS spherical coverage requirement will be taken as baseline assuming that the relaxation for 50%-tile point for power class 3 UE is introduced. The relaxation framework and value are FFS. Relaxation value cannot be 0.

# 2	Discussion

## 2.1	PSD

### 2.1.1	PSD difference for intra-band CA

MRTD requirement from the RRM has been defined in the specification TS38.133 [2] in subsection 7.6.4. The intra-band requirement on MRTD in FR2 is 0.26 us with an assumption on co-located deployment. In order to enable Rel-16 FR2 UEs it is important to aligning on the PSD levels in the case of intra-band CA. For a co-located deployment with a common beam management, we can assume an equal PSD between the CCs. Therefore, RAN shall assume 0 dB PSD imbalance between CCs for intra-band CA.

Proposal 1:	RAN4 shall assume an equal PSD between CCs for intra-band CA.

### 7.6.4	Minimum Requirements for NR Carrier Aggregation

For intra-band CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2 | 0.26 |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

For inter-band NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers to be aggregated at the UE receiver as shown in Table 7.6.4-2 below.

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2 | 8 |
| Between FR1 and FR2 | 25 |

### 2.1.2	PSD difference for inter-band CA

Table 1 lists our simulation parameters for the 28 GHz + 28 GHz, 39 GHz + 39 and 28 GHz + 39 GHz band groups. Scenario I considers the inter-band CA 28 GHz + 28 GHz, Scenario II considers inter-band CA 39 GHz + 39 GHz and Scenario III considers inter-band CA 28 GHz + 39 GHz.

Table 1: Simulation assumptions for PSD imbalance

| Simulation Parameters |  |
| --- | --- |
| Antenna array size | 4x1 |
| Element spacing | 5mm |
| Element pattern | See TR38.803 |
| Antenna impairments | Not considered |
| Phase shifter impairments | See TR38.817-01 |
| Transmission line impairments | Modeled TL length and loss per element |
| Frequency separation between carriers | Scenario I: [24.25 - 29.5] GHz Scenario II: [37.0 - 43.5] GHz Scenario II: [26.5 - 40.25] GHz |
| Gain difference between CC1 and CC2 | 0 dB and 6.5 dB |

#### 2.1.2.1	28 GHz + 28 GHz & 39 GHz + 39 GHz

Last agreed WF on beam management [1] listed two alternatives for the 28 GHz + 28 GHz and 39 GHz + 39 GHz band groups, the alternatives for the UE assumptions were common beam management or independent beam management. From our perspective, the UE shall assume common beam management between 28 GHz + 28 GHz and between 39 GHz + 39 GHz. Figure 1 (left) plots the coverage pattern for beam CC1 and beam CC2 in the azimuth plane. The figure shows how the beams overlap when considering the extreme frequencies within the band group “28 GHz”, such that the center frequency of beam CC1 is 24.25 GHz and the center frequency of CC2 is 29.5 GHz.

Figure 1 (right) provides the CDF Plot of the PSD between CC1 and CC2, it is possible to see that even though the power difference is 0 dB in RIB, after spatial filtering the power difference between CCs is larger than 0 dB. In this scenario the value at the 95%-tile of the CDF is 2.1 dB, the mean PSD is 1 dB and the maximum PSD is 6.8 dB. In other words, it can be expected a worst-case scenario of power imbalance between the two CCs of 6.8 dB after spatial filtering.

Figure 1: (Scenario I) Coverage Pattern Beam CC1 and Beam CC2 (left) and CDF Plot for  PSD - RIB 0 dB

In Scenario II we can observe a similar trend for the 39 GHz + 39 GHz, in Figure 2 (left) we see the beams overlap when considering the extreme of the frequency range within the band group “39 GHz”, the center frequency in this case for beam CC1 is 37 GHz and the center frequency of CC2 is 43.5 GHz. Figure 2 (right) gives the CDF Plot of the PSD and in this scenario a CC offset of 0 dB has been computed in RIB as well. The value at the 95%-tile of the CDF is 3.5 dB, the mean PSD is 1.7 dB and the maximum PSD is 9 dB. For this scenario the worst-case results in a power imbalance between the two CCs of 9 dB after spatial filtering.

Figure 2: (Scenario II) Coverage Pattern Beam CC1 and Beam CC2 (left) and CDF Plot for  PSD - RIB 0 dB

Summary of the simulation results are captured in Table 2 for 28 GHz + 28 GHz considering a CC offset of 0 dB in the RIB:

Table 2: Simulation results for 28 GHz + 28 GHz & 39 GHz + 39 GHz

| Parameters | 28 GHz + 28 GHz RiB 0 dB | 39 GHz + 39 GHz RiB 0 dB |
| --- | --- | --- |
| CDF 95%-tile | 2.1 dB | 3.5 dB |
| Max PSD | 6.8 dB | 9 dB |
| Mean PSD | 1 dB | 1.7 dB |

From the simulations results we can see that 0 dB PSD difference between CC1 and CC2 becomes after spatial filtering 2.1 dB for 28 GHz band group and 3.5 dB for 39 GHz band group at the 95%-tile of the CDF and with a mean PSD of 1 dB and 1.7 dB, respectively. Based on the provided simulations, we propose an equal PSD for 28 + 28 GHz and 39 + 39 GHz.

Proposal 2:	RAN4 shall define an equal PSD and co-located scenario for inter-band CA 28+28 GHz and 39+39 GHz at RIB.

#### 2.1.2.2	28 GHz + 39 GHz

Scenario III considers the inter-band CA of 28 GHz + 39 GHz. The plots for the coverage pattern in the azimuth plane show the Beam CC1 with a center frequency of 26.5 GHz and Beam CC2 with center frequency of 40.25. The center frequency of each beam is located in the center of the 28 GHz band group and 39 GHz band group, respectively. In this scenario we have included the simulation of a CC offset equal to 6.5 dB in addition to the simulation CC offset of 0 dB.

Figure 3 (right) shows the CDF Plot of the PSD with a CC offset of 0 dB in RIB. The value at the 95%-tile of the CDF is 9.8 dB, the mean PSD is 5.5 dB and the maximum PSD is 13.8 dB. In Scenario III a larger CC offset after spatial filtering is observed compared to Scenarios I and II due to the wider frequency separation between CCs and the independent beam management.

Figure 4 (right) gives the CDF Plot of the PSD with a CC offset of 6.5 dB in RIB. For this simulation the value at the 95%-tile of the CDF is 14.8 dB, the mean PSD is 7.7 dB and the maximum PSD is 20.5 dB. For this scenario the worst-case results in a power imbalance between the two CCs of 20.5 dB after spatial filtering when considering a CC offset of 6.5 dB in RIB.

We can see that there are two main parameters contributing to the PSD imbalance, the first one is the frequency separation between the CCs and the second is the relative gain differences between the CCs due to antenna array performance variation over the entire frequency span of the combination.

Figure 3: (Scenario III) Coverage Pattern Beam CC1 and Beam CC2 (left) and CDF Plot for  PSD – RIB 0 dB

Figure 4: (Scenario III) Coverage Pattern Beam CC1 and Beam CC2 (left) and CDF Plot for  PSD – RIB 6.5 dB

Summary of the simulation results for 28 GHz + 39 GHz considering a CC offset 0 dB and 6.5 dB in the RIB are provided in Table 3:

Table 3: Simulation results for 28 GHz + 39 GHz

| Parameters | 28 GHz + 39 GHz RIB 0 dB | 28 GHz + 39 GHz RIB 6.5 dB |
| --- | --- | --- |
| CDF 95%-tile | 9.8 dB | 14.8 dB |
| Max PSD | 13.8 dB | 20.5 dB |
| Mean PSD | 5.5 dB | 7.7 dB |

The simulations results demonstrate that 6.5 dB PSD difference between CC1 and CC2 becomes much larger after the spatial filtering, the initial 6.5 dB PSD difference in RIB results in 14.8 dB PSD difference (CDF 95%-tile) that the UE receiver needs to handle.

Due to the absence of filter technologies able to suppress aggressors and the weak selectivity of the antenna between 28 GHz and 39 GHz band groups, defining a large PSD difference between these band groups will result in a severe degradation of the receiver performance. Based on the simulation provided in this contribution, we propose a 6.5 dB for the PSD difference between 28 GHz and 39 GHz.

Proposal 3:	RAN4 shall define the PSD difference of 6.5 dB for inter-band CA 28 GHz + 39 GHz.

### 2.1.3	Homogeneous Network with distributed gNBs

In this subsection we provide our simulation results for analysis for the UE beam management with SLS when considering a homogeneous network at 28 GHz with distributed gNBs. The SLS setup consist of 19 sites with Euclidean distance wrap-around, each site has 3 sectors and in each sector 10 users are located [3]. The scenario considers a network ISD of 300 m. The UE antennas consist of 2 panels with 4x1 elements each panel and the BS antennas consist of 1 panel with 16x8 elements per sector, 3 sectors per site. For the simulation we have considered only azimuth plane (over 360 degrees) with no elevation.

We are modelling the Path loss (UMa-LOS) with a common beam management (CBM), in which the UE selects the best panel and best beam for CC1 and uses the same beam for CC2. It is worth mentioning that from UE perspective, we can make an assumption on which to define minimum requirements, but we should not restrict the UE from optimizations.

The figure on the right provides the CDF of the user to site path gain distribution, in which the green curve represents the 2nd strongest site (CC2) and the blue curve is the 1st strongest site (CC1). In this plot we can observe that the 95%-tile Path gain difference between the two CCs is 27.1 dB. In practice, the path loss difference would be even greater for sub-optimal choices of CC2 since the network would not always optimally select the gNB site for CC2. Based on this analysis we show that for the scenario of distributed gNBs with CBM the large path loss difference between CC1 and CC2 will make not possible a CA deployment. Therefore, we propose that for the minimum requirements in 28+28 GHz it shall be considered a co-located scenario.

Proposal 4:	RAN4 shall assume for inter-band CA 28+28 GHz a co-located scenario with CBM.

## 2.2	Common beam management for FR2 inter-band CA

Common beam management is assumed for FR2 intra-band CA since Rel-15. In this section, the assumptions of CBM and the related RRM requirements impacts are summarized.

All RRM requirements for intra-band FR2 are defined based on the assumption of CBM, which is defined in 3.6.3, TS38.133 as

3.6.3 Applicability for intra-band FR2

For the requirements in RRC connected state specified in this version of the specification, UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in the same band in FR2. Otherwise, the UE is not supposed to satisfy any requirements for SCell.

This assumption in section 3.6.3 consists of multiple important information for CBM, which includes

- Same Tx beam is assumed for all CC during one OFDM symbol across all CCs

- Symbol level alignment is assumed across all CC

If symbol boundary is not aligned with CP, one OFDM symbol from one CC will be partially overlapped with two OFDM symbols from the other CC. In this case, same Tx beam can never be assumed on one OFDM symbol across all CCs.

Observation 1: For FR2 intra-band CA, symbol level alignment within CP length is essential to guarantee the same downlink spatial domain transmission filter on one OFDM symbol.

With this assumption, the follow RRM requirements are specified

- In section 8.3.2 TS38.133, all FR2 being activated SCell can be considered as known cell if there is at least one active serving cell on that FR2 band

Typically, a cell can be defined as known cell only if the corresponding measurement is reported not long time ago such that the related timing and spatial information can still be assumed up to date. Otherwise, time and spatial tracking is needed. The aforementioned RRM requirement suggest that fine timing and spatial information from one CC can be directly used by the other CC when common beam management is assumed.

Observation 2: FR2 SCell activation requirements for intra-band CA suggests that when common beam management is assumed, fine timing and spatial information from one CC can be directly re-used by the other CC. This again makes it important that symbol level alignment should be with CP length.

The other related RRM requirements where L1 or L3 measurement on one CC can be reused for the other CCs include L3-RSRP/RSRQ, CBD, BFD etc.

Observation 3: For CBM based FR2 intra-band CA, L1 and L3 measurements on one CC can be reused for all the other CC. That implies the same Tx and Rx beams used across all CCs per OFDM symbol.

As defined in FR2 intra-band CA, CBM in FR2 inter-band CA should be also based on the assumption that the same Tx and Rx beams used across all CCs per OFDM symbol. That means UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in all CCs. To guarantee the same DL spatial domain transmission filter, the common beam management is only viable when the gNB for all CC are collocated and all CC should be aligned in slot level with MRTD less than CP. If different SCS is configured for different CC, the CP length of the largest SCS is considered. Meanwhile, it should be assumed as the baseline that single Rx antenna array at UE is shared by all CC.

Proposal 5: Common beam management in FR2 inter-band CA is defined as that UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in all CCs.

Proposal 6: In case of common beam management, it is assumed that gNB for all CC are collocated and single Rx antenna array at UE is shared by all CC.

Obviously, BS TAE requirement for inter-band CA, i.e. 3us, is too large for common beam management. More specifically, when MRTD exceeds CP length, it is impossible to assume the the same downlink spatial domain transmission filter on one OFDM symbol in all CCs.

# 3	Conclusions

This contribution provides our view on the PSD difference for intra-band CA and inter-band CA in FR2 and makes the following proposals:

Proposal 1:	RAN4 shall assume an equal PSD between CCs for intra-band CA.

Proposal 2:	RAN4 shall define an equal PSD and co-located scenario for inter-band CA 28+28 GHz and 39+39 GHz.

Proposal 3:	RAN4 shall define the PSD difference of 6.5 dB for inter-band CA 28 GHz + 39 GHz.

Proposal 4:	RAN4 shall assume for inter-band CA 28+28 GHz a co-located scenario with CBM.

On common beam management, the observation on beam management related implications in TS38.133 are summarized as

Observation 1: For FR2 intra-band CA, symbol level alignment within CP length is essential to guarantee the same downlink spatial domain transmission filter on one OFDM symbol.

Observation 2: FR2 SCell activation requirements for intra-band CA suggests that when common beam management is assumed, fine timing and spatial information from one CC can be directly re-used by the other CC. This again makes it important that symbol level alignment should be with CP length.

Observation 3: For CBM based FR2 intra-band CA, L1 and L3 measurements on one CC can be reused for all the other CC. That implies the same Tx and Rx beams used across all CCs per OFDM symbol.

With this, it is proposed that

Proposal 5: Common beam management in FR2 inter-band CA is defined as that UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in all CCs.

Proposal 6: In case of common beam management, it is assumed that gNB for all CC are collocated and single Rx antenna array at UE is shared by all CC.

# 4	References

- R4-1916024, “WF on FR2 inter-band DL CA,” Nokia, Nokia Shanghai Bell, 3GPP RAN4 #93, November 2019

- 3GPP TS 38.133

- 3GPP TR 38.803