3GPP TSG-RAN WG4 Meeting #95-e	R4-2006661

Electronic Meeting, 25th May - 5th Jun, 2020

Source:	ZTE Corporation

Title:	Initial simulation results on BS demodulation for 2-step RACH

Agenda Item:	6.20.2

Document for:	Discussion

# Introduction

In RAN4#94bis-e, a way forward was agreed on BS demodulation performance requirements for 2-step RACH [1], and some initial setup for performance evaluation was listed. In this contribution, we provide simulation results based on these initial setup.

Table-1 Initial simulation setup

| Parameter | Values |
| --- | --- |
| Waveform | CP-OFDM |
| Subcarrier spacing for PUSCH | FR1: 15kHz, 30kHz FR2: 60kHz, 120 kHz |
| PUSCH Mapping type | FR1: A FR2: B |
| Number of symbols | FR1: 14 FR2: 10 |
| Number of PRBs | 1 PRB, 2 PRBs |
| TBS | - 56 bits as starting point for minimum payload size - Other values are not precluded |
| DMRS | 1+1+1 |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx |
| Propagation channel & UE velocity | TDL-A 30ns, TDL-C 300ns, 3km/h |
| TO | 15 kHz: 6.77, 1.92, 0.77 us 30 kHz: 1.92, 0.77, 0.38 us 60 kHz: 0.77, 0.38 us 120 kHz: 0.38, 0.19 us Other options are not precluded |
| Test metric | BLER of MsgA when preambles are correctly detected - Option 1: 0.01 - Option 2: 0.1 - Other values are not precluded |

# Discussion

The simulated performance curves are shown in the following figures:

Fig. 1, Simulated performance for TDL-A, SCS = 15 kHz, FR1

Fig. 2, Simulated performances for TDL-C, SCS = 15 kHz, FR1

Fig. 3, Simulated performances for TDL-A, SCS = 30 kHz, FR1

Fig. 4, Simulated performances for TDL-C, SCS = 30 kHz, FR1

Fig. 5, Simulated performances for TDL-A, SCS = 60 kHz, FR1

Fig. 6, Simulated performances for TDL-C, SCS = 60 kHz, FR1

Fig. 7, Simulated performances for TDL-A, SCS = 120 kHz, FR1

Fig. 8, Simulated performances for TDL-C, SCS = 120 kHz, FR1

Table-2 SNRs (dB) under different conditions

| SCS (kHz) | Number of PRB(s) | Propagation Channel | TO (us) | SNR(dB) for BLER=0.1 | SNR(dB) for BLER=0.01 |
| --- | --- | --- | --- | --- | --- |
| 15 | 1 | TDLA30 | 6.77 | 2.69 | 8.39 |
|  |  |  | 3.85 | 2.05 | 8 |
|  |  |  | 1.92 | 1.88 | 7.61 |
|  |  |  | 0.77 | 1.84 | 7.55 |
|  |  | TDLC300 | 6.77 | 2.54 | 7.79 |
|  |  |  | 3.85 | 1.88 | 6.93 |
|  |  |  | 1.92 | 1.77 | 6.85 |
|  |  |  | 0.77 | 1.76 | 6.82 |
| 15 | 2 | TDLA30 | 6.77 | -0.88 | 4.94 |
|  |  |  | 3.85 | -1.72 | 3.94 |
|  |  |  | 1.92 | -2.05 | 3.55 |
|  |  |  | 0.77 | -2.1 | 3.67 |
|  |  | TDLC300 | 6.77 | -0.48 | 4.42 |
|  |  |  | 3.85 | -1.07 | 3.92 |
|  |  |  | 1.92 | -1.19 | 3.78 |
|  |  |  | 0.77 | -1.21 | 3.54 |
| 30 | 1 | TDLA30 | 1.92 | 1.76 | 7.45 |
|  |  |  | 0.77 | 1.67 | 7.64 |
|  |  |  | 0.38 | 1.69 | 7.67 |
|  |  | TDLC300 | 1.92 | 1.89 | 6.82 |
|  |  |  | 0.77 | 1.7 | 6.79 |
|  |  |  | 0.38 | 1.71 | 6.61 |
| 30 | 2 | TDLA30 | 1.92 | -1.8 | 4.22 |
|  |  |  | 0.77 | -2 | 3.81 |
|  |  |  | 0.38 | -2.19 | 3.85 |
|  |  | TDLC300 | 1.92 | -1.59 | 2.39 |
|  |  |  | 0.77 | -1.73 | 2.21 |
|  |  |  | 0.38 | -1.75 | 2.1 |
| 60 | 1 | TDLA30 | 0.77 | 3.57 | 9.17 |
|  |  |  | 0.38 | 3.45 | 9.07 |
|  |  | TDLC300 | 0.77 | 3.79 | 8.69 |
|  |  |  | 0.38 | 3.72 | 8.53 |
| 60 | 2 | TDLA30 | 0.77 | -0.06 | 5.38 |
|  |  |  | 0.38 | -0.27 | 5.38 |
|  |  | TDLC300 | 0.77 | -0.01 | 3.88 |
|  |  |  | 0.38 | -0.14 | 3.8 |
| 120 | 1 | TDLA30 | 0.38 | 3.14 | 8.7 |
|  |  |  | 0.19 | 2.94 | 8.6 |
|  |  | TDLC300 | 0.38 | 3.55 | 8.95 |
|  |  |  | 0.19 | 3.39 | 8.73 |
| 120 | 2 | TDLA30 | 0.38 | -0.65 | 4.04 |
|  |  |  | 0.19 | -0.89 | 3.95 |
|  |  | TDLC300 | 0.38 | -0.93 | 2.62 |
|  |  |  | 0.19 | -1.12 | 2.39 |

# Reference

[1] R4-2005555 WF on BS demodulation performance requirements for 2-step RACH, ZTE