3GPP TSG-RAN WG4 Meeting #95-e                 	 R4-2006888

Electronic Meeting, 25th May – 5th June, 2020

Agenda Item:	6.15.1.5

Source:	MediaTek Inc.

Title:	Inter-frequency measurement requirement without gap

Document for:		Discussion

# Introduction

In last meeting, a WF [1] was agreed to capture the conclusions of the discussion as well as the open issues to be further addressed in this meeting. In short the open issues are

Issue #1: Capability of supporting inter-frequency measurement without MG - Option 1 (vivo, MediaTek, OPPO, Huawei, Qualcomm, vivo, Intel, Ericsson) : Optional with UE capability signaling - Option 2 (CMCC): Mandatory with UE capability signaling Issue #2: Scheduling restriction when the target SSB has a different SCS grid - Option 1 (MediaTek, OPPO, Qualcomm) : When the target SSB has a different SCS grid as that of UE’s serving cell, UE is allowed to have scheduling restriction in the entire SMTC duration. - Option 2 (Huawei, Apple, CMCC, Huawei, Ericsson): No need to specify the scheduling restriction for the case the target SSB has a different SCS grid as that of UE’s serving cell.

In this paper, we provide our views on above issues #1 and #2.

# Discussion

Issue #1: UE capability

Considering that this feature might need additional UE implementation effort. So we prefer

Proposal 1: Inter-frequency measurement requirement without gap is an optional feature with UE capability signaling.

Issue #2: Scheduling restriction when the target SSB has a different SCS grid

The SCS applicable for SSB are 15KHz, 30KHz, 120KHz and 240KHz, while the granularity of ARFCN that are used in MO to indicate the frequency location of SSB can be 5KHz.

In TS38.331 MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected … In TS38.101-1

That means the spec allows the indication of an inter-frequency MO with SSB not on the same SCS grid as UE’s serving cell, even if they are with the same SCS. Figure 1 provides one example with 15KHz SCS for both UE’s serving cell and the target SSB but on different grids. In this case, additional effort is required by UE to perform measurement and data reception at the same time.

Figure 1. Unaligned grid between target SSB and UE’s serving cell

Introducing a UE capability to indicate this additional capability to network could be one solution. However, we believe this scenario can be easily avoided by network deployment. So adding new UE capability does not make much sense. An easiest way to address this issue is to add a condition in the scheduling restriction requirements, e.g., when the target SSB has a different SCS grid as that of UE’s serving cell, UE is allowed to have scheduling restriction in the entire SMTC duration.

Proposal 2: When the target SSB has a different SCS grid as that of UE’s serving cell, UE is allowed to have scheduling restriction in the entire SMTC duration.

# Summary

In this paper, we discuss the requirement of inter-frequency measurement without gap. We have the following proposals.

Proposal 1: Inter-frequency measurement requirement without gap is an optional feature with UE capability signaling.

Proposal 2: When the target SSB has a different SCS grid as that of UE’s serving cell, UE is allowed to have scheduling restriction in the entire SMTC duration.

# Reference

[1] R4-2005348, “WF on inter-frequency without MG”, CMCC, Apr., 2020.