3GPP TSG-RAN WG4 Meeting #95	R4-2006949

Electronic Meeting, 25 May. – 5 Jun., 2020

Agenda item:	6.16.1.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on CSI-RS parameters for RRM core requirements

Document for:	Discussion

1. Introduction

At the RAN4#94-bis e-meeting, the WF on the core requirements of CSI-RS based L3 measurement was agreed[1]. Followings are the agreements on the configuration sets of the number of PRBs and density of CSI-RS.

WF on CSI-RS configuration for measurement requirement

- Define requirements for CSI-RS configuration with {D=3 with PRBs ≥ 48}. FFS whether to define requirements for {D=1 with PRBs ≥ 96}.

- Not define additional CSI-RS based measurement requirement

In this contribution, we show our views on whether to define requirements or not about the configuration set of {D=1 with PRBs ≥ 96} which is concluded as FFS at the previous meeting.

2. Discussion

We think one of the advantages of the L3 measurement with CSI-RS compared to that with SSB is its configurability. For example, the number of configuration patterns of CSI-RS is greatly higher than that of SSB. Table 1 shows the number of CSI-RS REs depending on the number of PRBs and the density. In the point of view of the number of REs, CSI-RS for L3 measurement can be configured with 10 patterns at the maximum as shown in Table 1, while SSB for L3 measurement has only one pattern of the number of REs.

Table 1. The number of REs of CSI-RS

|  |  | Number of PRBs |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 24 | 48 | 96 | 192 | 264 |
| Density | 1 | 24 | 48 | 96 | 192 | 264 |
|  | 3 | 72 | 144 | 288 | 576 | 792 |

However, according to the agreements from the discussions in some recent meetings, we only consider 1 or 2 patterns of configuration patterns of CSI-RS to specify requirements. At the moment, we think there remain very few patterns considering the flexibility of CSI-RS, thus we should specify requirements for as many configuration patterns as possible. In this sense, we prefer to specify requirements not only {D=3 with PRBs ≥ 48} but also {D=1 with PRBs ≥ 96}.

Proposal 1: The core requirements for CSI-RS based L3 measurement should be specified so that they can guarantee as many configuration patterns as possible for keeping its flexibility.

Proposal 2: We prefer to specify requirements for {D=1 with PRBs ≥ 96}.

3. Conclusion

In this contribution, we showed our views on whether to define requirements or not about the configuration set of {D=1 with PRBs ≥ 96}. Proposals are shown below.

Proposal 1: The core requirements for CSI-RS based L3 measurement should be specified so that they can guarantee as many configuration patterns as possible for keeping its flexibility.

Proposal 2: We prefer to specify requirements for {D=1 with PRBs ≥ 96}.

References

[1] R4-2005355, CATT, “WF on CSI-RS configuration and intra/inter-frequency measurements definition for CSI-RS based L3 measurement,” Apr. 2020.