3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2007061

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.124 | CR | 0004 | rev | <-> | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.124 on Emissions and Immunity Applicability and ranges |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Alignment with ETSI discussion on Emissions and Immunity Applicability and frequency ranges. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adjustment of Emissions and Immunity Applicability including wired network ports and updating the frequency range for RI testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignament with ETSI standard and lack of information on Emissions and Immunity applicability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1;7.2;9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of new section ------------------------------

## 7.1	EMC emissions

Table 7.1-1: Emission applicability

|  |  | Equipment test requirement |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Phenomenon | Application | Equipment connected to fixed AC or DC power installations | Equipment connected to vehicular DC supplies | Equipment powered by integral battery | Reference clause in the present document | Reference standard |
| Radiated emission | Enclosure | applicable | applicable | applicable | 8.2 | ITU-R SM.329 [5] TS 38.101-1 [3] |
| Conducted emission | DC power input/output port | applicable | applicable | not applicable | 8.3 | CISPR 32 [20], CISPR 16-1 [7] |
| Conducted emission | AC mains input/output port | applicable | not applicable | not applicable | 8.4 | CISPR 32 [20] |
| Harmonic current emissions | AC mains input port | applicable | not applicable | not applicable | 8.5 | IEC 61000-3-2 [8] |
| Voltage fluctuations and flicker | AC mains input port | applicable | not applicable | not applicable | 8.6 | IEC 61000-3-3 [9] |
| conducted emission | wired network port | applicable | not applicable | not applicable | 8,7 | CISPR 32 [20] |

For UE equipment operating in FR1 (table 4.1-1), the radiated emission applies to the enclosure port antenna ports available.

For UE equipment supporting operations in FR2 (table 4.1-1) the enclosure port is inseparable from the antenna port.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

## 7.2	Immunity

Table 7.2-1: Immunity applicability

|  |  | Equipment test requirement |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Phenomenon | Application | Equipment connected to fixed AC or DC power installations | Equipment connected to vehicular DC supplies | Equipment powered by integral battery | Reference clause in the present document | Reference standard |
| RF electromagnetic field (80 MHz to 6000 MHz) | Enclosure | applicable | applicable | applicable | 9.2 | IEC 61000-4-3 [10] |
| Electrostatic discharge | Enclosure | applicable | applicable | applicable | 9.3 | IEC 61000-4-2 [11] |
| Fast transients common mode | Signal, wired network and control ports, DC and AC power input ports | applicable | not applicable | not applicable | 9.4 | IEC 61000-4-4 [12] |
| RF common mode 0.15 MHz to 80 MHz | Signal, wired network and control ports, DC and AC power input ports | applicable | applicable | applicable | 9.5 | IEC 61000-4-6 [13] |
| Transients and surges, vehicular environment | DC power input ports | not applicable | applicable | not applicable | 9.6 | ISO 7637 Part 1 [14] and ISO 7637 Part 2 [15] |
| Voltage dips and interruptions | AC mains power input ports | applicable | not applicable | not applicable | 9.7 | IEC 61000-4-11 [16] |
| Surges, common and differential mode | DC and AC power input and wired network ports | applicable | not applicable | not applicable | 9.8 | IEC 61000-4-5 [17] |

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

## 9.2	RF electromagnetic field (80 MHz – 6000 MHz)

The test shall be performed on a representative configuration of the equipment or a representative configuration of the combination of UE and ancillary equipment.

### 9.2.1	Definition

This test assesses the ability of UE and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic field disturbance at the enclosure.

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [10]:

-	for UE and ancillary equipment, the following requirements shall apply;

-	the test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	the stepped frequency increments shall be 1 % of the momentary frequency;

-	when using the max hold detector method (see annex A) at each test frequency step initially an unmodulated test signal shall be applied. Then the test modulation shall be applied;

-	the test shall be performed over the frequency range 80 MHz to 6000 MHz;

-	responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see clause 4.3;

-	the frequencies selected during the test shall be recorded in the test report.

### 9.2.3	Performance criteria

The performance criteria of clause 6.1 shall apply.

------------------------------ End of section ------------------------------