3GPP TSG-RAN WG4 Meeting #95-e	R4-2007238

# Electronic Meeting, 25th May - 5th Jun, 2020

Title: 	Discussion on NR 2-step RACH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.20.2

Document for:	Discussion

Background

During RAN4#94e-bis meeting, way forward [1] for NR 2-step RACH demodulation was approved. In this contribution, we share our views about the BS demodulation requirements for NR 2-step RACH.

Discussion

Whether or not TO compensation is assumed when specifying BS performance requirements

For some cases with large TO, if TO compensation is not performed, performance degradation will become negligible. We propose to define performance requirements for the cases that a large performance difference can be observed between performing TO compensation or not.

Proposal 1: Define performance requirements for the cases that obvious performance difference should be observed between performing TO compensation and not.

Whether or not to exclude Local Area BS for BS demodulation requirements for 2-step RACH

Performance requirements for 2-step RACH is aim to test whether BS can properly handle larger TO. For Local Area BS, no performance degradation with and without TO compensation. Therefore, we don’t think it is necessary to define performance requirements for Local Area BS.

Proposal 2: Exclude Local Area BS for BS demodulation requirements for 2-step RACH.

Simulation parameters

For PUSCH mapping type, Different manufacturers may use different configurations. Same as NR Rel-15, PUSCH performance requirements for both PUSCH mapping Type A and Type B for FR1 are defined, we propose to follow NR Rel-15 to consider both Type A and Type B with test applicability rule, i.e. BS can be tested either PUSCH mapping type A or B based on declaration.

Proposal 3: Consider both PUSCH mapping Type A and Type B for performance requirements definition with test applicability rule.

Proposals

In this contribution, we discuss on NR 2-step RACH performance requirements. Our observations and proposals are:

Proposal 1: Define performance requirements for the cases that obvious performance difference should be observed between performing TO compensation and not.

Proposal 2: Exclude Local Area BS for BS demodulation requirements for 2-step RACH.

Proposal 3: Consider both PUSCH mapping Type A and Type B for performance requirements definition with test applicability rule.

Reference

R4-2005555, WF on BS demodulation performance requirements for 2-step RACH, RAN4#94e-bis, ZTE