3GPP TSG-RAN WG4 Meeting #95-e	R4-2007351

Electronic Meeting, 25 May – 5 June, 2020

Title:	On remaining issues for definition of intra-frequency measurement

Source:	OPPO

Agenda Item:	6.16.1.2

Document for:	Discussion

- Introduction

In RAN4#94e-bis meeting, a WF [1] on intra/inter-frequency measurements definition for CSI-RS based L3 measurement was agreed. In this contribution, we provide our views on the remaining issues.

- Discussion

- Whether to share the same intra/inter-frequency definition

- The CSI-RS configurations in the same MO share the same intra/inter-frequency definition.

- FFS the SSB and CSI-RS configured in the same MO share the same intra/inter-frequency definition

When network indicates a certain MO as a serving cell MO, then the measurement on the SSB and all CSI-RS resources in this MO should be considered as intra-frequency measurement. In this way, the definition of intra-frequency avoids contradiction between RAN2 and RAN4 specs.

Proposal 1: SSB and CSI-RS configured in the same MO share the same intra/inter-frequency definition.

- Whether CSI-RS resource on the serving cell indicated in servingCellMO

- When CSI-RS resource of serving cell is available

  - CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of CSI-RS on the serving cell and neighbor cell is the same, and

    - the CP type of CSI-RS on serving cell and target cell is the same, and

      - It is applied for SCS = 60KHz

    - Option 1 (Intel, MTK, Huawei, Apple, OPPO)

      - the centre frequency of CSI-RS resources on the target cell configured for measurement is the same as centre frequency of CSI-RS resource on the serving cell indicated in servingCellMO

    - Option 2 (ZTE, QC, CATT, NEC, Nokia, DOCOMO)

      - the centre frequency of CSI-RS resources on the target cell configured for measurement is the same as centre frequency of CSI-RS resource on the serving cell

      - Common understanding: centre frequency of CSI-RS resource on the serving cell is indicated in servingCellMO

  - CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement if it is not a CSI-RS based intra-frequency measurement

- When CSI-RS resource of serving cell is not available

  - Option 1: All MO are inter-frequency (ZTE, vivo, Apple, CATT, NEC, DOCOMO)

  - Option 2: No requirement is applied (Apple, Nokia, CATT, Qualcomm, MTK)

  - Other options are not precluded

When CSI-RS resource of serving cell is available, how to decide the CSI-RS resource on the serving cell needs to be clarified. It is common understanding that centre frequency of CSI-RS resource on the serving cell is indicated in servingCellMO. We share the similar view that if an MO is indicated as servingCellMO, then it should always be an intra-frequency MO due to CSI-RS transmitted by UE’s serving cells provided in this MO.

Proposal 2: Support option 1 for intra-frequency measurement that

- the centre frequency of CSI-RS resources on the target cell configured for measurement is the same as centre frequency of CSI-RS resource on the serving cell indicated in servingCellMO.

In addition, when CSI-RS resource of serving cell is not available, we prefer option 2 no requirements is applied.

Proposal 3: No requirement is applied when CSI-RS resource of serving cell is not available.

- Conclusion

In this contribution, we proposed our observations as follows.

Proposal 1: SSB and CSI-RS configured in the same MO share the same intra/inter-frequency definition.

Proposal 2: Support option 1 for intra-frequency measurement that

- the centre frequency of CSI-RS resources on the target cell configured for measurement is the same as centre frequency of CSI-RS resource on the serving cell indicated in servingCellMO.

Proposal 3: No requirement is applied when CSI-RS resource of serving cell is not available.

- Reference

- R4-2005355，WF on CSI-RS configuration and intra/inter-frequency measurements definition for CSI-RS based L3 measurement，CATT