3GPP TSG-RAN WG4 Meeting #95-e	R4-2007382

Electronic Meeting, 25 May – 5 June 2020

Agenda Item:	6.17.2.1.1

Source:	Ericsson

Title:	Transmission scheme in NR PDSCH demodulation requirements for HST

Document for:	Discussion

# 1	Introduction

RAN4#94-e-Bis discussed the transmission schemes assumed for UE demodulation requirements with high speed train scenario and agreed with the way forward [1].

- Further discuss the test case design for DPS transmission scheme 1, and verify whether new specific UE receiver processing from demod aspect required compared to UE to handle HST-single Tap and HST-SFN channel model.

We discuss our view on the UE receiver processing used for DPS and HST-SFN joint transmission.

# 2	Discussion

In our understanding the difference between JT and DPS is how to update channel analysis information if we exclude TCI handling. This means once UE decides the channel estimation filter, UE can apply the same PDSCH demodulation for both JT and DPS. Therefore we will focus the discussion on the TRS processing for channel analysis.

Figure 1 illustrates our understanding of TCI configurations for HST-HST Joint Transmission (JT) and DPS with HST-SFN configurations. In the figure we assume two TRPs share the same cell ID and SSB#0 is transmitted from TRP#0. Note SSB#0 can be also transmitted from both TRP#0 and TRP#1.

| (a) TCI configurations for HST-SFN JT | (b) TCI configurations for DPS with HST-SFN deployment |
| --- | --- |

Figure 1	Comparison of TCI configurations between HST-SFN Joint Transmission and DPS with HST-SFN configurations.

In the case of JT, gNB transmits one TRP#0 from both TRPs and CORESET/PDSCH is QCLed with TRP#0. In this configuration, if UE is capable of the advanced receiver for HST-SFN, DemodulationEnhancementforNR-HST, UE are supposed to estimate the Doppler shift per TRP and demodulate PDCCH/PDSCH accordingly. To enable the advanced receiver, UE uses the network assisted signaling (highSpeedEnhDemodFlagforNR).

For DPS, gNB configures two TRSs; TRS#0 for TRP#0 (TCI#1) and TRS#1 for TRP#1 (TCI#2). CORESET/PDSCH is configured with TCI#1 and TCI#2, but UE monitors CORESET from one of TCIs activated by MAC-CE if UE is capable of one active TCI state monitoring. If UE is capable of two or more active TCI states monitoring, UE will monitor CORESET from both TCIs. Because of the per-TRP TRS configuration, UE can perform the channel analysis (e.g., Doppler estimation, Delay spread estimation) per TRP without the advanced receiver.

| (a) Doppler shift for JT | (b) Doppler shift for DPS |
| --- | --- |

Figure 2	Comparison of Doppler shift between HST-SFN JT and DPS.

Figure 2 illustrates the Doppler shift from the visible TRPs. In the case of JT, Figure 2 (a), as we discussed, the same TRS symbols are transmitted from two or more TRPs and UE advanced receiver will update the channel analysis information by selecting the strongest path or combine paths according to the signal strength.

For DPS, on the other hand, since the test equipment selects one TRP based on the simulated UE location, UE only need to update the channel analysis information based on the transmitted TRS as in Figure 2 (b). This is the reason DPS is similar to the HST single tap test case. RAN4 also discussed the difference between DPS and single tap is the big Doppler shift jump. This happens when the network indicates the TCI state changes, but it is signaled from the gNB and UE just update (reset) the channel information. If companies want to verify the verify the Doppler tracking per TRP, we think it could be covered by eMIMO multi-TRP transmission requirements.

In summary we don’t think any new UE receiver processing is required from demodulation aspect compared to HST-single Tap and HST-SFN channel model. Moreover handling of TCI switching can by verified by eMIMO multi-TRP transmission requirements.

Proposal 1: RAN4 does not define new PDSCH demodulation requirements assuming DPS in HST WI.

# 3	Summary

Proposal 1: RAN4 does not define new PDSCH demodulation requirements assuming DPS in HST WI.

# References

- R4- 2005532, “WF on UE demodulation for NR HST”, CMCC.