3GPP TSG-RAN WG4 Meeting # 95-e 														R4-2007578 Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.5.2.2.3

Source: 	Ericsson

Title:	TP to TR 38.809: OTA In-band blocking

Document for:	Approval

- Introduction

After RAN4#94bis-e, the IAB TR [1] is email approved. Section 2 provides text proposal on 10 OTA In-band blocking for the IAB TR 38.809 (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

#### 10.5	OTA in-band selectivity and blocking

#### 10.5.1 In-band blocking for IAB-MT of type 2-O

Traditionally the in-band blocker is set according to the 99.99% probability of interferer cdf curve. It is recommended in [6] that between 99% and 99.9% could be considered due to the NR OFDMA scheme does not suffer greatly as for WCDMA.

As the interfere level to IAB-MT receiver relates to the distance between victim IAB and aggressor BS, coexisting simulation is done assuming the 40m, 50m , 60m and 80m and companies result is captured in Table 10.5.3-1.

Table 10.5.3-1: Coexisting simulation result for In-band blocking level comparision

| Company |  | Blocker level @99 Percentile point (dBm) |  | Blocker level @ 99.9 perncentile point (dBm) |  | Physical distance to Agressor BS (m) |  | Simulation scenario (Acc. to R4-1907825) |  | RAN4 contribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | -67.5 | -65.5 | 40 | Layout 2 for FR2 | R4-2001873 |
|  | -75 | -66 | 50 |  |  |
|  | -81 | -75 | 60 |  |  |
|  | -55 | -48 | 40 | Layout 2 for FR1 |  |
| Nokia | -52 | -45 | 40 | Layout 2 for FR2 | R4-2001432 |
|  | -55 | -54 | Max grid shift |  |  |
| Qualcommn incopration | -45 |  | 40 | Layout 2 for FR2 | R4-2001282 |
|  | -49 |  | 60 |  |  |
|  | -53 |  | 80 |  |  |
| Huawei | -64 |  | 40 | Layout 2 for FR2 (element OTA result) | R4-1914757 |
|  | -69 |  | 60 |  |  |
| ZTE | -42 |  | 40 | Layout 2 for FR1(element OTA result) | R4-2000977 |

It is concluded from the coexisting simulation results in Table 10.5.3-1 that IAB-MT needs to improve the inband blocking level compared with the NR OTA BS. Therefore, the inband blocking level for wide area IAB-MT of type 2-O is specified with 33 dB higher than OTA REFSENS power level with inteferer signal of CP-OFDM waveform. The inband blocking level for local area IAB-MT of type 2-O is specified with 35.5 higher than OTA REFSENS power level with inteferer signal of CP-OFDM waveform for bands n257, n258, n261. The inband blocking level for local area IAB-MT of type 2-O is specified with 34.5 higher than OTA REFSENS power level with CP-OFDM waveform of inteferer signal for bands n260.

--------------------------------------------------End of TP------------------------------------------------------

- References

- TR 38.809, Background for Integrated access and backhaul radio transmission and reception