3GPP TSG-RAN WG4 Meeting # 95-e 	                             R4-2007684

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:	6.5.3.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on RLM requirement for IAB-MT

Document for:	Discussion

- Introduction

In last meeting RAN4 94-e-bis meeting, the RRM measurement for IAB-MT were discussed. There are some remaining issues about RLM. We further investigate these issues and present views in this paper.

- Discussion

In the last RAN4 94-e-bis meeting, it was agreed that the RLM evaluation period of IAB-MT shall be scaled with respect to that of Rel-15 UEs [1]. The remaining issues are the value of the scaling factor for FR1 and FR2:

The CSI-RS requirements shall apply only for CSI-RS density 3 over the bandwidth ≥ 24 PRBs. Agreement: The following RLM related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs. - Measurement restrictions for SSB based RLM and CSI-RS based RLM - Minimum requirement at transitions - Minimum requirement for L1 indication. - Scheduling availability during RLM - Relax the RLM evaluation period of IAB-MTs with respect to that of Rel-15 UEs - Scale the number of samples and the lower boundary of the SSB and CSI-RS based RLM evaluation period by a factor. - FFS: The value of the scaling factor - FFS: different values of scaling factor for FR1 and FR2 - Sharing factor P is also necessary since measurement gap can be configured for IAB-MTs.

As mentioned in previous discussion, the IAB is deployed as fixed node without mobility, which leads to less possibility of radio link failure. Power consumptions and efforts can be saved by relaxing RLM evaluation period for both out-of-sync and in-sync. From the previous discussion, extending the evaluation period by a scaling factor of 5 is a reasonable number for both FR1 and FR2.

Proposal 1: It is suggested to extend the evaluation period in TS 38.133 by scaling factor of 5 for IAB MT RLM for both FR1 and FR2.

- Conclusion

Proposal 1: It is suggested to extend the evaluation period in TS 38.133 by scaling factor of 5 for IAB MT RLM.

References

[1] R4-2005322, WF on RLM requirements and sharing factor in RLM/BFD/CBD evaluation for IAB-MTs