3GPP TSG-RAN4  #95-e															     	R4-2007880

25th May – 5th June 2020

Source: 	Qualcomm Incorporated

Title: 	TP to TR38.886 V2X PSSCH PSCCH A-MPR

Agenda item:	6.4.3.1

Document for:	Approval

- Introduction

We study the required A-MPR and propose a specification structure and values.	The study is conducted against requirements signaled under NS_33 and NS_NEW.

- Simulation description and findings

All valid waveforms were simulated at fc = 5860, 5870, …, 5920 MHz for 15, 30, and 60 kHz SCS; and all subband lengths (10,15,20,25, 50,75 and 100) . Plots of some of these waveforms are shown in R4-2003438.

- 10MHz 5870 to 5920 with NS_33 and 5860 with NS_NEW

For 5870-5920 under NS_33 the data in Table 1 is used to formulate the expression for A-MPR. The outer allocations in carriers nearer the edges of the band exhibit some sensitivity to Gpostconnector (5870, 5910, and 5920 MHz). Inner allocations are not affected by Gpostconnector . The effect of Gpostconnector  is higher for 15 kHz SCS than for 30 kHz SCS.

The simple formulation for backoff shown in Table 2. A-MPRStep is used to adjust  A-MPR with Gpostconnector as is done in 36.101.

Table 1: Backoff vs Gpostconnector for Inner and Outer Allocations under NS_33

|  |  | Backoff (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5870 MHz |  |  |  | 5880 MHz |  |  |  | 5910 MHz |  |  |  | 5920 MHz |  |  |  |
|  |  | Gpost=0 |  | Gpost=7 |  | Gpost=0 |  | Gpost=7 |  | Gpost=0 |  | Gpost=7 |  | Gpost=0 |  | Gpost=7 |  |
| SCS | Mod | Inr | Out | Inr | Out | Inr | Out | Inr | Out | Inr | Out | Inr | Out | Inr | Out | Inr | Out |
| 15 | 2 | 2.7 | 4.3 | 2.7 | 5.7 | 2.7 | 4.3 | 2.7 | 4.3 | 2.7 | 4.3 | 2.7 | 5.6 | 2.7 | 4.3 | 2.7 | 5.6 |
| 15 | 4 | 2.8 | 4.3 | 2.8 | 5.6 | 2.8 | 4.3 | 2.8 | 4.3 | 2.8 | 4.3 | 2.8 | 5.6 | 2.8 | 4.3 | 2.8 | 5.6 |
| 15 | 6 | 2.7 | 4.3 | 2.7 | 5.7 | 2.7 | 4.3 | 2.7 | 4.3 | 2.7 | 4.3 | 2.7 | 5.7 | 2.7 | 4.3 | 2.7 | 5.7 |
| 15 | 8 | 2.8 | 4.3 | 2.8 | 5.6 | 2.8 | 4.3 | 2.8 | 4.3 | 2.8 | 4.3 | 2.8 | 5.6 | 2.8 | 4.3 | 2.8 | 5.6 |
| 30 | 2 | 2.6 | 4.2 | 2.6 | 4.8 | 2.6 | 4.2 | 2.6 | 4.2 | 2.6 | 4.2 | 2.8 | 4.8 | 2.6 | 4.2 | 2.6 | 4.8 |
| 30 | 4 | 2.5 | 4.2 | 2.5 | 4.8 | 2.5 | 4.2 | 2.5 | 4.2 | 2.5 | 4.2 | 2.5 | 4.7 | 2.5 | 4.2 | 2.5 | 4.7 |
| 30 | 6 | 2.5 | 4.1 | 2.6 | 4.8 | 2.5 | 4.1 | 2.5 | 4.1 | 2.5 | 4.1 | 2.5 | 4.8 | 2.5 | 4.1 | 2.5 | 4.8 |
| 30 | 8 | 2.6 | 4.2 | 2.6 | 4.8 | 2.6 | 4.2 | 2.6 | 4.2 | 2.6 | 4.2 | 2.6 | 4.8 | 2.6 | 4.2 | 2.6 | 4.8 |

|  |  | Backoff (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5900 MHz |  |  |  |
|  |  | Gpost=0 |  | Gpost=7 |  | Gpost=0 |  | Gpost=7 |  | Gpost=0 |  | Gpost=7 |  | Gpost=0 |  | Gpost=7 |  |
| SCS | Mod | Inr | Out | Inr | Out | Inr | Out | Inr | Out | Inr | Out | Inr | Out | Inr | Out | Inr | Out |
| 15 | 2 |  |  |  |  |  |  |  |  |  |  |  |  | 2.7 | 4.3 | 2.7 | 4.3 |
| 15 | 4 |  |  |  |  |  |  |  |  |  |  |  |  | 2.8 | 4.3 | 2.8 | 4.3 |
| 15 | 6 |  |  |  |  |  |  |  |  |  |  |  |  | 2.7 | 4.3 | 2.7 | 4.3 |
| 15 | 8 |  |  |  |  |  |  |  |  |  |  |  |  | 2.8 | 4.3 | 2.8 | 4.3 |
| 30 | 2 |  |  |  |  |  |  |  |  |  |  |  |  | 2.6 | 4.2 | 2.6 | 4.2 |
| 30 | 4 |  |  |  |  |  |  |  |  |  |  |  |  | 2.5 | 4.2 | 2.5 | 4.2 |
| 30 | 6 |  |  |  |  |  |  |  |  |  |  |  |  | 2.5 | 4.1 | 2.5 | 4.1 |
| 30 | 8 |  |  |  |  |  |  |  |  |  |  |  |  | 2.6 | 4.2 | 2.6 | 4.2 |

Table 2: A-MPR for NS_33 for PSSCH/PSCCH Transmission at 5870 to 5920 in n47

|  |  | Inner Allocations |  | Outer Allocations |  |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency(MHz) | SCS (kHz) | A-MPRBase | A-MPRStep | A-MPRBase | A-MPRStep |
| 5870, 5910, 5920 | 15 | ≤ 2.8 | 0 | ≤ 4.3 | 0.2 |
|  | 30, 60 |  |  |  | 0.1 |
| 5880, 5890, 5900 | 15, 30, 60 |  |  |  | 0 |

Table 3: A-MPR for NS_NEW for PSSCH/PSCCH Transmission in n47

|  |  | Inner Allocations | Outer Allocations |
| --- | --- | --- | --- |
| Carrier frequency(MHz) | SCS (kHz) | A-MPR | A-MPR |
| 5860, 5870, 5880, 5890, 5900, 5910, 5920 | 15,30,60 | ≤ 2.8 | ≤ 4.3 |

- 10MHz at 5860 under NS_33

From the data we conclude that A-MPR under NS_33 is higher at fc=5860 MHz due to the coexistence emission requirement under NS_33. The coexistence requirement starting at 5855 MHz and below requires significant A-MPR when operating at 5860 MHz.

For 5860 we evaluate the required backoff in 2 dB increments, chosen to minimize excess backoff. We found backoff did not change much for modulation order, so one spec for all modulation orders is appropriate. A plot of the 15kHz SCS results is shown in Figure 1.

Note some of the colors repeat in the figure, and the backoffs near RBstart = 0 map to the higher numbers on the scale. Also note the tick marks are a bit misaligned, the max green backoff is 24 dB. The same data is shown in Table 1. Similarly data for 30kHz SCS is shown in Figure 2 and Table 2.

NS_33 contains a radiated requirement that is converted to a conducted requirement by a scale factor called A-MPRStep and declared Gpostconnector value. We computed the difference between A-MPR at Gpostconnector 0 and 7 dB, then we divide that result by 8 to determine the A-MPRStep value. Figure 3 shows this result. We find the  A-MPRStep is strongly dependent on RBstart. We propose a RBstart-dependent formulation in Table 3.

Figure 1: NS_33 Backoff at 5860 15kHz SCS for all modulation orders

Table 1: A-MPR for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (15kHz SCS)

| LCRB Note1 | RBstart | A-MPRBase (dB) |
| --- | --- | --- |
| 10 | 0 | ≤ 24 |
| 15, 20 | 0 | ≤ 22 |
| 25, 30 | 0 | ≤ 20 |
| 40, 45 | 0 | ≤ 18 |
| 10 | 1 |  |
| 15,20,25 | 1 | ≤ 16 |
| 50 | 0 |  |
| 30, 40 | 1 | ≤ 14 |
| 10 | 2 |  |
| 45, 50 | 1 | ≤ 12 |
| 15, 20 | 2 to 4 |  |
| 25 | 2 to 3 |  |
| 30, 40 | 2 |  |
| 10 | 3 |  |
| 45, 50 | ≥2 | ≤ 10 |
| 30 | 3 to 19 |  |
| 40 | ≥3 |  |
| 10 | 4 to 6 |  |
| 25 | 4 to 16 |  |
| 15 | 5 to 10 |  |
| 20 | 5 to 13 |  |
| 10 | 7 to 15 | ≤ 8 |
| 15 | 11 to 16 |  |
| 20 | ≥14 |  |
| 25 | ≥17 |  |
| 30 | ≥20 |  |
| 15 | ≥33 |  |
| 10 | ≥39 |  |
| 10 | 16 to 18 | ≤ 6 |
| 15 | 17 to 32 |  |
| 10 | 36 to 38 |  |
| 10 | 19 to 35 | ≤ 4 |
| Note 1: LCRB is an integer multiple of any single value from the set {10,15,20,25,30,40,45,50}. |  |  |

Figure 2: NS_33 AMPRstep at 5860 15kHz SCS

Table 2: A-MPRStep for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (15kHz SCS)

| RBstart | A-MPRStep |
| --- | --- |
| 0 to 1 | 1.2 |
| 2 to 12 | 0.7 |
| ≥13 | 0.3 |

Figure 3: NS_33 Backoff at 5860 30kHz SCS for all modulation orders

Table 3: A-MPR for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (30kHz SCS)

| LCRB Note1 | RBstart | A-MPRBase (dB) |
| --- | --- | --- |
| 10,15 | 0 | ≤ 22 |
| 20 | 0 | ≤ 20 |
| 10 | 1 | ≤ 18 |
| 15, 20 | 1 | ≤ 16 |
| 10, 15, 20 | 2 | ≤ 12 |
| 10 | 3 to 6 | ≤ 10 |
| 15 | 3 to 9 |  |
| 20 | ≥3 |  |
| 10 | ≥7 | ≤ 8 |
| 15 | ≥10 |  |
| Note 1: LCRB is an integer multiplied by any single value from the set {10,15,20}. |  |  |

Table 4: A-MPRStep for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (30kHz SCS)

| RBstart | A-MPRStep |
| --- | --- |
| 0 to 2 | 1 |
| 3 to 6 | 0.5 |
| ≥7 | 0.25 |

Table 5: A-MPR for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (60kHz SCS)

| LCRB Note1 | RBstart | A-MPRBase (dB) |
| --- | --- | --- |
| 10 | 0 | ≤ 22 |
| 10 | 1 | ≤ 18 |
| 10 | 2 | ≤ 12 |
| Note 1: The value of LCRB is only allowed to be 10. |  |  |

Table 6: A-MPRStep for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (60kHz SCS)

A-MPRStep

1

- 40MHz at 5885 under NS_48

For 40 MHz A-MPR, all valid waveforms were simulated at fc = 5885 MHz for 15, 30, and 60 kHz SCS; all subband lengths (10, 15, 20, 25, 50, 75 and 100); and for QPSK, 16QAM, 64QAM, and 256 QAM. Plots of some of the simulations are shown in R4-2003436.

The values in Table 7  below are used to derive the TP specification proposal. Table 8 shows the specification formulation that is used in the text proposal.

Table 7: Backoff for NS_48 at 5885 MHz PSSCH/PSCCH Transmission in n47

| Channel BW (MHz) | Carrier Frequency (MHz) | SCS (kHz) | Modulation | Backoff Inner (dB) | Backoff Outer (dB) | Backoff Edge (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 40 | 5885 | 15000 | 2 | 5.5 | 7.9 | 8.3 |
| 40 | 5885 | 30000 | 2 | 3.8 | 6.9 | 8.1 |
| 40 | 5885 | 60000 | 2 | 2.1 | 7.5 | 8.4 |
| 40 | 5885 | 30000 | 4 | 3.8 | 6.9 | 8.2 |
| 40 | 5885 | 60000 | 4 | 2.1 | 7.3 | 8.5 |
| 40 | 5885 | 15000 | 6 | 5.5 | 7.8 | 8.3 |
| 40 | 5885 | 30000 | 6 | 3.7 | 6.9 | 7.7 |
| 40 | 5885 | 60000 | 6 | 2.2 | 7.6 | 8.2 |
| 40 | 5885 | 15000 | 8 | 5.5 | 7.8 | 8.4 |
| 40 | 5885 | 30000 | 8 | 3.8 | 7 | 8 |
| 40 | 5885 | 60000 | 8 | 2.1 | 7.3 | 9 |

Table 8: A-MPR for NS_48 for PSSCH/PSCCH Transmission in n47

| Carrier frequency (MHz) | SCS (kHz) | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner RB allocations | Outer RB allocations | Edge RB allocations |
| 5885 | 15 | ≤ 5.5 | ≤ 7.9 | ≤ 8.4 |
|  | 30 | ≤ 3.8 | ≤ 7 | ≤ 8.2 |
|  | 60 | ≤ 2.2 | ≤ 7.6 | ≤ 9 |

<<begin changes TR38.886 v0.6.0>>

### 8.1.3	UE maximum output power with additional requirements

To comply with the EN 302 571 standard  emission mask, RAN4 needs to derive A-MPR requirements considering the additional A-SEM and A-SE requirements in EN 302 571 as shown in Table 8.1.3-1 and Table 8.1.3-2.

An NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33, NS_48, or NS_NEW are configured from pre-configured radio parameters or when signalled from the cell.

Table 8.1.3-1: Additional SEM requirements for 10MHz channel bandwidth under NS_33 and NS_NEW

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depending on the post antenna connector gain Gpost connector declared by the UE following the principle described in annex G in TS38.101-1.

Table 8.1.3-2: Additional SE requirements

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to a conducted requirement depending on the post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: Applicable when NS_NEW is configured. |  |  |  |  |  |  |  |

Also, FCC has regulatory requirements for 40MHz in the ITS spectrum as shown in Table 8.1.3-3

Table 8.1.3-3: Additional SEM requirements for 40MHz channel bandwidth (fc =5885 MHz) when NS_48 is configured

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0 - 2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40 - 100 | -50 | 100kHz |

To comply with regional regulatory requirements, RAN4 specifies the A-MPR requirements according to different network signalling by pre-configured radio parameters or the cell , or when signalled from the cell to the NR V2X UE.

The allowed additional Maximum Power Reduction (A-MPR) for the maximum output power due to higher order modulation and transmit bandwidth configuration (resource blocks) will be specified in TS38.101-1 as below.

Table 8.1.3-4: Additional Maximum Power Reduction (A-MPR) for NR V2X

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_33 | Table 8.1.3-1 (A-SEM) Table 8.1.3-2 (A-SE) | n47 | 10 | Table 8.1.3-5 Table 8.1.3-6 Table 8.1.3-7 |  |
| NS_48 | Table 8.1.3-3 (A-SEM) | n47 | 40 | Table 8.1.3-8 |  |
| NS_NEW | Table 8.1.3-1 (A-SEM) | n47 | 10 | Table 8.1.3-5 Table 8.1.3-6 Table 8.1.3-7 |  |

When UE is configured for NR V2X PSSCH/PSCCH 10MHz transmissions in n47, and NS_33 is configured, the maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows for NS_33

MA = A-MPRBase + Gpost connector * A-MPRStep

Table 8.1.3-5: A-MPR for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (15kHz SCS)

| LCRB Note1 | RBstart | A-MPRBase (dB) |
| --- | --- | --- |
| 10 | 0 | ≤ 24 |
| 15, 20 | 0 | ≤ 22 |
| 25, 30 | 0 | ≤ 20 |
| 40, 45 | 0 | ≤ 18 |
| 10 | 1 |  |
| 15,20,25 | 1 | ≤ 16 |
| 50 | 0 |  |
| 30, 40 | 1 | ≤ 14 |
| 10 | 2 |  |
| 45, 50 | 1 | ≤ 12 |
| 15, 20 | 2 to 4 |  |
| 25 | 2 to 3 |  |
| 30, 40 | 2 |  |
| 10 | 3 |  |
| 45, 50 | ≥2 | ≤ 10 |
| 30 | 3 to 19 |  |
| 40 | ≥3 |  |
| 10 | 4 to 6 |  |
| 25 | 4 to 16 |  |
| 15 | 5 to 10 |  |
| 20 | 5 to 13 |  |
| 10 | 7 to 15 | ≤ 8 |
| 15 | 11 to 16 |  |
| 20 | ≥14 |  |
| 25 | ≥17 |  |
| 30 | ≥20 |  |
| 15 | ≥33 |  |
| 10 | ≥39 |  |
| 10 | 16 to 18 | ≤ 6 |
| 15 | 17 to 32 |  |
| 10 | 36 to 38 |  |
| 10 | 19 to 35 | ≤ 4 |
| Note 1: LCRB is an integer multiple of any single value from the set {10,15,20,25,30,40,45,50}. |  |  |

Table 8.3.1-6: A-MPRStep for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (15kHz SCS)

| RBstart | A-MPRStep |
| --- | --- |
| 0 to 1 | 1.2 |
| 2 to 12 | 0.7 |
| ≥13 | 0.3 |

Table 8.3.1-7: A-MPR for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (30kHz SCS)

| LCRB Note1 | RBstart | A-MPRBase (dB) |
| --- | --- | --- |
| 10,15 | 0 | ≤ 22 |
| 20 | 0 | ≤ 20 |
| 10 | 1 | ≤ 18 |
| 15, 20 | 1 | ≤ 16 |
| 10, 15, 20 | 2 | ≤ 12 |
| 10 | 3 to 6 | ≤ 10 |
| 15 | 3 to 9 |  |
| 20 | ≥3 |  |
| 10 | ≥7 | ≤ 8 |
| 15 | ≥10 |  |
| Note 1: LCRB is an integer multiplied by any single value from the set {10,15,20}. |  |  |

Table 8.3.1-8: A-MPRStep for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (30kHz SCS)

| RBstart | A-MPRStep |
| --- | --- |
| 0 to 2 | 1 |
| 3 to 6 | 0.5 |
| ≥7 | 0.25 |

Table 8.3.1-9: A-MPR for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (60kHz SCS)

| LCRB Note1 | RBstart | A-MPRBase (dB) |
| --- | --- | --- |
| 10 | 0 | ≤ 22 |
| 10 | 1 | ≤ 18 |
| 10 | 2 | ≤ 12 |
| Note 1: The value of LCRB is only allowed to be 10. |  |  |

Table 8.3.1-10: A-MPRStep for NS_33 at 5860 MHz PSSCH/PSCCH Transmission in n47 (60kHz SCS)

A-MPRStep

1

Table 8.1.3-11: A-MPR for NS_33 for PSSCH/PSCCH Transmission at 5870 to 5920 in n47

|  |  | Inner Allocations |  | Outer Allocations |  |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency(MHz) | SCS (kHz) | A-MPRBase | A-MPRStep | A-MPRBase | A-MPRStep |
| 5870, 5910, 5920 | 15 | ≤ 2.8 | 0 | ≤ 4.3 | 0.2 |
|  | 30, 60 |  |  |  | 0.1 |
| 5880, 5890, 5900 | 15, 30, 60 |  |  |  | 0 |

When UE is configured for NR V2X PSSCH/PSCCH 40MHz transmissions in n47, and NS_48 is configured, the maximum output power reduction specified in Table 8.1.3-12.

Table 8.1.3-12: A-MPR for NS_48 for PSSCH/PSCCH Transmission in n47

| Carrier frequency (MHz) | SCS (kHz) | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner allocations | Outer allocations | Edge allocations |
| 5885 | 15 | ≤ 5.5 | ≤ 7.9 | ≤ 8.4 |
|  | 30 | ≤ 3.8 | ≤ 7 | ≤ 8.2 |
|  | 60 | ≤ 2.2 | ≤ 7.6 | ≤ 9 |

When UE is configured for NR V2X PSSCH/PSCCH 10MHz transmissions in n47, and NS_NEW is configured, the maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows for NS_33

MA = A-MPRBase + Gpost connector * A-MPRStep

Table 8.1.3-13 specifies the A-MPR in n47 under NS_NEW

Table 8.1.3-13: A-MPR for NS_NEW for PSSCH/PSCCH Transmission in n47

| Carrier frequency(MHz) | SCS (kHz) | A-MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Inner Allocations | Outer Allocations |
| 5860, 5870, 5880, 5890, 5900, 5910, 5920 | 15,30, 60 | ≤ 2.8 | ≤ 4.3 |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(NRB /3.5))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ ceil(NRB/3.5)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ floor(NRB*0.2) RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

[Editor Notes]: A-MPR requirements for PSSCH/PSCCH, simultaneous PSFCH and S-SSB will be updated based on the RAN4 agreements. The captured A-MPR requirements for PSSCH/PSCCH is based on approved TP (R4-2002783)

<<end changes TR38.886 v0.6.0>>