3GPP TSG-RAN WG4 Meeting #95-e 	R4-2007906

Online, 25 May - 05 Jun 2020

Source: 	Huawei

Title: 	TP to TS 38.174: Tx IAB-MT Dynamic range

Agenda Item:	6.5.2.2.3

Document for:	Approval

# Introduction

The exact values for the IAB-MT dynamic range are still open (R4-2005484) however enough has been agreed about the nature of the requirement that the TS drafting can be attempted.

Compared to the IAB-DU requirement we have:

RE power control – this does not apply to IAB-MT – options are to list it and state it doesn’t apply or just have no sub-clause. As there is no requirement there is no need for a sub-clause so its just omitted.

Total power dynamic range is a BS term, the parameter being discussed is dynamic range, but in terms of the uplink the definition is loser to UE min power, rather than the OFDM symbol dynamic range defined for the BS. The UE min power definition has been modified to defined dynamic range

# TP to TS 38.174 v0.0.1

--- Start of changes ---

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 38.104 (V16.3.0): “NR; Base Station (BS) radio transmission and reception”

--- Next change ---

### 6.3.2	IAB-MT Output Power Dynamics

#### 6.3.2.1	Total power dynamic range

##### 6.3.2.1.1	General

The IAB-MT total power dynamic range is the difference between the maximum and the minimum controlled transmit power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to the maximum and minimum values. The maximum and minimum output powers are defined as the mean power in at least one sub-frame 1ms

##### 6.3.2.1.2	Minimum requirement for IAB-DU type 1-H

For a wide area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to [0,5,10] dB.

For a local area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to [10,20] dB.

--- Next change ---

#### 9.4.3.1	OTA total power dynamic range

##### 9.4.3.1.1	General

The OTA total power dynamic range is the difference between the maximum and the minimum controlled transmit power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to the maximum and minimum values. The maximum and minimum output powers are defined as the mean power in at least one sub-frame 1ms

This requirement shall apply at each RIB supporting transmission in the operating band.

##### 9.4.3.1.2	Minimum requirement for IAB-DU type 1-O

For a wide area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to [0,5,10] dB.

For a local area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to [10,20] dB.

##### 9.4.3.1.3	Minimum requirement for IAB-DU type 2-O

For a wide area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to [0,5,10] dB.

For a local area IAB-MT the total power dynamic range for each NR carrier shall be larger than or equal to [10,20] dB.

--- End of changes ---