3GPP TSG-RAN WG4 Meeting #95-e		R4-2007923

Electronic meeting, 25th May – 5th June, 2020

Agenda item:	6.17.2.1.3

Source:	Ericsson

Title:	Simulation results for NR UE HST Single tap

Document for:	Discussion

# Introduction

In this paper we provide simulation results based on the assumptions found in [1].

# Simulation results

## FDD

Figure 1 FDD Throughput curve MCS17

Table 1 SNR at 70% maximum throughput

| MCS | Doppler [Hz] | SNR @ 70% TP mark |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1Tx 2Rx (ideal) | 1Tx 2Rx (impairments) | 1Tx 4Rx (ideal) | 1Tx 4Rx (impairments) |
| 17 | 875 | 6.41 | 7.41 | 3.41 | 4.41 |
|  | 972 | 6.41 | 7.41 | 3.42 | 4.42 |
|  | 1250 | 6.41 | 7.41 | 3.40 | 4.40 |

From a doppler estimation point of view, we can see that the demodulation performance does not degrade when comparing 1250Hz doppler with 875Hz for FDD test cases.

Observation 1: FDD 15kHz SCS at 1250Hz doppler does not degrade demodulation performance compared to 875Hz doppler or 972Hz doppler.

Proposal 1: We suggest setting 1250Hz doppler for FDD but can compromise to 972Hz.

## TDD

Figure 2 TDD throughput curve MCS17

Table 2 SNR at 70% maximum throughput

| MCS | Doppler [Hz] | SNR @ 70% TP mark |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1Tx 2Rx (ideal) | 1Tx 2Rx (impairment) | 1Tx 4Rx (ideal) | 1Tx 4Rx (impairment) |
| 17 | 1667 | 6.53 | 7.53 | 3.53 | 4.53 |

# Conclusions

We provide simulation results for HST single tap for both FDD, and TDD test cases. The SNR at the 70% maximum throughput mark are provided in table 1, and table 2.

Observation 1: FDD 15kHz SCS at 1250Hz doppler does not degrade demodulation performance compared to 875Hz doppler or 972Hz doppler.

Proposal 1: We suggest setting 1250Hz doppler for FDD but can compromise to 972Hz.

# References

- R4-2002418, “WF on UE demodulation for NR HST”, CMCC