3GPP TSG-RAN4 Meeting #95-e	R4-2008047

# Online, , 25th May 2020 - 5th Jun 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0363 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UL MIMO changes MPR and emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Incorrect emission requirement handling for UL MIMO and missing MPR requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | section 6.1 applicability is clarified for sections section 6.1D is created to address UL MIMO MPR tables adde to 6.2D.2 6.5D emission requirements modified to apply two antenna connectors |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP emission requirements for UL MIMO operation mode are not aligned with regulatory requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.1D, 6.2D.2, 6.5D.1, 6.5D.2, 6.5D.3, 6.5D.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: New requirements are set but legacy UEs are accomodated with output power relaxations |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of changes>

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE.For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Unless otherwise stated, transmitter requirements apply when the UE is configured with nrofSRS-Ports =1, or if txConfig is not configured. UE requirements when it is configured with nrofSRS-Ports = 2 is addressed in sub sections with ‘D’-suffix.

## 6.1D	General for UL MIMO

Requirements for UL MIMO operation apply when the UE is configured with nrofSRS-Ports = 2.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

## 6.2	Transmitter power

### <End of changes>

### <Start of changes>

### 6.2D.2	UE maximum output power reduction for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Tables 6.2D.2-1 and 6.2D.2-2. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output powers at each UE antenna connector.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

Table 6.2D.2-1 Maximum power reduction (MPR) for power class 3 for UL MIMO

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 4.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 4.5 |  | ≤ 3.5 |
|  | 64 QAM | ≤ 5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2D.2-2 Maximum power reduction (MPR) for power class 2 for UL MIMO

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4.5 | ≤ 3 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

### <End of changes>

### <Start of changes>

## 6.5D	Output RF spectrum emissions for UL MIMO

### 6.5D.1	Occupied bandwidth for UL MIMO

For UE configured for UL MIMOin accordance to section 6.2D, the requirements for occupied bandwidth is specified for sum of emissions from  each transmit antenna connector. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth of sum of signals from each transmitter antenna shall be less than the channel bandwidth specified in table 6.5.1-1. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.

### 6.5D.2	Out of band emission for UL MIMO

For UE configured for UL MIMO in accordance with section 6.2D, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters are specified for sum of emissions from each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2 apply to the sum of emissions from each transmit antenna connector. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.

### 6.5D.3	Spurious emission for UL MIMO

For UE configured for UL MIMO in accordance with section 6.2D, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products are specified for sum of emissions from each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in subclause 6.5.3 apply to the sum of emissions from each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in clause 6.2D.1.

### 6.5D.4	Transmit intermodulation for UL MIMO

For UE supporting UL MIMO, the transmit intermodulation requirements are specified for sum of emissions from each transmit antenna connector and the wanted signal is defined as the sum of output power at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in clause 6.5.4 apply to the sum of emissions from each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in clause 6.2D.1.

### <End of changes>