3GPP TSG-RAN WG4 Meeting#95-e	R4-2008113

Online, 25th May – 5th Jun, 2020

Source: 	Qualcomm Incorporated

Title: 	Views on CA CQI Reporting Tests

Agenda item:	6.18.1.5

Document for:	Discussion

# Introduction

In WF [1], there were many open issues listed for CA CQI reporting requirements. In this paper, we discuss our views on test cases for CA CQI reporting.

# Duplex Mode and SCS

In WF [1], there were two options listed for duplex mode and SCS. As we have already covered all the CA combinations for NR CA normal demodulation test cases, we do not need to cover all of them again for CA CQI reporting requirements. Therefore, our preference is Option 2 and we propose the following.

Proposal 1: Define the CA CQI requirements for following cases:

- FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

- FR2: TDD + TDD with 120 kHz SCS

# FR2 TDD Pattern

In 38.101-4, all the existing FR2 CQI requirements are defined with TDD pattern DDSU. So, we would like to reuse the same pattern for defining CA CQI requirements for FR2. Therefore, we propose the following.

Proposal 2: For defining FR2 CA CQI requirements, use DDSU (S = 11D+3G) TDD Pattern and CSI reporting periodicity of 8 slots.

# Test Framework

In our opinion, LTE test framework for CA CQI reporting can be applied to NR as well. Therefore, we propose the following.

Proposal 3: Reuse the test framework for CA CQI reporting tests in LTE for defining NR CA CQI reporting tests.

Based on our simulation results for FR1, we are ok with Option 1 in WF [1] for SNR configurations and threshold for FR1 CA CQI reporting tests. For FR2, we need more time to run the simulations and we may be able to confirm during the meeting. Therefore, we propose the following.

Proposal 4: Define FR1 CA CQI reporting requirements with the following configuration:

- SNR configuration for 2DL CA CQI test: SNRPcell = 10dB and SNRScell = 4dB

- SNR configuration for 3 or more DL CA CQI test: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB

- Delta CQI threshold for CA CQI test = 2 for 2 or more DL CA

# Conclusions

This paper discusses open issues for power imbalance test cases. Following has been proposed.

Proposal 1: Define the CA CQI requirements for following cases:

- FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

- FR2: TDD + TDD with 120 kHz SCS

Proposal 2: For defining FR2 CA CQI requirements, use DDSU (S = 11D+3G) TDD Pattern and CSI reporting periodicity of 8 slots.

Proposal 3: Reuse the test framework for CA CQI reporting tests in LTE for defining NR CA CQI reporting tests.

Proposal 4: Define FR1 CA CQI reporting requirements with the following configuration:

- SNR configuration for 2DL CA CQI test: SNRPcell = 10dB and SNRScell = 4dB.

- SNR configuration for 3 or more DL CA CQI test: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB.

- Delta CQI threshold for CA CQI test = 2 for 2 or more DL CA.

# References

[1] R4-2005548, “Way forward on CA CQI reporting requirements”, 3GPP TSG RAN WG4 Meeting #94-bis-e, Online, Apr 20 - Apr 30, 2020.