3GPP TSG-RAN WG4 Meeting #94bis-e                                                          R4-2008350

Online, 25th May-5th June, 2020

Source:	Samsung, KDDI

Title:	TP for TR 38.716-02-00: CA_n3/n28-n77(2A)/n78(2A)_UL_n77(2A)/n78(2A)

Agenda item:			8.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.716-02-00 to include CA_n3-n77(2A)_UL_n77(2A), CA_n28-n77(2A)_UL_n77(2A), CA_n3-n78(2A)_UL_n78(2A) and CA_n28-n78(2A)_UL_n78(2A) according to the request in [1].

# 2. Reference

- RP-200168, Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2).

# 3. Text Proposal

<Start of Text Proposal>

## 6.48	CA_n3-n77

### 6.48.1		Common for 1 band UL and 2 bands UL CA

#### 6.48.1.1		Operating bands for CA

Table 6.48.1.1-1: CA band combination of band n3+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.48.1.2		Channel bandwidths per operating band for CA

Table 6.48.1.2-1: Supported bandwidths per CA band combination of band n3+n77

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n3A-n77A | CA_n3A-n77A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n3A-n77(2A) | CA_n77(2A) CA_n3A-n77A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.48.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n3 + n77 have been already completed and captured into TR 37.865-01-01.

#### 6.48.1.4		∆TIB and ∆RIB values

For CA_n3-n77, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

#### 6.48.1.5		REFSENS requirements

The studies for 1 band UL for the CA band combination of band n3 + n77 have been already completed and captured into TR 37.865-01-01.

## 6.x	CA_n3-n78

### 6.x.1		Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n3+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n3+n78

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n3A-n78(2A) | CA_n78(2A) | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n3 + n78 have been already completed and captured into TR 37.865-01-01.

#### 6.x.1.4		∆TIB and ∆RIB values

For CA_n3-n78, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

#### 6.x.1.5		REFSENS requirements

The studies for 1 band UL for the CA band combination of band n3 + n78 have been already completed and captured into TR 37.865-01-01.

## 6.27	CA_n28-n77

### 6.27.1		Common for 1 band UL and 2 bands UL CA

#### 6.27.1.1		Operating bands for CA

Table 6.27.1.1-1: CA band combination of band n28+n77

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n77 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.27.1.2		Channel bandwidths per operating band for CA

Table 6.27.1.2-1: Supported bandwidths per CA band combination of band n28+n77

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n77A | CA_n28A-n77A | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n28A-n77(2A) | CA_n77(2A)3 CA_n28A-n77A | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note3: For CA_n77(2A), if one assigned carrier frequency of UE channel bandwidth is within 3300-3400MHz frequency range, the other carrier freuqency should not be assigned within 4000-4200MHz or vice versa. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.27.1.3		Co-existence studies

Table 6.27.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n28-n77.

Table 6.27.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 | 3515 | 3740 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on the above table, there is 5th harmonic issue for the band combination of n28 and n77.

The IMD2 of n77(2A) may fall into band n28 DL when the two UL carriers are at the same time assigned within the frequency range of 3300MHz-3400MHz and the frequency range of 4000MHz-4200MHz separately.

#### 6.27.1.4		∆TIB and ∆RIB values

For CA_n28-n77, the TIB,c and RIB,c values for UEs not supporting simultaneous Rx/Tx are given in the tables below.

Table 6.27.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n77 | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.27.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28-n77 | n28 | 0.2 |
|  | n77 | 0.5 |

#### 6.27.1.5		REFSENS requirements

Due to identified harmonic issues, MSD for CA_n28-n77 is derived and need to be defined in 38.101-1 as below.

Table 6.27.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n28 | n771, 2 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 | 1.7 | 1.2 | 0.7 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that fULLB=fDLHB/0.50.1 in MHz and FUL_lowLB+BWChannelLB/2≤fULLB≤FUL_highLB-BWChannelLB/2 with fDLHB the carrier frequency of a high band in MHz and BWChannelLB the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.27.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | n77 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |

## 6.37	CA_n28-n78

### 6.37.1		Common for 1 band UL and 2 bands UL CA

#### 6.37.1.1		Operating bands for CA

Table 6.37.1.1-1: CA band combination of band n28+n78

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n78 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 6.37.1.2		Channel bandwidths per operating band for CA

Table 6.37.1.2-1: Supported bandwidths per CA band combination of band n28+n78

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n78(2A) | CA_n78(2A) | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.37.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n28 + n78 have been already completed and captured into TR 37.865-01-01.

#### 6.37.1.4		∆TIB and ∆RIB values

For CA_n28-n78, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

#### 6.37.1.5		REFSENS requirements

The studies for 1 band UL for the CA band combination of band n28 + n78 have been already completed and captured into TR 37.865-01-01.

<End of Text Proposal>